IEEE P802.11
Wireless LANs

| Proposed Changes to D0.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Self | Southampton, UK |  | stephen.mccann@ieee.org |

### Abstract

This document describes proposed updates to 11bc D0.1 based on the August 2020 comment collection.

V0	Presented on August 11th teleconference up to CID 6

V1	Update for CID 3 (Clause 9.4.2.bc.248), CID 51 (Clause 3.2). and CID 316 (Clause 9.4.2.1)

V2	Various changes discussed on the August 18th 2020 teleconference

V3,4	Various changes discussed during August and September 2020 teleconferences

V5	Changes discussed on the September 14th teleconference

V6	Changes discussed on the September 15th teleconference

## 3.2 Definitions specific to IEEE 802.11

Insert the following definitions maintaining alphabetical order:

[CID 51] enhanced broadcast service (eBCS): A service that provides uplink and downlink broadcast traffic. It additionally provides means for protecting broadcast traffic and the privacy of the stations receiving that traffic, including protection of origin authenticity between STAs.

9.4.2 Elements

## 9.4.2.1 General

Insert the following new rows into Table 9-94 (Element IDs) (header row shown for convenience):

## Table 9-94 -- Element IDs

| Element | Element ID | Element ID Extension | Extensible | Fragmentable |
| --- | --- | --- | --- | --- |
| eBCS Parameters | 255 | <ANA> | Yes |  |
| [CID 316] eBCS Capabilities | 255 | <ANA+1> | Yes | No |
| eBCS Container | 255 | <ANA+2> | Yes |  |

Insert the following subclauses:

9.4.2.bc eBCS Capabilities element

## 9.4.2.bc.1 General

The eBCS Capabilities element contains fields that are used to advertise the capabilities of an eBCS

STA related to forwarding service to a remote destination.

An eBCS AP declares support for forwarding service and related capabilities by including the eBCS

Capabilities element in a Beacon and broadcast Probe Response frame.

An eBCS non-AP STA can advertise the eBCS Capabilities element if it intends for an

AP to forward its data to a remote destination.

|  | Element ID | Length | Element ID Extension | eBCS Capabilities |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

# Figure 9-bc1 - eBCS Capabilities element format

The format of eBCS Capabilities element is shown in Figure 9-bc1 (eBCS Capabilities element format).

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The contents of an eBCS Capabilities field is defined in 9.4.2.bc.2 when the element is transmitted by an eBCS AP and defined in 9.4.2.bc.3 when the element is transmitted by an eBCS non-AP STA.

## 9.4.2.bc.2 [CID 3] eBCS Capabilities for an AP

The format of an eBCS Capabilities field when transmitted by an eBCS AP is shown in Figure 9-bc2 (Format of eBCS Capabilities field for an AP).

|  | AP Control | Next eBCS Info frame |
| --- | --- | --- |
| Octet: | 2 | 2 (optional) |

# Figure 9-bc2 - Format of eBCS Capabilities field for an AP

The format of AP Control is shown in Figure 9-bc3 (AP Control field format).

|  | B0 B1 | B2 B3 | B4 | B5 | B6 | B7 B15 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Authentication Mode | Limiting Mode | Location Embedding Supported | Data-Time Embedding Supported | IP Address Embedding Supported | Reserved |
| Bits: | 2 | 2 | 1 | 1 | 1 | 9 |

## Figure 9-bc3 - AP Control field format

The encoding of Authentication Mode subfield is shown in Table 9-bc4 (Encoding of Authentication Mode subfield).

## Table 9-bc4 - Encoding of Authentication Mode subfield

| Subfield value | Definition | Encoding |
| --- | --- | --- |
| 0 | No Authentication | AP forwards contents of an eBCS UL frame to the remote destination identified in the frame without authenticating the transmitter of the frame. |
| 1 | Per Destination | AP forwards contents of an eBCS UL frame only if it is able to successfully able to authenticate the transmitter of the frame based on an established relationship with the remote destination identified in the frame. |
| 2 – 3 | Reserved |  |

The encoding of Limiting Mode subfield is shown in Table 9-bc5 (Encoding of Limiting Mode subfield).

## Table 9-bc5 - Encoding of Limiting Mode subfield

| Subfield value | Definition | Encoding |
| --- | --- | --- |
| 0 | No Throttling | AP applies no restrictions on the amount/frequency of ULs from a non-AP STA destined to a remote destination. |
| 1 | Per Destination | AP applies forwarding limits as specified by the remote destination with whom it has established a relationship. |
| 2 – 3 | Reserved |  |

Location Embedding Supported subfield is set to 1 if the AP supports embedding of location information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an eBCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

Date-Time Embedding Supported subfield is set to 1 if the AP supports embedding of date and time information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an eBCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

IP Address Embedding Supported subfield is set to 1 if the AP supports embedding of IP address information, based on a non-AP STA’s request, before forwarding the HLP payload carried in an eBCS UL frame to the remote destination. Otherwise, the subfield is set to 0.

The Next eBCS Info frame subfield is 2 octets in lengths and indicates the number of TBTTs until the next eBCS Info frame is transmitted.  If the STA does not transmit eBCS Info frames, this subfield is not used.

### 9.4.2.bc.3 eBCS Capabilities for a non-AP STA

The format of an eBCS Capabilities field when transmitted by an eBCS non-AP STA is shown in Figure 9-bc6 (Format of eBCS Capabilities field for a non-AP STA).

|  | Non-AP STA Control |
| --- | --- |
| Octets: | 1 |

# Figure 9-bc6 - Format of eBCS Capabilities field for a non-AP STA

The format of Non-AP STA Control is shown in Figure 9-bc7 (Non-AP STA Control field format).

|  | B0 | B1 | B2 | B3 | B4 B7 |
| --- | --- | --- | --- | --- | --- |
|  | No Forwarding Without Embedding | Location Embedding Requested | Date-Time Embedding Requested | IP Address Embedding Requested | Reserved |
| Bits: | 1 | 1 | 1 | 1 | 4 |

## Figure 9-bc7 - Non-AP STA Control field format

The No Forwarding Without Embedding subfield is set to 1 to indicate that the AP is not required to forward the contents of the frame transmitted by non-AP STA to the remote destination if it cannot append the requested information before forwarding. Otherwise the subfield is set to 0.

The Location Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append location information before forwarding. Otherwise the subfield is set to 0.

The Date-Time Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append date and time information before forwarding. Otherwise the subfield is set to 0.

The IP Address Embedding Requested subfield is set to 1 to indicate that the non-AP STA transmitting the element is requesting an eBCS AP that will forward its content to the remote to append AP’s IP address information before forwarding. Otherwise the subfield is set to 0.

### [CID 348]

Add the following three new clauses after 9.4.5.29.

## 9.4.5.bc. Enhanced Broadcast Services ANQP-element

The Enhanced Broadcast Services ANQP-element provides a list of one or more enhanced broadcast services that are available from a peer STA.  The format of the Enhanced Broadcast Services ANQP-element is defined in Figure 9-bc8

| Info ID | Length | Broadcast | Enhanced Broadcast Services Tuples |
| --- | --- | --- | --- |

Octets: 	2 	2 	1 	variable

# Figure 9-bc8 Enhanced Broadcast Services ANQP-element format

The Info ID and Length fields are defined in9.4.5.1 (General).

[CIDs 7, 8, 9, 160, 164, 165, 167] The Broadcast field is defined in Figure 9-bc8a:

B0                  B1                  B2               B3           B7

| Transmit Capability | Receive Capability | Service Advertisement | Reserved |
| --- | --- | --- | --- |

Bits: 	                                     1 	                   1                  1                     5

# Figure 9-bc8a Broadcast field format

The Transmit Capability subfield is set to 1 by a STA to indicate that it supports the transmission of eBCS.  This subfield is set to 0 to indicate that there is no support for the transmission of eBCS.

The Receive Capability subfield is set to 1 by a STA to indicate that it supports the reception of eBCS.  This subfield is set to 0 to indicate that there is no support for the reception of eBCS.

The Service Advertisement subfield is set to 1 by a STA to indicate that the Enhanced Broadcast Services Tuples subfield contains information about the eBCS transmitted by the STA.  This subfield is set to 0 to indicate that there are no Enhanced Broadcast Services Tuples subfields at the time of transmission from the STA.

[CIDs 10, 11, 12, 42, 43 44, 169, 170, 171] The Enhanced Broadcast Services Tuples field contains one or more Enhanced Broadcast Services Tuple fields as shown in Figure 9-bc10.

|  | Control | Broadcast Time Length (Optional) | Broadcast Time (Optional) | Broadcast Duration Length (Optional) | Broadcast Duration (Optional) | Broadcast Stream Information Length (Optional) | Broadcast Stream Information (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 0 or 1 | 0 or 8 | 0 or 1 | 0 or 2 | 0 or 1 | variable |

Figure 9-bc10 - Enhanced Broadcast Services Tuple field format

The Control field defines which of the optional fields are present in the Enhanced Broadcast Services Tuple field and is defined in Figure 9-bc10a:

B0                  B1                  B2                   B3           B7

| Broadcast Time Present | Broadcast Duration Present | Broadcast Stream Information Present | Reserved |
| --- | --- | --- | --- |

Bits: 	                                 1 	               1                      1                         5

# Figure 9-bc10a Control field format

The Broadcast Time Present subfield is set to 1 by a STA to indicate that the Enhanced Broadcast Services Tuple field contains a Broadcast Time Length and a Broadcast Time field.  This subfield is set to 0 to indicate that there is no Broadcast Time field.

The Broadcast Duration Present subfield is set to 1 by a STA to indicate that the Enhanced Broadcast Services Tuple field contains a Broadcast Duration Length and a Broadcast Duration field.  This subfield is set to 0 to indicate that there is no Broadcast Duration field.

The Broadcast Stream Information Present subfield is set to 1 by a STA to indicate that the Enhanced Broadcast Services Tuple field contains a Broadcast Stream Information Length and a Broadcast Stream Information field.  This subfield is set to 0 to indicate that there is no Broadcast Stream Information field.

[CID 174] The Broadcast Time Length field is an optional 1-octet field whose value is set to the length of the Broadcast Time field. If the Broadcast Time field is not used, the Broadcast Time Length field is also not used.

[CID 175] The Broadcast Time field contains information about when the broadcast (indicated by the Broadcast Stream Information field) is available, in terms of time and whether the broadcast repeats. The format of the Broadcast Time field is the same as the Timestamp field defined in 9.4.1.10. If the Broadcast Time field is not present, then the receiving STA can assume that the broadcast stream is always available.

The Broadcast Duration Length field is an optional 1-octet field whose value is set to the length of the Broadcast Duration field. If the Broadcast Duration field is not used, the Broadcast Time Duration field is also not used.

[CIDs 47, 176, 177] The Broadcast Duration field contains information about the duration of a broadcast (indicated by the Broadcast Stream Information field). The format of the Broadcast Duration field is an integer value in seconds. If present the value 0 is used to indicate that the broadcast is continuous.

The Broadcast Stream Information Length field is an optional 1-octet field whose value is set to the length of the Broadcast Stream Information field. If the Broadcast Stream Information field is not used, the Broadcast Stream Information Length field is also not used.

[CID 13]

[CIDs 45, 173] The Broadcast Stream Information field is used to advertise a source of the broadcast stream (e.g. a channel number, or URL), when the value of the Broadcast Stream Information Present field bit 1 is true (see Table 9-bc8a).

[CID 48]

9.4.5.bc1. Enhanced Broadcast Request ANQP-element

The Enhanced Broadcast Request ANQP-element transmits a register (or de-register) request from a STA to a peer STA to receive (or stop receiving) enhanced broadcast services that are available from the peer STA.  The format of the Enhanced Broadcast Request ANQP-element is defined in Error: Reference source not found.

|  | Info ID | Length | Broadcast Action | Enhanced Broadcast Services Tuples (Optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |

Figure 9-bc11 - Enhanced Broadcast Request ANQP-element format

The Info ID and Length fields are defined in Error: Reference source not found.

The Broadcast Action field is a 1 octet field with values defined in Table 9-bc12:

Table 9-bc12 – Broadcast Action field values

| Value | Description |
| --- | --- |
| 0 – 1 | Reserved |
| 2 | Register to receive broadcast |
| 3 | Unregister from receiving broadcast |
| 4 | Register with Broadcast Service(s) as indicated in the Enhanced Broadcast Services Tuples field |
| 5 | [CID 161] Unregister Broadcast Service(s) as indicated in the Enhanced Broadcast Services Tuples field |
| 6 - 7 | Reserved |

The Enhanced Broadcast Services Tuples field is described in 9.4.5.bc.

9.4.5.bc2. Enhanced Broadcast Response ANQP-element

The Enhanced Broadcast Response ANQP-element provides a list of zero or more enhanced broadcast services that are available from a peer STA.  The format of the Enhanced Broadcast Response ANQP-element is defined in Error: Reference source not found.

|  | Info ID | Length | Broadcast Action Response | Enhanced Broadcast Services Tuples (Optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |

Figure 9-bc13 - Enhanced Broadcast Response ANQP-element format

The Info ID and Length fields are defined in Error: Reference source not found.

[CID 179] The Broadcast Action Response field is a 1 octet bitmap and is defined in Table 9-bc14:

Table 9-bc14 – Broadcast Action Response field values

| Value | Description |
| --- | --- |
| 0 | Success |
| 1 | Failure |
| 2 - 7 | Reserved |

The Enhanced Broadcast Services Tuples field is described in 9.4.5.bc.

## 9.6.7.bc eBCS UL frame format

The eBCS UL frame is transmitted by an eBCS non-AP STA carrying data intended for a remote destination identified in the frame. The format of eBCS UL frame Action field is shown in Figure 9-bc15 (eBCS UL frame Action field format).

|  | Category | Public Action | eBCS UL Control | HLP Payload Length | HLP Payload | STA Certificate Length | STA Certificate | Timestamp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | variable | 0 or 2 | 0 or variable | 0 or 8 |

|  | Destination URI | eBCS Capabilities | Frame Signature Length | Frame Signature |
| --- | --- | --- | --- | --- |
| Octets: | variable | 0 or 4 | 0 or 2 | 0 or variable |

## Figure 9-bc15 - eBCS UL frame Action field format

The Category field is defined in 9.4.1.11 (Action field).

The Public Action field is defined in 9.6.7.1 (Public Action frames).

| B0 | B1 | B2 | B3 | B4 B6 | B7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | STA Certificate Present | Packet Number Present | eBCS Capabilities Present | Timestamp Present | Reserved | Frame Signature Present |
| Bits: | 1 | 1 | 1 | 1 | 3 | 1 |

# Figure 9-bc16 - eBCS UL Control field format

The format of eBCS UL Control field is shown in Figure 9-bc16 (eBCS UL Control field format).

The STA Certificate Present subfield is set to 1 when the STA Certificate Length and STA Certificate fields are carried in the frame. Otherwise the subfield is set to 0.

The Packet Number Present subfield is set to 1 when the Packet Number field is carried in the frame. Otherwise the subfield is set to 0.

The eBCS Capabilities Present subfield is set to 1 when the eBCS Capabilities element is carried in the frame. Otherwise the subfield is set to 0.

The Timestamp Present subfield is set to 1 when the Timestamp field is carried in the frame. Otherwise the subfield is set to 0.

The Frame Signature Present subfield is set to 1 when the Frame Signature Length and Frame Signature fields are carried in the element. Otherwise the subfield is set to 0.

The HLP Payload Length indicates the length of the HLP Payload field.

The HLP Payload field carries the higher layer protocol (HLP) payload.

The STA Certificate Length field indicates the length of the STA Certificate field. The presence of this  field is indicated by the STA Certificate Present subfield in the eBCS UL Control field being equal to 1.

The STA Certificate field carries the certificate of the transmitting STA. The presence of this field is indicated by the STA Certificate Present subfield in the eBCS UL Control field being equal to 1.

The Timestamp field provides protection against replay attack. The presence of this field is indicated by the  Timestamp Present subfield in the eBCS UL Control field being equal to 1.

| The format of the Timestamp field is shown in Figure 9-bc17 - Timestamp field format. | Time | Counter |
| --- | --- | --- |
| Octets: | 4 | 4 |

## Figure 9-bc17 - Timestamp field format

The 32 bit Time subfield carries time information. See clause 11.bc.1.3 (eBCS UL operation at an eBCS non-AP STA) for details on the value carried in this subfield.

The 32 bit Counter subfield carries a numeric value which is incremented for each packet transmission.

The Destination URI element is defined in 9.4.2.89 (Destination URI element) and carries the address of the remote destination where the packet needs to be forwarded to.

Note: The length of the Destination URI element is computed based on the value carried in the Length field in the element (value in Length field + 2 octets).

The eBCS Capabilities element is defined in 9.4.2.bc (eBCS Capabilities element). The element is 31 optionally present and carries the capabilities of the transmitting non-AP STA. The presence of this 32 element is indicated by the eBCS Capabilities Present subfield in the eBCS UL Control field being equal to 1.

The Frame Signature Length field indicated the length of the Frame Signature field. The presence of this field indicated by the Frame Signature Present subfield in the eBCS UL Control field being equal to 1.

The Frame Signature field carries signature for the contents of the eBCS UL frame Action field except the 39 Frame Signature field. The presence of this field indicated by the Frame Signature Present subfield in the eBCS UL Control field being equal to 1.

# 11.bc.3.2 eBCS UL operation at an eBCS AP

An eBCS AP may provide forwarding service in which it can support forwarding the contents of an eBCS UL frame received from an eBCS non-AP STA to a remote destination identified in the frame.

An eBCS AP that supports forwarding service shall declare its ability to forward by include eBCS Capabilities element (see 9.4.2.bc.2 (eBCS AP UL Capabilities)) in the Beacon and broadcast Probe Response frames that it transmits.

An eBCS AP that supports forwarding and is capable of embedding the requested metadata shall indicate its ability to support embedding by setting the corresponding subfield in the eBCS Capabilities element to 1 and shall append the requested metadata to the content received from the STA before forwarding it to the remote destination. Format TBD.

An eBCS AP that supports forwarding but doesn’t support embedding of the requested metadata shall not forward the frame to the remote destination if the No Forwarding Without Embedding subfield in the eBCS Capabilities element carried in the eBCS UL frame is set to 1. Otherwise the AP shall forward the frame to the remote destination identified in the frame.

An eBCS AP may authenticate the transmitter of the packet before forwarding it to a remote destination and shall provide an indication of the authentication scheme in the eBCS Capabilities element that it transmits. An eBCS AP that does not require authentication of the transmitter shall forward the frame to the remote destination indicated in the frame irrespective of whether the frame carries the STA Certificate field or the Packet Number field or the Frame Signature field. In order to prevent DoS or injection attacks directed towards the remote destination, it is strongly recommended that eBCS APs that support forwarding service perform source authentication and validate the frame signature.

An eBCS AP may limit the amount/frequency of ULs it forwards to a remote destination and shall provide an indication of the throttling scheme in the eBCS Capabilities element that it transmits.

NOTE – Forwarding service is best effort and an eBCS AP, that supports forwarding service, is not required to forward a STA’s data to the destination identified in the STA’s eBCS UL frame if the conditions indicated by the AP (such as authentication and/or throttling) are not satisfied or for other reasons.