IEEE P802.11
Wireless LANs

| MIB and PICS text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-09-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | Self | Southampton, UK |  | stephen.mccann@ieee.org |

Abstract

This document proposes text for the MIB and PICS clauses.

# Annex B

## (normative)

## Protocol Implementation Conformance Statement (PICS) - proforma

## PICS proforma - IEEE Std 802.11-<year>

### IUT configuration

Insert the new entries into B.4.3. as follows

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| CFEBCS | Enhanced Broadcast Services | 11.bc | O | Yes  No  N/A  |

Insert the following new subclause at the end of clause B.4

### B.4.37   Enhanced Broadcast Services (eBCS) features

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following MAC protocol features supported? |  |  |  |
| eBCS1 | eBCS Request and Response Procedure | 11.bc.6.2 | CFEBCS:M | Yes □ No □ N/A □ |
| eBCS2 | eBCS Advertisement Procedure | 11.bc.6.3 | CFEBCS:M | Yes □ No □ N/A □ |
| eBCS3 | ANQP eBCS Procedures | 11.23.3.3.bc | CFEBCS:O | Yes □ No □ N/A □ |

# Annex C

(normative)

## ASN.1 encoding of the MAC and PHY MIB

## C.3 MIB Detail

Change the end of the “Dot11StationConfigEntry” of the “dot11StationConfig TABLE” as follows:

…

dot11EBCSActivated, Truthvalue,

}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11EBCSActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute when true, indicates that the capability of the STA to operate Enhanced Broadcast Services is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <ANA> }

Insert the following compliance statement after the “Compliance Statements – S1G” section:

-- ********************************************************************

-- * Compliance Statements - EBCS

-- ********************************************************************

dot11EBCSComplianceGroup OBJECT-GROUP

OBJECTS {

dot11EBCSActivated

}

STATUS current

DESCRIPTION

"This object group provides the objects from the IEEE 802.11

MIB required to manage Enhanced Broadcast Services functionality."

::= { dot11Groups <ANA> }

dot11EBCSCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11

MIB required to manage Enhanced Broadcast Services functionality."

MODULE -- this module

MANDATORY-GROUPS

{

dot11EBCSComplianceGroup

}

::= { dot11Compliances <ANA> }

1Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.