IEEE P802.11
Wireless LANs

| MIB and PICS text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-10-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | Huawei | Southampton, UK |  | stephen.mccann@ieee.org |

Abstract

This document proposes text for the MIB and PICS clauses.

# [TGbc editor: Replace Annex B with the following text]

# Annex B

## (normative)

## Protocol Implementation Conformance Statement (PICS) - proforma

## PICS proforma - IEEE Std 802.11-<year>

### IUT configuration

Insert the new entries into B.4.3. as follows

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| CFEBCS | Enhanced Broadcast Services | 11.bc | O | Yes  No  N/A  |

Insert the following new subclause at the end of clause B.4

### B.4.37   Enhanced Broadcast Services (eBCS) features

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following MAC protocol features supported? |  |  |  |
| eBCS1 | eBCS Request and Response Procedure | 11.bc.6.2 | CFEBCS:M | Yes □ No □ N/A □ |
| eBCS2 | eBCS Advertisement Procedure | 11.bc.6.3 11.bc.6.3.1 | CFEBCS:M | Yes □ No □ N/A □ |
| eBCS3 | ANQP eBCS Procedures | 11.23.3.3.bc | CFEBCS:M | Yes □ No □ N/A □ |
| eBCS4 | eBCS Uplink Procedure | 11.bc.3 | CFEBCS:O | Yes □ No □ N/A □ |
| eBCS5 | eBCS Termination Notice Procedure | 11.53.1 | CFEBCS:O | Yes □ No □ N/A □ |

# [TGbc editor: Replace Annex C with the following text]

# Annex C

(normative)

## ASN.1 encoding of the MAC and PHY MIB

## C.3 MIB Detail

Add Xiofei’s bits about the termination MIB variables

Change the end of the “Dot11StationConfigEntry” of the “dot11StationConfig TABLE” as follows:

…

dot11eBCSSupportActivated, Truthvalue,

dot11eBCSInfoSequence, Unsigned32,

dot11eBCSInfoInterval, Unsigned32,

dot11eBCSHCFAKeyChangeInterval, Unsigned32,

dot11eBCSHCFAHashDistance, Unsigned32,

dot11eBCSInfoTxRate, Unsigned32,

dot11eBCSDataTxRate, Unsigned32,

dot11eBCSTerminationNoticeTime, Unsigned32,

dot11eBCSTerminationNoticeMinimumInterval, Unsigned32,

dot11eBCSTerminationNoticeMaximumInterval, Unsigned32

}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11eBCSSupportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute when true, indicates that the capability of the STA to operate Enhanced Broadcast Services is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <ANA> }

dot11eBCSInfoSequence OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when <TBD> an MLME-POWERMGT.request primitive is

received.”

DEFVAL {false}

::= { dot11StationConfigEntry <ANA+1> }

dot11eBCSInfoInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the interval of eBCS Info frames in units of 100ms."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+2> }

dot11eBCSHCFAKeyChangeInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the interval of a change to the eBCS HCFA Key Change in units of 10ms."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+3> }

dot11eBCSHCFAHashDistance OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the distance of an eBCS HCFA Hash."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+4> }

dot11eBCSInfoTxRate OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the eBCS transmitted info rate."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+5> }

dot11eBCSDataTxRate OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the eBCS transmitted data rate."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+6> }

dot11eBCSTerminationNoticeTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the eBCS termination notice time in TBTTs."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+7> }

dot11eBCSTerminationNoticeMinimumInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the eBCS termination notice minimum interval in TBTTs."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+8> }

dot11eBCSTerminationNoticeMaximumInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. This attribute indicates the eBCS termination notice maximum interval in TBTTs."

DEFVAL {1}

::= { dot11StationConfigEntry <ANA+9> }

Insert the following compliance statement after the “Compliance Statements – S1G” section:

-- ********************************************************************

-- * Compliance Statements - EBCS

-- ********************************************************************

dot11EBCSComplianceGroup OBJECT-GROUP

OBJECTS {

dot11eBCSSupportActivated,

dot11eBCSInfoSequence,

dot11eBCSInfoInterval,

dot11eBCSHCFAKeyChangeInterval,

dot11eBCSHCFAHashDistance,

dot11eBCSInfoTxRate,

dot11eBCSDataTxRate,

dot11eBCSTerminationNoticeTime,

dot11eBCSTerminationNoticeMinimumInterval,

dot11eBCSTerminationNoticeMaximumInterval

}

STATUS current

DESCRIPTION

"This object group provides the objects from the IEEE 802.11

MIB required to manage Enhanced Broadcast Services functionality."

::= { dot11Groups <ANA> }

dot11EBCSCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11

MIB required to manage Enhanced Broadcast Services functionality."

MODULE -- this module

MANDATORY-GROUPS

{

dot11EBCSComplianceGroup

}

::= { dot11Compliances <ANA> }

1Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.