IEEE P802.11
Wireless LANs

| Proposed resolutions to a few 11az LB249 CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-10-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Apple Inc. |  |  | qi_wang2@apple.com |

Abstract

This submission contains proposals to resolve three 11az LB#249 CIDs on the topic of AoD for passive ranging.

The reference document is 11az_D2.4.

Introduction

Introduction

This submission contains proposals to resolve three 11az LB#249 CIDs on the topic of AoD for passive ranging.

The reference document is 11az_D2.4.

| Comments: CID; Page/Line; Clause; Comment; Proposed change; Resolution. 3850; 55.28; 9.4.2.21.10; " the Number of Antenna subfield" -- no such subfield; Change to " the Number of Selected Antennas subfield"; Revise. “The number of Antenna subfield” should be revised to “The number of Selected Antenna subfield” to be consistent with “Figure 9-256c –Antenna information field format” TGaz editor: make the changes included in submission 20/1654r0 for consistency and clarify.. 3851; 55.28; 9.4.2.21.10; "indicates N Tx_sel" -- N Tx_sel is not used anwyhere; Delete "N Tx_sel,"; Revise. Revise N Tx_sel is used in “Figure 9-256d – Antenna Placement and Calibration field format”. TGaz editor: make the changes included in submission 20/1654r0 to improve clarity.. 3852; 55.28; 9.4.2.21.10; "indicates N Tx_sel" -- the encoding is not clear (without further clarification it would only be able to encode 0 to 3 antennas); Change "the total number of the antennas selected for transmission" to "the total number of antennas selected for transmission minus one"; Revise. TGaz editor: make the changes included in submission 20/1654r0 to improve clarity.. TGaz editors: please make the changes to the text in line 24-26, page 56 of 11az_D2.4 as shown below. 9.4.2.21.10 LCI report (Location configuration information report) … The Antenna Information field is formatted as shown in Figure 9-256c (Antenna Information field format), where the Number of Selected Antenna subfield indicates the total number of the antennas selected for transmission minus one. The total number of the antennas selected for transmission is denoted as NTx_sel, as shown in Figure 9-256d. (CID 3850, 3851, 3852). |  |
| --- | --- |

References:

1. IEEE Draft P802.11az_D2.4, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing