IEEE P802.11
Wireless LANs

| PICS text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-10-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | Huawei | Southampton, UK |  | stephen.mccann@ieee.org |
| Hitoshi Morioka | SRC Software | Fukuoka, JAPAN |  | hmorioka@src-soft.com |

Abstract

This document proposes text for the PICs clause.

# [TGbc editor: Replace Annex B with the following text]

# Annex B

## (normative)

## Protocol Implementation Conformance Statement (PICS) - proforma

## PICS proforma - IEEE Std 802.11-<year>

### IUT configuration

Insert the new entries into B.4.3. as follows

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| * CFEBCS | Enhanced Broadcast Services | 11.bc | O | Yes  No |

Insert the following new subclause at the end of clause B.4

### B.4.37   Enhanced Broadcast Services (eBCS) features

| Item | Protocol capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| * EBCS1 | eBCS Downlink Procedure | 11.bc.2 (eBCS DL procedures) | (CFAP OR CFSTAofAP) AND CFEBCS: O.1 | Yes □ No □ N/A □ |
| EBCS1.1 | eBCS Info frame | 9.6.7.1.bc2 eBCS Info frame format | EBCS1: M | Yes □ No □ N/A □ |
| * EBCS2 | eBCS Uplink Procedure | 11.bc.3 (eBCS UL Service) | (CFAP OR CFSTAofAP) AND CFEBCS: O.1 | Yes □ No □ N/A □ |
| EBCS2.1 | UL eBCS frame | 9.6.7.1.bc1 UL eBCS frame format | EBCS2: M | Yes □ No □ N/A □ |
| * EBCS3 | eBCS frame authentication | 12.bc (Frame authentication for eBCS) | (CFAP OR CFSTAofAP) AND CFEBCS: M | Yes □ No □ N/A □ |
| * EBCS3.1 | PKFA | 12.bc.2 eBCS public key frame authentication (PKFA) | EBCS1 AND EBCS3: O.2 EBCS2 AND EBCS3: O.3 | Yes □ No □ N/A □ |
| EBCS3.1.1 | PKFA MPDU | 12.bc.2.3 PKFA MPDU format) | EBCS3.1: M | Yes □ No □ N/A □ |
| * EBCS3.2 | HCFA | 12.bc.3 eBCS hash chain frame authentication (HCFA) | EBCS1 AND EBCS3: O.2 | Yes □ No □ N/A □ |
| EBCS3.2.1 | HCFA MPDU | 12.bc.3.3 HCFA MPDU generation | EBCS3.2: M | Yes □ No □ N/A □ |
| EBCS3.3 | HLSA | 12.bc.4 No frame authentication with mandatory higher layer source authentication (HLSA) | EBCS1 AND EBCS3: O.2 EBCS2 AND EBCS3: O.3 | Yes □ No □ N/A □ |
| * EBCS4 | eBCS Request and Response Procedure | 11.53 eBCS Negotiation Procedure for Associated STAs | CFAP AND CFEBCS: M CFSTAofAP AND EBCS1: M | Yes □ No □ N/A □ |
| EBCS4.1 | eBCS Request frame | 9.6.31.2 eBCS Request Frame format | EBCS4: M | Yes □ No □ N/A □ |
| EBCS4.2 | eBCS Response frame | 9.6.31.3 eBCS Response Frame format | EBCS4: M | Yes □ No □ N/A □ |
| * EBCS5 | eBCS Termination Notice Procedure | 11.54 eBCS Termination Notice Procedure | CFAP AND CFEBCS: O CFSTAofAP AND EBCS1: M | Yes □ No □ N/A □ |
| EBCS5.1 | eBCS Termination Notice frame | 9.6.7.1.bc3 eBCS Termination Notice frame format | EBCS5: M | Yes □ No □ N/A □ |
| EBCS6 | ANQP eBCS Procedure | 11.23.3.3.bc Enhanced Broadcast Service procedures | CFAP AND CFEBCS: M CFSTAofAP AND CFEBCS: O | Yes □ No □ N/A □ |
| EBCS7 | Transmission Rate Control for eBCS | 10.6.5.3 Rate selection for eBCS frames | (CFAP AND EBCS1) OR (CFSTAofAP AND EBCS2): M | Yes □ No □ N/A □ |

1Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.