IEEE P802.11
Wireless LANs

| Proposed resolution to 11az LB249 CID 3900 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2020-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Apple Inc. |  |  | qi_wang2@apple.com |

Abstract

This submission proposes the resolution to 11az LB#249 CID 3900.

The reference document is 11az_D2.5.

Introduction

Introduction

This submission proposes the resolution to 11az LB#249 CID 3900.

The reference document is 11az_D2.5.

| Comments: CID; Page/Line; Clause; Comment; Proposed change; Resolution. 3900; 75/9; 9.4.2.296; "The Device Class and Full Bandwidth I2R MU-MIMO subfields are defined in Table 9-322b, Subfields of the HE PHY Capabilities Information field. For associated STAs their values are equal to the value of the Device Class and Full Bandwidth UL MU-MIMO subfields respectively that is exchanged during association. (#1103)." The behaviour corresponding to the setting of this field is not described in the spec.; In 11.22.6.4.3.3 (Measurement sounding phase of TB ranging) and 11.22.6.4.4.2 (Non-TB measurement sounding phase), add description of the behaviour corresponding to the setting of this field.; Revised. TGaz Editor: Please make the text changes included in 1817r0.. TGaz editors: please make the changes to the text in line 24-28, page 76 of 11az_D2.5 as shown below. 9.4.2.298 Ranging Parameters element … The Device Class and Full Bandwidth UL MU-MIMO subfields correspond to the Device Class and Full Bandwidth UL MU-MIMO fields (#3490) defined in Table 9-321b (Subfields of the HE PHY Capabilities Information field). For associated STAs the value of the Device Class and Full Bandwidth UL MU-MIMO subfields are equal to the value of the Device Class and Full Bandwidth UL MU-MIMO fields respectively that are exchanged during association. (#1103). The Full Bandwidth UL MU-MIMO field is set to 1 if the TB Ranging Responder field in the Extended Capabilities element is set to 1 or if the Ranging Parameters element in the IFTMR includes a TB Specific subelement. Otherwise, the Full Bandwidth UL MU-MIMO field is set to 1 or 0. TGaz editors: please make the changes to the text in line 31-38, page 124 of 11az_D2.5 as shown below. 11.21.6.3.3 Negotiation for TB and Non-TB Ranging measurement exchange For TB and Non-TB Ranging Measurement exchange the IFTMR frame shall have: — the Trigger field set to 1, — a set of range measurement parameters in a Ranging Parameters element that describe the ISTA’s availability for measurement exchange. (#1478) For TB and Non-TB Ranging measurement exchange the initial Fine Timing Measurement frame shall include a Ranging Parameters element containing either the Non-TB Specific subelement or the TB Specific subelement. (#1479) If the Ranging Parameters element in the IFTMR includes a TB Specific subelement, the Full Bandwidth UL MU-MIMO field in the Ranging Parameters element in the IFTMR shall be set to 1. Otherwise, the Full Bandwidth UL MU-MIMO field in the Ranging Parameters element in the IFTMR may be set to either 1 or 0. |  |
| --- | --- |

References:

1. IEEE Draft P802.11az_D2.5, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing