IEEE P802.11
Wireless LANs

| Proposed resolution to 11az LB249 CID 3900 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021-01-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Apple Inc. |  |  | qi_wang2@apple.com |
| Ali Raissinia | Qualcomm |  |  | alirezar@qti.qualcomm.com |

Abstract

This submission proposes the resolution to 11az LB#249 CID 3900.

The reference document is 11az_D2.6.

Introduction

Introduction

This submission proposes the resolution to 11az LB#249 CID 3900.

The reference document is 11az_D2.6.

| Comments: CID; Page/Line; Clause; Comment; Proposed change; Resolution. 3900; 75/9; 9.4.2.296; "The Device Class and Full Bandwidth I2R MU-MIMO subfields are defined in Table 9-322b, Subfields of the HE PHY Capabilities Information field. For associated STAs their values are equal to the value of the Device Class and Full Bandwidth UL MU-MIMO subfields respectively that is exchanged during association. (#1103)." The behaviour corresponding to the setting of this field is not described in the spec.; In 11.22.6.4.3.3 (Measurement sounding phase of TB ranging) and 11.22.6.4.4.2 (Non-TB measurement sounding phase), add description of the behaviour corresponding to the setting of this field.; Revised. TGaz Editor: Please make the text changes included in 1817r1.. TGaz editors: please modify the text on page 144 of 11az_D2.6 as shown below. 11.21.6.4.3 TB Ranging measurement exchange 11.21.6.4.3.1 General … During the availability window, measurement resources and results are made available to each ISTA whose poll response was received at the RSTA; see 11.21.6.4.3.3 (Measurement Sounding phase of TB Ranging) and 11.21.6.4.3.4 (Reporting Phase of TB Ranging Measurement). (#2156) This may also lead to extra instances of polling/sounding/reporting triplets, even if all ISTAs assigned to this availability window were polled in the first Polling phase instance (e.g., if the RSTA is not able to accommodate all ISTAs that responded in a single measurement sounding phase instance; see 11.21.6.4.3.3 (Measurement sounding phase of TB Ranging). An RSTA shall not transmit a trigger sounding frame soliciting an I2R NDP that uses UL MU-MIMO within an RU that spans the full bandwidth to an ISTA from which it has not received a Ranging Parameters element with the Full Bandwidth UL MU-MIMO subfield of the Ranging Parameters field of the Ranging Parameters element equal to 1. … |  |
| --- | --- |

References:

1. IEEE Draft P802.11az_D2.6, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications, Amendment 8: Enhancements for locationing