IEEE P802.11
Wireless LANs

| CR for CIDs related to NSTR Capability signalling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021-02-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dibakar Das | Intel |  |  | Dibakar.das@intel.com |
| Laurent Cariou |  |  |  | Laurent.cariou@intel.com |
| Minyoung Park |  |  |  | Minyoung.Park@intel.com |
| Po-kai Huang |  |  |  | Po-kai.huang@intel.com |
| Dmitry Akhmetov |  |  |  | Dmitry.akhmetov@intel.com |
| Dan Bravo |  |  |  |  |
| Yunbo Li | Huawei |  |  |  |
| Young Hoon Kwon | NXP |  |  |  |
| Zhiqiang Han | ZTE |  |  |  |
| Alfred Asterjadhi | Qualcomm |  |  |  |
| Gaurang Naik |  |  |  |  |

Abstract

This document proposes to resolve the following CIDs: 1078, 1475, 2981.

Rev0: initial version

Rev1: accommodate comments from Yunbo and Young Hoon

REV2: accommodate comments from Zhiqiang: clarify the index of the bitmap solution.

Rev6: updates based on comments received during the call

| CID | Page | Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1078 | 142 | 29 | 35.3.13.4 | Fix the TBD | Add a (sub)field in the common portion of multi-link element (basic variant) to signal the MLD's STR capabilities with respect to a pair of links | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r6 |
| 1475 | 142 | 29 | 35.3.13.4 | In R1 timeframe an EHT non-AP STA MLD just needs to signal which of its STA/link pairs are STR/NSTR | Allow a non-AP MLD to signal its STR/NSTR capabilities as a bitmap in the Per STA profile of an ML element. | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r6 |
| 2981 | 142 | 29 | 35.3.13.4 | To provide information on NSTR link pair, I think the Multi-Link element is the right place. And to minimize the field length, it may be better to set the maximum number of NSTR link pairs. | As in comment. | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r6 |

Discussion:

All the comments are related about how to signal the NSTR/STR link pair capability which is currently missing in draft 0.3. We propose the following simplified signalling:

- Have a bit in Common Info field to signal if there is at least one pair of NSTR link pairs among the set of STA profiles signalled in the Basic variant ML element when the MLD is a multi-radio MLD.

- If that bit is set to 1, include a bitmap inside STA info of the ML element that contains the STR/NSTR link pair signalling. Basically, each STA profile included in the Basic ML element will contain a bitmap that signals the STR/NSTR indication for each pair of links of which that STA Profile (/link) is a member. The bit positions are indexed based on the relative link IDs.

- The Common Info field contains a subfield representing #of radios (“ Maximum Number Of Simultaneous Links” in Figure 9-788ex) which is set to greater than or equal to 1 for a multi-radio MLD.

- The Common Info field contains a subfield indicating whether the bitmaps are 1 or 2 octets long. Note that per the format of Link ID field, there can be upto 16 STA-profiles overall.

- The signalling is symmetric that is, if link pairs (1,2) are NSTR, then (2,1) is clearly NSTR as well.

- Optimize the signalling in Common Info field by including some MLD-specific info in a separate field inside the Common Info field of the ML element.

TGbe editor: Revise Figure 9-788eg and the text starting in P74L20 of draft 0.3 as follows:

B0                Bx           Bx+1                       Bx+2          Bx+3  B15

| Type | MLD MAC Address Present | MLD Capabilities Present | Reserved |
| --- | --- | --- | --- |

Bits:                x+1                     1                                   1                   13-x

Figure 9-788eg—Multi-Link Control field (#1078, 1475, 2981)

The MLD MAC Address Present subfield is set to 1 if the MLD MAC Address field is present in the Common Info field. Otherwise the subfield is set to 0.

The MLD Capabilities Present subfield is equal to 1 if the MLD Capabilities subfield is present in the Common Info field. Otherwise the subfield is equal to 0 (#1078, 1475, 2981).

TGbe editor: Revise Figure 9-788eh and the text starting in P74L59 of draft 0.3 as follows:

9.4.2.295b.2 Basic variant Multi-Link element (#1078, 1475, 2981)

| MLD MAC Address | MLD Capabilities |
| --- | --- |

Octets:               0 or 6                  0 or 2

Figure 9-788eg—Common Info field of the Basic variant Multi-Link element format

The condition for the presence of the MLD Capabilities subfield in the Common Info field is defined in 35.3.13.4 (Capability signalling). The format of the MLD Capabilities field is defined in Figure 9-788eh1 (MLD Capabilities field).

B0               B3                        B4                       B5                 B8 B9                B15

| Maximum Number Of Simultaneous Links | NSTR Link-Pair Present | NSTR Bitmap Size | Reserved |
| --- | --- | --- | --- |

Bits:                4                        1                                       1                         10

Figure 9-788eh1- MLD Capabilities field format

The subfields of the MLD Capabilities fields are defined in Table 9-322xy (Subfields of the MLD Capabilities field)

Table 9-322xy- Subfields of the MLD Capabilities field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| Maximum Number Of Simultaneous Links | Indicates the maximum number of affiliated STAs in the MLD that support simultaneous transmission or reception of frames | Set to the maximum number of affiliated STAs in the MLD that support simultaneous transmission or reception of frames minus 1. |
| NSTR Link-Pair Present | Indicates if at least one NSTR link pair is present at the MLD | Set to 1 if the value of the Maximum Number Of Simultaneous Links field is greater than 0 and the MLD contains at least one NSTR link pair. Set to 0 otherwise. |
| NSTR Bitmap Size | Indicates the length of NSTR Indication Bitmap | If the NSTR Link-Pair Present subfield is 1: Set to 0 if the if the length of NSTR Indication Bitmap is 1 octet. Set to 1 if the if the length of NSTR Indication Bitmap is 2 octets. Reserved if the NSTR Link-Pair Present subfield is 0. |

Other fields are TBD.

TGbe editor: Add Figure 9-788eh and modify the text starting in P74L56 of draft 0.3 as follows:

The Complete Profile subfield is set to 1 when the Per-STA Profile subelement of the Multi-Link element is complete as defined in 35.3.2.2 (Complete or partial per-STA profile). Otherwise the subfield is set to 0.

If the Complete Profile subfield is equal to 1 and the NSTR Link-Pair Present subfield is equal to 1 in the MLD Capabilities field, then the second subfield in the Per-STA Profile field is the NSTR Indication Bitmap field defined in Figure 9-788ey (NSTR Indication Bitmap). Otherwise, the NSTR Indication Bitmap field is not present. When the NSTR Indication Bitmap field is present its length is 2 octets if the NSTR Bitmap Size subfield value in the MLD Capabilities field is equal to 1; otherwise, its length is 1 octet.

Each bit Bj  (j≠i)in the NSTR Indication Bitmap subfield included in the Per-STA Profile with Link ID subfield equal to i is set to 1 if the link pair corresponding to link IDs equal to i and j is NSTR and the Basic variant ML element contains a Per-STA Profile with link ID j; otherwise it is set to 0. The bit Bi in the NSTR Indication Bitmap subfield included in the Per-STA Profile with Link ID subfield equal to i is reserved.

B0                        B7 or B15

NSTR Indication Bitmap

Octets:                1 or 2

Figure 9-788ey- NSTR Indication Bitmap

Other subfields are TBD.

TGbe editor: Revise Figure 35-1 in P128L6 of draft 0.3 as follows:

Figure 35-1—Illustration of Basic variant Multi-Link element carrying a complete per-STA profile

TGbe editor: Add the following text before the paragraph starting in P142L38 of draft 0.3 as follows:

35.3.13.4 Capability signalling (#1078, 1475, 2981)

An MLD shall set the MLD Capabilities Present subfield in the Multi-Link Control field of the Basic variant Multi-Link element to 1 when carried in a (Re-)Association Request frame and (Re-)Association Response frames.

An MLD shall set the MLD Capabilities Present subfield in the Multi-Link Control field of the Basic variant Multi-Link element to 0 when carried in an Authentication frame.

An AP MLD shall set the Maximum Number Of Simultaneous Links subfield value to be greater than or equal to that of the number of Per-STA Profiles included in the Basic variant Multi-Link element containing the MLD Capabilities subfield.

A non-AP MLD shall set the NSTR Link-Pair Present subfield value to 1 only if it is a multi-radio MLD and contains at least one NSTR link pair; otherwise it shall set the subfield value to 0. An AP MLD shall set the NSTR Link-Pair Present subfield value to 0.

An MLD shall set to 0 every bit in the NSTR Indication Bitmap subfield that corresponds to a link-pair where one of the STAs in the link pair operates in the 2.4 GHz band and the other STA operates in the 5 GHz or 6 GHz band.

An MLD that sets the NSTR Link-Pair Present field to 1 may optionally include additional information to describe the NSTR constraints for link pairs that are signalled to be NSTR.