IEEE P802.11
Wireless LANs

| EHT-SIG CR on P802.11be D0.3 – part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.02.09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

9 comments under 36.3.11.8.1 General:

CID 1377

1378

1622

1970

3049

3184

3185

3293

3294

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Changes during presentation |

## CID 1377

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.02 | 36.3.11.8.1 | Definition of "equitable split" is weak | Try "as the deterministic split of User fields across EHT-SIG CCs, given the absence of any Common field.". | REVISED Agreed in principle. An equation of equitable split same as 11ax has been added in 11/21-220r1 for the description in the later subclause. Moreover, further add more description regarding the BW. Instructions to the editor Please make the changes as shown in 11/21-0235r1 |

Discussion

Instructions to the editor

Please make the changes to L02, P244 as shown below:

For OFDMA transmission (in U-SIG, the UL/DL field is set to 0, and the PPDU Type And Compression Mode field is set to 0), when the Bandwidth of the PPDU is greater or equal to 40 MHz, dynamic split is defined as the split of User fields across EHT-SIG content channels according to the Common field in each EHT-SIG content channel.

For non-OFDMA transmission to multiple users (in U-SIG, the UL/DL field is set to 0, and the PPDU Type And Compression Mode field is set to 2), when the Bandwidth of the PPDU is greater or equal to 40 MHz, equitable split is defined as the split of User fields across EHT-SIG content channels.

For non-OFDMA transmission to single user (in U-SIG, the UL/DL field is set to either 0 or 1, and the PPDU Type And Compression Mode field is set to 1), when the Bandwidth of the PPDU is greater or equal to 40 MHz, the only User field is repeated across EHT-SIG content channels.

## CID 1378

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.15 | 36.3.11.8.1 | "PE Related parameters" is undefined | Enumerate these mystical parameters | REVISED Note to the editor: the comment has been resolved in the PDT 11/21-140r2 and no further changes are needed. |

## CID 1622

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.09 | 36.3.11.8.1 | It would be better to describe the User field for EHT sounding NDP as well even though there is no user field in that case. | See the comment. | REVISED Agreed in principle Instructions to the editor Please make the changes as shown in 11/21-0235r1 |

Instructions to the editor

Please add the paragraph to L9, P244 as shown below:

For EHT Sounding NDP (in U-SIG, the UL/DL field is set to either 0 or 1, and the PPDU Type And Compression Mode field is set to 1), there is no User Field.

## CID 1970

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.14 | 36.3.11.8.1 | The following fields shall be the same in every 80 MHz segment: --GI+EHT-LTF Size --Number Of EHT-LTF Symbols --PE Related Parameters | Forcing "Number of EHT-LTF Symbols" seems uncessary. Of course there could be some PAPR enhancement. Suggest to remove Number of EHT-LTF Symbols in this clause. | REJECTED For 1x, 2x EHT-LTF, the EHT-LTF field shall be aligned. For 4x EHT-LTF, alignment of the EHT-LTF is helpful for PAPR reduction and easy for implementation. |

## CID 3049

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 243.11 | 36.3.11.8.1 | "The following fields shall be the same in every 80 MHz segment" change to "The following fields shall be the same in every 80 MHz segment in one PPDU" to compatible with A-PPDU in R2 | as commented | REVISED The comment has been resolved in the PDT 11/21-140r2 Note to the editor: The description is now within the corresponding subfield. No further changes are needed. |

## CID 3185

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.02 | 36.3.11.8.1 | "equitable split" does not seem to be used anywhere else in D0.3, or in 11-21/0140r2. So, why are we defining this? Also, the meaning of the definition at P244L2 is not clear. | Delete the paragraph at P244L1-4. | REVISED Note to the editor: An equation of equitable split same as 11ax has been added in 11/21-220r1 for the description in the later subclause. It is used for the case of non-OFDMA MU-MIMO transmission. No further changes are needed. |

## CID 3184

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 243.53 | 36.3.11.8.1 | Only EHT MU PPDU has EHT-SIG | Change "interpret the EHT PPDU" to "interpret the EHT MU PPDU" | ACCEPTED |

## CID 3293

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 243.28 | 36.3.11.8.1 | In "For OFDMA transmission", OFDMA should be DL OFDMA since UL/DL field is set to 0 | as in comment | ACCEPTED |

## CID 3294

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 244.01 | 36.3.11.8.1 | In "For non-OFDMA transmission", OFDMA should be DL non-OFDMA since UL/DL field is set to 0 | as in comment | REVISED Agreed in principle. Instructions to the editor Please add DL before non-OFDMA in Page 244, L1 |