IEEE P802.11
Wireless LANs

| EHT-SIG CR on P802.11be D0.3 – part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.02.19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Edward Au | Huawei Technologies |  |  |  |
| Stephen McCann | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

41 out of 42 comments under 36.3.11.8.3 Common field for OFDMA transmission:

CID

1394	1395	1396	1397	1398	1400	1402	1403	1564	1565

1623	2404	2405	2406	2407	2408	2409	2410	2682	2683

2684	2685	2810	2811	2934	3107	3186	3188	3192	3193

3295	3296	3297	3298	3299	3300	3301	3302	3303	3304

3305

are resolved in this document.

CID 3005 will be resolved later.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Add is referred to for 484+242-tone MRU for CID 1398. Modify the meaning of ‘[]’ within a large MRU or RU for CID 2934 etc. Change accepted to revised, add note to the editor for some resolutions where no further changes are needed. |

## CID 1394

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.18 | 36.3.11.8.3 | "Each RU Allocation subfield but we have an "RU Allocation-1" subfield here | Try ""Each RU Allocation-1 subfield ...". Ditto P249L21 ". | REVISED Agreed in principle. The specific changes for both 245.18 and 249.21 are reflected. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L18, P248 as shown below:

Each RU Allocation-1 subfield in an EHT-SIG content channel corresponding to a 20 MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain. It also indicates information needed to compute the number of users allocated to each RU, where the subcarrier indices of the RU(s) meet the conditions in Table 36-25 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth).

Please make the changes to L21, P249 as shown below:

Each RU Allocation subfield-2 in an EHT-SIG content channel corresponding to a 20 MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain. It also indicates information needed to compute the number of users allocated to each RU, where the subcarrier indices of the RU(s) meet the conditions in Table 36-25 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth).

## CID 1395

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.23 | 36.3.11.8.3 | "in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain" does not capture the notion that this RU Allocation-xx subfield applies to its 20M and overlapping RUs only (i.e. not the whole EHT MU PPDU). | Work in a reference to Table 36-25 (like 11ax). Ditto P249L26 | REVISED Agreed in principle. The changes are given after the discussion with the commenter. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L23, P248 as shown below:

Each RU Allocation-1 subfield in an EHT-SIG content channel corresponding to a 20 MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain., where the subcarrier indices of the RU(s) meet the conditions in Table 36-25 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth). Each RU Allocation-1 subfield also indicates information needed to compute the number of users allocated to each of these RU(s).

Please make the changes to L26, P249 as shown below:

Each RU Allocation-2 subfield in an EHT-SIG content channel corresponding to a 20 MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain., where the subcarrier indices of the RU(s) meet the conditions in Table 36-25 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth). Each RU Allocation-2 subfield also indicates information needed to compute the number of users allocated to each of these RU(s).

## CID 1396

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 252.60 | 36.3.11.8.3 | Should be "-3" in "[-244: 3]" | Should be "-3" in "[-244: 3]" | ACCEPTED |

## CID 1397

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.42 | 36.3.11.8.3 | "The CRC-2 is calculated over bits 27+9N and 26+9N+9M" is incorrect | Should be "The CRC-2 is calculated over bits 27+9N to 26+9N+9M" | ACCEPTED |

## CID 1398

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.60 | 36.3.11.8.3 | Label in "labeled from the first RU Allocation subfield to the seventh RU Allocation subfield" wihtout specifying an ordering | Indicate that first is lowest in freq, and higher labels are higher in freq. Ditto for the following paragraphs in P249-250 | REVISED Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L60, P249 as shown below:

A 3x996+484-tone MRU is referred to by seven RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The seven RU Allocation subfields per EHT-SIG content channel are labeled from the first RU Allocation subfield to the seventh RU Allocation subfield in an increasing frequency order.

A 3x996-tone MRU is referred to by six RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The six RU Allocation subfields per EHT-SIG content channel are labeled from the first RU Allocation subfield to the sixth RU Allocation subfield in an increasing frequency order.

A 2x996+484-tone MRU is referred to by five RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The five RU Allocation subfields per EHT-SIG content channel are labeled from the first RU Allocation subfield to the fifth RU Allocation subfield in an increasing frequency order.

A 2x996 tone RU is referred to by four consecutive RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The four RU Allocation subfields per EHT-SIG content channel are labeled from the first RU Allocation subfield to the fourth RU Allocation subfield in an increasing frequency order.

A 996+484-tone MRU is referred to by three RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The three RU Allocation subfields per EHT-SIG content channel are labeled from the first RU Allocation subfield to the third RU Allocation subfield in an increasing frequency order.

A 996-tone RU is referred to by two consecutive RU Allocation subfields per EHT-SIG content channel, for both EHT-SIG content channels. The two consecutive RU Allocation subfields per EHT-SIG content channel are labeled the first RU Allocation subfield and the second RU Allocation subfield in an increasing frequency order.

A 484+242-tone MRU is referred to by two RU allocation subfields in the EHT-SIG content channel that overlaps with the 242-tone RU and one RU allocation subfield in the other EHT-SIG content channel. The two RU Allocation subfields in the EHT-SIG content channel (with two RU Allocation subfields) are labeled the first RU Allocation subfield and the second RU Allocation subfield in increasing frequency order.

A 484-tone RU is referred to by a single RU Allocation subfield per EHT-SIG content channel, for both EHT-SIG content channels.

## CID 2404

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.58 | 36.3.11.8.3 | "3x996+484-tone RU" needs to be changed to "3x996+484 tone MRU". | As in the comment | ACCEPTED |

## CID 2405

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.1 | 36.3.11.8.3 | "3x996-tone RU" needs to be changed to "3x996-tone MRU". | As in the comment | ACCEPTED |

## CID 2406

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.6 | 36.3.11.8.3 | "2x996+484 tone RU" needs to be changed to "2x996+484 tone MRU". | As in the comment | ACCEPTED |

## CID 2407

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.15 | 36.3.11.8.3 | "996+484 tone RU" needs to be changed to "996+484 tone MRU". | As in the comment | ACCEPTED |

## CID 2408

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.25 | 36.3.11.8.3 | "484+242 tone RU" needs to be changed to "484+242 tone MRU". | As in the comment | ACCEPTED |

## CID 2409

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.38 | 36.3.11.8.3 | The sentence of "Smaller RUs are referred to by a single RU Allocation subfield in a single EHT-SIG content channel." can be modified as "RU/MRU of size smaller than or equal to 242-tone RU is referred to by a single RU Allocation subfield in a single EHT-SIG content channel." for the clarification. | As in the comment | ACCEPTED |

## CID 3303

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.38 | 36.3.11.8.3 | smaller RUs is ambiguous. Add the RU size instead of smaller RUs | as in comment | REVISED The sentence of "Smaller RUs are referred to by a single RU Allocation subfield in a single EHT-SIG content channel." can be modified as "RU/MRU of size smaller than or equal to 242-tone RU is referred to by a single RU Allocation subfield in a single EHT-SIG content channel." for the clarification. Note to the editor: please be noted that the resolution for CID 3303 is the same as CID2409 in 11/21-0236r1 |

## CID 1400

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | Notation "[]" should be defined in a note at the foot of the table | Notation "[]" should be defined in a note at the foot of the table | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0236r1 Note to the editor: the resolutions for CID 1400, CID 2811, CID 2934, CID 3193 in 11/21-0236r1 are the same. |

## CID 2811

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | Notation "MRU of []-242-484" is not clear. Similar comments on other lines of the Table. | Improve notation | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0236r1 Note to the editor: the resolutions for CID 1400, CID 2811, CID 2934, CID 3193 in 11/21-0236r1 are the same. |

## CID 2934

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | In Table 36-26, there is no definition of what the brackets [] mean, e.g. "MRU of []-242-484". | Please add the definition | REVISED Agreed in principle. Reflect the detailed explaintation. Instructions to the editor Please make the changes as shown in 11/21-0236r1 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 in 11/21-0236r1 are the same. |

## CID 3193

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.61 | 36.3.11.8.3 | What does the notation "[]-242-484" mean? | Define the notation. | REVISED Instructions to the editor Please make the changes as shown in 11/21-0236r1 Note to the editor: The resolutions for CID 1400, CID 2811, CID 2934, CID 3193 in 11/21-0236r1 are the same. |

Instructions to the editor

Please make the changes to L56, P256 as shown below:

Table 36-26—RU Allocation subfield (continued)

| 296-303 (100101y2y1y0) | MRU of 996-996-484-[] | 8 |
| --- | --- | --- |
| 304-511 (100110y2y1y0 -111111y2y1y0) | Disregard | 268 |
| If signaling RUs or MRUs of a size greater than or equal to 242 subcarriers, y2y1y0 = 000–111 indicates the number of User fields in the EHT-SIG content channel that contains the corresponding 9-bit RU Allocation subfield. The binary vector y2y1y0 indicates Nuser(r, c) = 22 × y2 + 21 × y1 + y0 + 1 User fields in the EHT-SIG content channel that contains the corresponding 9-bit RU Allocation subfield. [] indicates one or multiple RU/MRUs that are not overlapped with the RU or MRU indicated by the 9-bit RU allocation subfield and is to help to indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. |  |  |

## CID 1402

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 257.48 | 36.3.11.8.3 | "differ in the number of User fields per RU" is incomplete since, for wider RUs, the number of user fields per RU is the sum across content channels | Remove or clarify "per RU" e.g. change to "differ in the number of User fields indicated" or "differ in the number of User fields indicated by this EHTSIGB content channel per RU" or similar | REVISED Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L48, P257 as shown below:

In Table 36-26 (RU Allocation subfield), the number of entries column refers to the number of RU Allocation subfield values that refer to the same RU assignment to be used in the frequency domain but differ in the number of User fields indicated by this EHT-SIG content channel per RU. The number of User fields per RU indicated by the RU Allocation subfields of an EHT-SIG content channel indicate the number of User fields in the User Specific field of the EHT-SIG content channel.

## CID 1403

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 257.54 | 36.3.11.8.3 | For say an ru wider than 80MHz, it is unclear if different 80MHz segments can perform a different dynamic split. | Add something like "The dynamic split may be different per 80MHz frequency subblock". Also refresh this para in the sense tha we don't have just two EHTSIG content channels if they can change every 80MHz: we have min(BW,40)/20M)*max(1,BW/80M) | REVISED Agreed in principle with the first part of the proposed change. Reflect the detailed changes. There are 2CCs per 80MHz for OFDMA transmission and can be different per 80MHz.The spec (page 244 L21) has mentions this clearly. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L54, P257 as shown below:

For an MU-MIMO allocation of RU/MRU size greater than 242 subcarriers in an OFDMA transmission, the dynamic split of User fields between EHT-SIG content channel 1 and EHT-SIG content channel 2 per 80 MHz is decided by the AP (on a per case basis) and signaled by the AP using the RU Allocation subfields in each EHT-SIG content channel. The dynamic split may be different per 80 MHz frequency subblock.

## CID 1564

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 247.15 | 36.3.11.8.3 | The value of the Spatial Reuse field for an HE SU PPDU, HE ER SU PPDU, and HEMU PPDU is defined in table 27-22. and the value of SR defined in 11ax can be re-used for MU-PPDU of 11be. | add the following text to the description in a row " Spatial Reuse" of Table 36-24. " Set to a value from Table 27-22 (Spatial Reuse field encoding for an HE SU PPDU, HE ER SU PPDU, and HE MU PPDU), see 26.11.6 (SPATIAL_REUSE) and 26.10 (Spatial reuse operation). | REVISED Note to the editor: the proposed changes have been reflected in 11/21-140r2 and no further changes are needed. |

## CID 1565

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 258.15 | 36.3.11.8.3 | The value of the Spatial Reuse field for an HE SU PPDU, HE ER SU PPDU, and HEMU PPDU is defined in table 27-22. and the value of SR defined in 11ax can be re-used for MU-PPDU of 11be | add the following text to the description in a row " Spatial Reuse" of Table 36-27 " Set to a value from Table 27-22 (Spatial Reuse field encoding for an HE SU PPDU, HE ER SU PPDU, and HE MU PPDU), see 26.11.6 (SPATIAL_REUSE) and 26.10 (Spatial reuse operation)." | REVISED Note to the editor: the proposed changes have been reflected in 11/21-140r2 and no further changes are needed. |

## CID 1623

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 247.15 | 36.3.11.8.3 | Add a detailed description or refer to a table for each value in the Spatial Reuse subfield. | See the comment. | REVISED Note to the editor: the proposed changes have been reflected in 11/21-140r2 and no further changes are needed. |

## CID 2410

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 251.29 | 36.3.11.8.3 | The sentence of "For a 996-tone RU, the second 9-bit RU Allocation subfield referring to the RU shall be set to 30 (000011110 in binary representation), which encodes zero additional User fields in the corresponding content channel." and "The RU Allocation subfield corresponding to 996-tone RU in large-size MRU combinations of 996+484-tone MRU, 2x996+484-tone MRU, 3x996+484-tone MRU, 3x996-tone-MRU, and&#65440;2x996-tone RU is set to 30 (000011110 in binary representation) to indicate the zero users, which encodes zero additional User fields in the corresponding content channel." can be merged. | Delete the sentence of "For a 996-tone RU, the second 9-bit RU Allocation subfield referring to the RU shall be set to 30 (000011110 in binary representation), which encodes zero additional User fields in the corresponding content channel." and modify the sentence of " The RU Allocation subfield corresponding to 996-tone RU in large-size MRU combinations of 996+484-tone MRU, 2x996+484-tone MRU, 3x996+484-tone MRU, 3x996-tone-MRU, and 2x996-tone RU is set to 30 (000011110 in binary representation) to indicate the zero users, which encodes zero additional User fields in the corresponding content channel" as follows: The RU Allocation subfield corresponding to 996-tone RU in large-size MRU combinations of 996+484 tone MRU, 2x996+484 tone MRU, 3x996+484 tone MRU, and 3x996-tone MRU and in RUs of 996-tone RU and 2x996-tone RU is set to 30 (000011110 in binary representation) to indicate the zero users, which encodes zero additional User fields in the corresponding content channel. | ACCEPTED |

Discussion

The commenter tries to merge the first and fourth subbullet

## CID 2682

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 247.18 | 36.3.11.8.3 | In 11ax, the GI+LTF indication is interpretated as: " set to 0 to indicate 1x HE-LTF and 0.8 μs GI set to 1 to indicate 2x HE-LTF and 0.8 μs GI set to 2 to indicate 2x HE-LTF and 1.6 μs GI set to 3 to indicate 4x HE-LTF and 3.2 μs GI " It would be easier just to replace 1xLTF+0.8 with 4xLTF+0.8 | Change to the "GI+LTF size" mapping to " Indicates the GI duration and EHT-LTF size: Set to 0 to indicate 4xLTF + 0.8 μs GI. Set to 1 to indicate 2xLTF + 0.8 μs GI. Set to 2 to indicate 2xLTF + 1.6 μs GI. Set to 3 to indicate 4xLTF + 3.2 μs GI." | REJECTED The current order is to have 2x first, then 4x. Within 2x and 4x, smaller GI first, then larger GI, which is also very easy to interprete. |

## CID 2683

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.33 | 36.3.11.8.3 | CRC and Tail bits are independently added for each BCC block, and not need to have index of 1 and 2 for different BCC blocks. | Change "CRC-1" to "CRC", and change "Tail-1" to "Tail" | REVISED Reflect the changes also for CRC-2 and Tail-2 Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

## CID 2684

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.35 | 36.3.11.8.3 | CRC and Tail bits are independently added for each BCC block, and not need to have index of 1 and 2 for different BCC blocks. | Change "CRC-2" to "CRC", and change "Tail-2" to "Tail" | REVISED Also reflected other related changes Note to the editor: the resolutions for CIDs 2683, 2684, 3107 are the same. |

## CID 3107

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.08 | 36.3.11.8.3 | In Table 36-24, Bit range B27+9N-B26+9N+9M is better represented as "B45 - B44+9M" since RU allocation-2 subfield exsits only when N = 2. Please make the change "B27+9N-B26+9N+9M" to "B45 - B44+9M". Same applies to CRC-2 and Tail-2 fields | As in comment | REVISED The commenter is correct regarding N=2. Add a note to reflect that. However, directly use B45 may make readers confuse about the bits index. Note to the editor: the resolutions for CIDs 2683, 2684, 3107 are the same. |

Instructions to the editor

Please make the changes to L33, P248 as shown below:

| B17+9N –B20+9N. | CRC. | 1 | 4 | The CRC is calculated over bits 0 to 16+9N.. |
| --- | --- | --- | --- | --- |
| B21+9N –B26+9N. | Tail | 1 | 6 | Used to terminate the trellis of the convolutional decoder. Set to 0.. |

Instructions to the editor

Please make the changes to L35, P249 as shown below:

| B27+9N+9M –B30+9N+9M.. | CRC. | 1 | 4 | The CRC subfield is present if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU and not present otherwise. If present, the CRC is calculated over bits 27+9N to 26+9N+9M Note -- N=2 when the CRC subfield exists... |
| --- | --- | --- | --- | --- |
| B31+9N+9M –B36+9N+9M.. | Tail | 1 | 6 | The Tail subfield is present if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU and not present otherwise. If present, then it is used to terminate the trellis of the convolutional decoder. Set to 0. Note -- N=2 when the Tail subfield exists... |

## CID 2810

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.08 | 36.3.11.8.3 | "subfields are present" and "M=0" is not consistent. If M=0, the subfields are not present. | change to "subfield is present if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU" | REVISED Agreed in principle. Reflected the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0236r1 |

Instructions to the editor

Please make the changes to L08, Page 249 as shown in 11/21-0236r1

| B27+9N - B26+9N+9M | RU Allocation-2 | M | 9 | M RU Allocation-2 subfields are present in an EHT-SIG content channel if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU where M is equal to 2 or 6 as follows: M=2 if the Bandwidth field in the U-SIG field is 3 (indicating a 160MHz EHT MU PPDU) M=6 if the Bandwidth field in the U-SIG field is 4 or 5 (indicating a 320MHz-1 or 320MHz-2 EHT MU PPDU) and are not present otherwise. Each RU Allocation subfield in an EHT-SIG content channel corresponding to a 20MHz frequency segment indicates the RU assignment, including the size of the RU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain. It also indicates information needed to compute the number of users allocated to each RU, where the subcarrier indices of the RU(s) meet the conditions in Table 36-22 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth). |
| --- | --- | --- | --- | --- |

## CID 2685

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 256.51 | 36.3.11.8.3 | 304~511 are defined the same as other large MRU with the last 3-bits being Num of users. To match the Num of user description in the text below, it will be helpful to add a bracket indicating the location of y2y1y0. | Change "304-511" to "304-511 (100110y2y1y0-111111y2y1y0)" | REVISED Note to the editor: the proposed changes have been reflected in 11/21-140r2 and no further changes are needed. |

Discussion: the changes in 11/21-140r2 are as follows

## CID 3186

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 245.36 | 36.3.11.8.3 | The description of Common field for OFDMA is hard to read/understand. Note that the Common field for OFDMA is clearly defined in Table 36-24 anyway. | Change "In OFDMA transmission, the Common field of the EHT-SIG ... The Common field for OFDMA transmission is defined in 36.3.11.8.3." to "The Common field for OFDMA transmission is defined in Table 36-24." | ACCEPTED |

Discussion:

The commenter tries to remove the following highlighted part:

## CID 3188

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 247.18 | 36.3.11.8.3 | Comment on 11-21/0140r2. Table 36-24 is a 'format', so should not contain normative language. | Delete "The values shall be the same in different 80MHz subblocks" from all subfields in Table 36-24. After Table 36-24, add "Value for the following subfields in the Common field for OFDMA transmission have the same value in every EHT-SIG Content Channel: - GI+LTF Size - Number Of EHT-LTF Symbols - LDPC Extra Symbol Segmet - Pre-FEC Padding Factor - PE Disambiguity" Also, make similar changes to Table 36-27 ahd 36-23a. | ACCEPTED Note to the editor: please be noted that Table 36-23a in 11-21/0140r2 is indexed as Table 36-28 in P802.11be D0.3 |

## CID 3192

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 251.19 | 36.3.11.8.3 | In P249L58 ~ P250L49, the term was "first RU Allocation subfield", not "first 9-bit RU Allocation subfield". | Change "the first 9-bit RU Allocation subfield" to "the first RU Allocation subfield" | ACCEPTED |

## CID 3295

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.08 | 36.3.11.8.3 | RU Allocaton-1 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0236r1 Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

## CID 3296

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.33 | 36.3.11.8.3 | CRC-1 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

## CID 3297

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 248.37 | 36.3.11.8.3 | Tail-1 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

## CID 3298

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.8 | 36.3.11.8.3 | RU Allocaton-2 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

## CID 3299

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.35 | 36.3.11.8.3 | CRC-2 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

## CID 3300

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.35 | 36.3.11.8.3 | Tail-2 should be reflected in the related figures such as Figure 36-35, or Figure 36-36, ... and add it is the first encoding block in description | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Note to the editor: the proposed resolutions for CID3295, 3296, 3297,3298, 3299, 3300 in 11/21-0236r1 are the same |

Instructions to the editor

Please make the changes to L54, P249 as shown below:

B0-B16 of Table 36-24 are U-SIG Overflow bits for OFDMA transmission and are duplicated in each content channels.

B0–B26+9N of Table 36-24 composes the first Encoding Block. B27+9N–B36+9N+9M composes the second Encoding Block if present.

Instructions to the editor

Please replace the existing figures with the new figures (Page 44 L46) as shown below:

Figure 36-35 (EHT-SIG content channel format for OFDMA transmission if BW is 20/40/80 MHz)

Figure 36-36 (EHT-SIG content channel format for OFDMA transmission if BW is 160 MHz)

Figure 36-37 (EHT-SIG content channel format for OFDMA transmission if BW is 320 MHz)

## CID 3301

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.55 | 36.3.11.8.3 | 4x996 tone RU should be 4x996-tone RU | as in comment | ACCEPTED |

## CID 3302

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 249.55 | 36.3.11.8.3 | cannot" should be "is not" | as in comment | REJECTED Similar language is used in 11ax (page 599, L46 of P802.11ax D8.0): If a Common field is present in a 160 MHz or 80+80 MHz PPDU, a 2×996 tone RU is not permitted and cannot be indicated by the RU allocation subfield. |

## CID 3304

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 251.22 | 36.3.11.8.3 | "y2y1y0 indicating the number of User fields" is not technically correct. It should indicate the number of User fields - 1. or reuse the N_user(r,c) | as in comment | REVISED Note to the editor: the proposed changes have been reflected in 11/21-220r1 and no further changes are needed. |

Discussion: Changes in 11/21-220r1 are:

## CID 3305

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 256.53 | 36.3.11.8.3 | "y2y1y0 = 000-111 indicates the number of User fields" is not technically correct. It should indicate the number of User fields - 1. or delete the sentence because the same content is repeated with N_user(r,c) at the bottom of the table. | as in comment | REVISED Note to the editor: the proposed changes have been reflected in 11/21-220r1 and no further changes are needed.. |

CID 3005, TBD

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 255.96 | 36.3.11.8.3 | Rather than using []-242-484, 242-[]-484, etc, better to use MRU index. | See comment. |  |