IEEE P802.11
Wireless LANs

| CR on some CIDs in P802.11be D0.3 U-SIG sub-clause 36.3.11.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.02.17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following comments on P802.11be D0.3 clause 36.3.11.7.2 (“Content” sub-clause in the U-SIG clause):

1369, 1616, 1992, 2444, 2632, 2669, 2798, 2799, 3289

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1369

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 235.08 | 36.3.11.7.2 | "Puncturing pattern" does not have ordering defined | Add "See NOTE 1". At the foot of the table, create a new row in the table and write "NOTE 1- "Parameters from left to right refer to 20MHz or 40MHz channels in order of increasing frequency" or similar. Also, move the note at P236L1 into this final row of the table as NOTE too and upgrade the column header for puncturing pattern to "See NOTE 1 and 2". Then, likely just merge the two notes. | Revised: Modify Table 36-21 as follows: Upgrade the column header from “Puncturing pattern” to "Puncturing pattern (See NOTEs 1 and 2)". create a new row at the foot of the table and add the following text to it: "NOTE 1— In the puncturing patterns a “1” denotes a nonpunctured subchannel and an “x” denotes a punctured subchannel. The puncturing granularity for 80 MHz and 160 MHz PPDU bandwidth is 20 MHz, and the puncturing granularity for 320 MHz PPDU bandwidth is 40 MHz. NOTE 2 — Parameters from left to right refer to 20 MHz or 40 MHz subchannels in the order of increasing frequency" Remove the note at P236L1 Note to editor: Same resolution for CIDs 1369 & 2633 |

## CID 1616

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 235.50 | 36.3.11.7.2 | Change "320-80-40" to "40 MHz and 80 MHz puncturing". | See the comment. | Revised In P235L49, change the text from “320-80-40” to “80 MHz + 40 MHz puncturing” Note to editor: Same resolution for CIDs 1616, 1992, 2799 and 3289 |

## CID 1992

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 235.50 | 36.3.11.7.2 | Change "320-80-40" to "40 MHz and 80 MHz puncturing". | See the comment. | Revised In P235L49, change the text from “320-80-40” to “80 MHz + 40 MHz puncturing” Note to editor: Same resolution for CIDs 1616, 1992, 2799 and 3289 |

## CID 2444

| Page Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 233.46 | 36.3.11.7.2 | In Table 36-20, EHT-TB is noted as to be used with UL-OFDMA. However, EHT-TB may also be transmitted using non-OFDMA over the entire BW | In the note, change from "UL OFDMA..." to "UL OFDMA or non-OFDMA...” | Accepted |

## CID 2632

| Page Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 232.39 | 36.3.11.7.2 | For the OFDMA case, '1111' constitutes a valid puncturing combination (no puncturing), but is not listed as one of the valid puncturing patterns for U-SIG2 B3-B7. As described currently, '1111' will result in "Validate" RX behavior. | Table 36-19 (U-SIG field of an EHT MU PPDU), edit description of "Punctured Channel Information" as follows: The following allowed punctured patterns are defined for an 80 MHz segment: 1111, 0111, 1011, 1101, 1110, 0011, 1100, and 1001. | Accepted. |

## CID 2633

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 36.3.11.7.2 | 236.01 | Order of the "Puncturing pattern" column in table 36-21 (5-bit punctured channel indication for the non-OFDMA case in an EHT MU PPDU) needs to be clarified - whether left-to-right indicates lowest (negative) to highest (positive) frequency, or vice-versa. | Add clarification in the "NOTE" immediately after Table 36-21 | Revised. Note to editor: Same resolution for CIDs 1369 & 2633 |

## CID 2669

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 36.3.11.7.2 | 236.07 | Propose to add puncturing info frequency order | Proposal: "Puncuting pattern in table 36-21 indicates puncture info from low to high frequency (left to right)" | Accepted. Note to editor: No further edits are needed as this is already explained in the puncturing description of table 36-19 |

## CID 2798

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 232.52 | 36.3.11.7.2 | "Maybe used for an expanded set of puncturing modes in future releases or amendments". This is more like an Editor's note and should not be in the spec text. | Remove | Accepted |

## CID 2799

| Page Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 235.49 | 36.3.11.7.2 | Notation"320-80-40" is not clear. | Change to e.g. "80+40 MHz puncturing" to be consistent with rest of the table. | Revised In P235L49, change the text from “320-80-40” to “80 MHz + 40 MHz puncturing” Note to editor: Same resolution for CIDs 1616, 1992, 2799 and 3289 |

## CID 3289

| Page Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 235.49 | 36.3.11.7.2 | use better wording for 320-80-40 | as in comment | Revised In P235L49, change the text from “320-80-40” to “80 MHz + 40 MHz puncturing” Note to editor: Same resolution for CIDs 1616, 1992, 2799 and 3289 |