IEEE P802.11
Wireless LANs

| CR on P802.11be D0.3 preamble puncturing subclause |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.02.16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |
| Ross Jian Yu | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following comments on P802.11be D0.3 subclause 36.3.11.11 (Preamble punctured EHT PPDU):

1585, 1586, 1952, 2707, 2985

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Some modifications based on comments given in the April-26th CC |

## CID 1585

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 286.28 | 36.3.11.11.2 | For UL OFDMA (EHT TB PPDU), puncturing information is not included in U-SIG or EHT-SIG. Describe the signaling of puncturing information by separating DL and UL. | See the comment. | Revised Instruction to the editor: change the beginning of 36.3.12.11.2 in D0.4 to: “Preamble puncturing may exist in PPDUs transmitted to one or more users using OFDMA transmission. For MU PPDU, U-SIG and EHT-SIG include information on the preamble puncturing. |

## CID 1586

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 286.36 | 36.3.11.11.2 | Specify which preamble puncturing patterns are allowed and which are mandatory cases for OFDMA. | See the comment. | Revised Instruction to the editor: add the following text at the end of 36.3.12.11.2 in D0.4 (P390L16): “The following allowed punctured patterns are defined for an 80 MHz subblock: 1111, 0111, 1011, 1101, 1110, 0011, 1100, and 1001. The puncturing pattern may vary for different 80 MHz frequency subblocks. In these patterns an ‘1’ denotes a nonpunctured 20 MHz subchannel and a ‘0’ denotes a punctured 20 MHz subchannel” |

## CID 1952

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 286.01 | 36.3.11.11 | Preamble punctured EHT PPDU subclause should not be part of the EHT preamble subclasue | Move to appropriate position, e.g., after 36.3.14 Non-HT duplicate transmission | Revised Instruction to the editor: In D0.4, Change "36.3.12.11 Preamble Punctured EHT PPDU" to "36.3.12.11 EHT Preamble of Preamble Punctured EHT MU PPDU". |

## CID 2707

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 286.06 | 36.3.11.11.1 | "Preamble puncturing" is a confusing term since it sounds like only the preamble part is punctured. It is better to use "subchannel puncturing". In fact, in Line 12 of this page, it is defined as "transmission of a PPDU in which no signal is present in at least one 20 MHz subchannel within the PPDU BW." | Change "preamble puncturing" to "subchannel puncturing" | Rejected The term “preamble puncturing” was voted for in a SP, see “Terminology discussion on preamble puncturing” in https://mentor.ieee.org/802.11/dcn/20/11-20-1093-10-00be-minutes-for-tgbe-phy-ad-hoc-cc-july-to-sept-2020.docx Page 68-69 |

## CID 2985

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 156.1.1 | 36.1.1 | In P156L11, "Transmission of an OFDMA EHT MU PPDU with any preamble puncturing pattern as specified in 36.3.11.11 (Preamble punctured EHT PPDU) but excluding any pattern needed to support mandatory MRU for non-OFDMA as specified in 36.3.2.3.3 (Large size multiple RUs)." But there is no preamble puncturing pattern specified in 36.3.11.11. | Add all supported puncturing pattern in section 36.3.11.11.2. | Revised For non-OFDMA all supported puncturing patterns are clearly specified in the Large-MRU sub-clause and a reference to this sub-clause is given, so there is no point in specifying them again. For the allowed patterns for OFDMA, we propose in CID 1586 to add the following text at the end of 36.3.12.11.2 in D0.4 (P390L16): “The following punctured patterns are supported for an 80 MHz subblock: 1111, 0111, 1011, 1101, 1110, 0011, 1100, 1001 and 0000. The puncturing pattern may vary for different 80 MHz frequency subblocks. In these patterns an ‘1’ denotes a nonpunctured 20 MHz subchannel and a ‘0’ denotes a punctured 20 MHz subchannel” Note to the editor: The allowed patterns for OFDMA are resolved in CID 1586 above (because they are specified only in a description of a field in table 36-27, hence not emphasized enough) |