IEEE P802.11
Wireless LANs

| EHT-SIG CR on P802.11be D0.3 – Annex Z EHT-SIG examples |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Xiaoyang Wang |  |  |  |  |
| Mengshi Hu |  |  |  |  |
| Ming Gan |  |  |  |  |
| Wei Jiang |  |  |  |  |
| Renjie Li |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

2 comments under 36.3.11.8.6 Encoding and modulation are resolved.

CID 3055, 3063

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Specify 2SS clearly in the examples |
| R2 | One CC for example 6 |

## CID 3055

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 250.25 | 36.3.11.8.6 | "A 484+242-tone RU by two RU allocation subfields" missed "is refered to". In addition, highly recommond add an example for this case as what we did in 11ax annex z. | as commented | REVISED Agreed in principle Instructions to the editor Please make the changes as shown in 11/21-0350r2 |

## CID 3063

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 257.56 | 36.3.11.8.6 | Add example not only for MUMIMO but also MRU. It will be helpful for people implement EHT-SIG | as commented | REVISED Agreed in principle Instructions to the editor Please make the changes as shown in 11/21-0350r2 Note that the resolutions for CID 3055 and CID3063 are the same. |

Instructions to the editor

Please make the changes to L25, P250 as shown below:

A 484+242-tone MRU is referred to by two RU allocation subfields in the EHT-SIG content channel that overlaps with the 242-tone RU and one RU allocation subfield in the other EHT-SIG content channel. The two RU Allocation subfields in the EHT-SIG content channel (with two RU Allocation subfields) are labeled the first RU Allocation subfield and the second RU Allocation subfield

Instructions to the editor

Please add the following to L55, P246 as shown below:

The examples of EHT-SIG are shown in Annex Z.

Instructions to the editor

Please make the changes to L11, P811 as shown below:

Annex Z

(informative)

HE-SIG-B and EHT-SIG content examples

This annex provides a number of examples to illustrate the content of HE-SIG-B content channels and the content of EHT-SIG content channels.

HE-SIG-B content channels are padded to the same length and to an OFDM symbol boundary as defined in 27.3.11.8.5 (Encoding and modulation). For illustration simplicity, the examples do not include the complete padding bits but only pad to make the two HE-SIG-B content channels equal in length and an integer number of 4 bits. All the padding bits in the examples are set to 0.

In the examples, the binary sequence of each HE-SIG-B field is in transmission order. For the entire content of each HE-SIG-B content channel, the binary sequences are converted to hexadecimal listed in transmission order as pairs of hexadecimal digits, where the first bit transmitted is the LSB of the first transmitted octet and is the LSB of the second hexadecimal digit.

EHT-SIG content channels are padded to the same length and to an OFDM symbol boundary as defined in 36.3.11.8.6 (Encoding and modulation). For illustration simplicity, the examples do not include the complete padding bits but only pad to make different EHT-SIG content channels equal in length and an integer number of 4 bits. All the padding bits in the examples are set to 0.

In the examples, the binary sequence of each EHT-SIG field is in transmission order. For the entire content of each EHT-SIG content channel, the binary sequences are converted to hexadecimal listed in transmission order as pairs of hexadecimal digits, where the first bit transmitted is the LSB of the first transmitted octet and is the LSB of the second hexadecimal digit.

Z.2 HE-SIG-B Example 1

Z.3 HE-SIG-B Example 2

Z.4 HE-SIG-B Example 3

Z.5 HE-SIG-B Example 4

Please add the following at the end of Annex Z:

Note that Annex Z is in P802.11ax D8.0

# Z.6  EHT-SIG Example 1

An example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for an 80 MHz OFDMA transmission using EHT MU PPDU are shown in Table Z-8 and Table Z-9 respectively.

Table Z-8 U-SIG Overflow example 1

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 001 | 2 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |

Table Z-9 Resource allocation signaling example 1

| RU/MRU | 484+242-tone MRU 2 (242-[]-484) | 242-tone RU 2 |
| --- | --- | --- |
| SS1 | STA-ID 1441, EHT-MCS10, LDPC, no beamforming | STA-ID 1442, EHT-MCS 4, BCC, 2SS, Tx beamforming |
| SS2 | N/A |  |

The illustration of a RU allocation subfield is given in Table Z-10:

Table Z-10 RU allocation subfield illustration example 1

|  | RU Allocation subfield illustration |  |
| --- | --- | --- |
| Content channel 1 | 242-[]-484, MRU 2, (1 User field) | 484 (0 User field) |
| Content channel 2 | 242 (1 User field) | 484 (0 User field) |

The User field for STA 1441 is in content channel 1 while User field for STA 1442 is in content channel 2. The content of the entire EHT-SIG field for this example is shown in Table Z-11.

Table Z-11—EHT-SIG content for example 1

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+ RU allocation-1 subfield+CRC+Tail) | 1111 11 100 1 10 0 1111 000101100 101110000 1000 000000 |  | 1111 11 100 1 10 0 1111 000000100 101110000 1000 000000 |  |
| User Specific field | STA 1441 | 10000101101 0101 1 0000 0 1 | STA 1442 | 01000101101 0010 1 1000 1 0 |
|  | CRC and Tail | 1010 000000 | CRC and Tail | 0001 000000 |
|  | Padding | 000 | Padding | 000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111110 01100111 10001011 00101110 00010000 00000100 00101101 01011000 00110100 00000000 |  | 11111110 01100111 10000001 00101110 00010000 00000010 00101101 00101100 01000010 00000000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 01111111 11100110 11010001 01110100 00001000 00100000 10110100 00011010 00101100 00000000 |  | 01111111 11100110 10000001 01110100 00001000 01000000 10110100 00110100 01000010 00000000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | 7F E6 D1 74 08 20 B4 1A 2C 00 |  | 7F E6 81 74 08 40 B4 34 42 00 |  |

# Z.6  EHT-SIG Example 2

An example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for an 80 MHz OFDMA transmission using EHT MU PPDU are shown in Table Z-12 and Table Z-13 respectively.

Table Z-12 U-SIG Overflow example 2

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 011 | 6 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |

Table Z-13 Resource allocation signaling example 2

| RU/MRU | 484+242-tone MRU 2 (242-[]-484) | 242-tone RU 2 |
| --- | --- | --- |
| SS1 | STA-ID 1441, EHT-MCS 10, LDPC, 2SS | STA-ID 1442, EHT-MCS 4, BCC, 2SS, Tx beamforming |
| SS2 |  |  |
| SS3 | STA-ID 1443, EHT-MCS 7, LDPC | N/A |
| SS4 | STA-ID 1445, EHT-MCS 5, LDPC |  |

The illustration of a RU allocation subfield is given in Table Z-14:

Table Z-14 RU allocation subfield illustration example 2

|  | RU Allocation subfield illustration |  |
| --- | --- | --- |
| Content channel 1 | 242-[]-484, MRU 2, (2 User fields) | 484 (0 User field) |
| Content channel 2 | 242 (1 User field) | 242-[]-484, MRU 2, (1 User field) |

The AP can perform a dynamic split of the User fields for the three MU-MIMO STAs on 484+242-tone MRU 2, with zero, one, two or three User fields assigned to EHT-SIG content channel 1 and three, two, one or zero User fields assigned to EHT-SIG content channel 2 correpsondingly. In this example, there are two User fields assigned to EHT-SIG content channel 1 and one User field assigned to EHT-SIG content channel 2, to avoid a disparity in the number of User fields between content channels.

The User fields for STA 1441 and STA 1443 are in content channel 1 while User fields for STA 1445 and STA 1442 are in content channel 2. The content of the entire EHT-SIG field for this example is shown in Table Z-15.

Table Z-15—EHT-SIG content for example 2

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+ RU allocation-1 subfield+CRC+Tail) | 1111 11 110 1 10 0 1111 100101100 101110000 1110 000000 |  | 1111 11 110 1 10 0 1111 000000100 000101100 1011 000000 |  |
| User Specific field | STA 1441 | 10000101101 0101 1 100000 | STA 1442 | 01000101101 0010 1 1000 1 0 |
|  | STA 1443 | 11000101101 1110 1 100000 | STA 1445 | 10100101101 1010 1 100000 |
|  | CRC and Tail | 1011 000000 | CRC and Tail | 1110 000000 |
|  | Padding | 0 | Padding | 0 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111111 01100111 11001011 00101110 00011100 00000100 00101101 01011100 00011000 10110111 10110000 01011000 0000 |  | 11111111 01100111 10000001 00000101 10010110 00000010 00101101 00101100 01010100 10110110 10110000 01110000 0000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 11111111 11100110 11010011 01110100 00111000 00100000 10110100 00111010 00011000 11101101 00001101 00011010 0000 |  | 11111111 11100110 10000001 10100000 01101001 01000000 10110100 00110100 00101010 01101101 00001101 00001110 0000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | FF E6 D3 74 38 20 B4 3A 18 ED 0D 1A 00 |  | FF E6 81 A0 69 40 B4 34 2A 6D 0D 0E 00 |  |

# Z.7  EHT-SIG Example 3

An example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for a 160 MHz OFDMA transmission using EHT MU PPDU are shown in Table Z-16 and Table Z-17, respectively.

Table Z-16 U-SIG overflow example 3

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 001 | 2 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |

Table Z-17 Resource allocation signaling example 3

| RU/MRU | 996+484-tone MRU 2 (484-[]-996) | 484-tone RU 2 |
| --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS10, LDPC, no Tx Beamforming | STA-ID 1442, EHT-MCS 4, LDPC, 2SS, Tx beamforming |
| SS1 | N/A |  |

The illustrations of a RU allocation subfield for the lower 80MHz and upper 80MHz are in Table Z-18 and Z-19 respectively:

Table Z-18 RU allocation subfield illustration for the lower 80 MHz example 3

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 484+[]+996, MRU 2, (1 User field) | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |
| Content channel 2 | 484 (0 User field) | 484 (1 User field) | 996 (0 User field) | 996 (0 User field) |

Table Z-19 RU allocation subfield illustration for the upper 80 MHz example 3

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 484 (0 User field) | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |
| Content channel 2 | 484 (0 User field) | 484 (0 User field) | 996 (0 User field) | 996 (0 User field) |

The EHT-SIG content channels per 80 MHz are allowed to carry different information when EHT MU PPDU is wider than 80 MHz and for OFDMA transmission and for non-OFDMA transmission to multiple users. In this example, STA 1441 and STA 1442 are operating on the primary 80 MHz channel, which is the lower 80 MHz in this example. The User field for STA 1441 is in content channel 1 while the User field for STA 1442 is in content channel 2 in the lower 80 MHz. No User field exists in the upper 80 MHz. The contents of the entire EHT-SIG field in the lower 80 MHz and higher 80 MHz for this example are shown in Table Z-20 and Z-21, respectively. The EHT-SIG content channels per 80 MHz can also be the same.

Table Z-20—EHT-SIG content in the lower 80 MHz for example 3

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+2 RA-1 subfields+CRC+Tail+2 RA-2 subfields+CRC+Tail) | 1111 11 100 1 10 0 1111 000100010 101110000 0010 000000 011110000 011110000 0101 000000 |  | 1111 11 100 1 10 0 1111 101110000 000100100 0000 000000 011110000 011110000 0101 000000 |  |
| User Specific field | STA 1441 | 10000101101 0101 1 0000 0 1 | STA 1442 | 01000101101 0010 1 1000 1 1 |
|  | CRC and Tail | 1010 000000 | CRC and Tail | 0001 000000 |
|  | Padding | 000 | Padding | 000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111110 01100111 10001000 10101110 00000100 00000011 11000001 11100000 10100000 01000010 11010101 10000011 01000000 0000 |  | 11111110 01100111 11011100 00000100 10000000 00000011 11000001 11100000 10100000 00100010 11010010 11000110 00100000 0000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 01111111 11100110 00010001 01110101 00100000 11000000 10000011 00000111 00000101 01000010 10101011 11000001 00000010 0000 |  | 01111111 11100110 00111011 00100000 00000001 11000000 10000011 00000111 00000101 01000100 01001011 01100011 00000100 0000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | 7F E6 11 75 20 C0 83 07 05 42 AB C1 02 00 |  | 7F E6 3B 20 01 C0 83 07 05 44 4B 63 04 00 |  |

Table Z-21—EHT-SIG content in the upper 80 MHz for example 3

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+2 RA-1 subfields+CRC+Tail+2 RA-2 subfields+CRC+Tail) | 1111 11 100 1 10 0 1111 101110000 101110000 1011 000000 011110000 011110000 0101 000000 |  | 1111 11 100 1 10 0 1111 101110000 101110000 1011 000000 011110000 011110000 0101 000000 |  |
| User Specific field | padding | 00000000 00000000 00000000 00000000 000 | padding | 00000000 00000000 00000000 00000000 000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111110 01100111 11011100 00101110 00010110 00000011 11000001 11100000 10100000 00000000 00000000 00000000 00000000 0000 |  | 11111110 01100111 11011100 00101110 00010110 00000011 11000001 11100000 10100000 00000000 00000000 00000000 00000000 0000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 01111111 11100110 00111011 01110100 01101000 11000000 10000011 00000111 00000101 00000000 00000000 00000000 00000000 0000 |  | 01111111 11100110 00111011 01110100 01101000 11000000 10000011 00000111 00000101 00000000 00000000 00000000 00000000 0000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | 7F E6 3B 74 16 C0 83 07 05 00 00 00 00 00 |  | 7F E6 3B 74 16 C0 83 07 05 00 00 00 00 00 |  |

# Z.8  EHT-SIG Example 4

Another example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for a 160 MHz OFDMA transmission using EHT MU PPDU are shown in Table Z-22 and Table Z-23, respectively.

Table Z-22 U-SIG overflow example 4

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 011 | 6 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |

Table Z-23 Resource allocation signaling example 4

| RU/MRU | 242-tone RU 1 | 484+242 MRU 1 ([]-242-484) | 484+242 MRU 7 (484+[]+242) | 242-tone RU 7 |
| --- | --- | --- | --- | --- |
| SS0 | STA-ID 1441, EHT-MCS 8, BCC | STA-ID 1445, EHT-MCS 10, LDPC, 2SS | STA-ID 1447, EHT-MCS 4, LDPC, 2SS, Tx beamforming | STA-ID 1448, EHT-MCS 7, LDPC |
| SS1 | STA-ID 1442, EHT-MCS 7, BCC |  |  | STA-ID 1449, EHT-MCS 6, LDPC |
| SS2 | STA-ID 1443, EHT-MCS 6, BCC | STA-ID 1446, EHT-MCS 4, LDPC | N/A | STA-ID 1450, EHT-MCS 5, LDPC |
| SS3 | STA-ID 1444, EHT-MCS 5, BCC |  |  | STA-ID 1451, EHT-MCS 5, LDPC |

The illustrations of a RU allocation subfield for the lower 80 MHz and upper 80 MHz are in Table Z-24 and Z-25 respectively:

Table Z-24 RU allocation subfield illustration for the lower 80 MHz example 4

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 242 (4 User fields) | 484 (0 User field) | 484 (0 User field) | 242 (0 User field) |
| Content channel 2 | []-242-484, MRU 1 (2 User fields) | 484 (0 User field) | 484 (0 User field) | 242 (0 User field) |

Table Z-25 RU allocation sibfield illustration for the upper 80 MHz example 4

|  | RU Allocation subfield illustration |  |  |  |
| --- | --- | --- | --- | --- |
| Content channel 1 | 242 (0 User field) | 484 (0 User field) | 484 (0 User field) | 242 (4 User fields) |
| Content channel 2 | 242 (0 User field) | 484 (0 User field) | 484+[]+242, MRU 7 (1 User field) | 242 (0 User field) |

The EHT-SIG content channels per 80 MHz are allowed to carry different information when EHT MU PPDU is wider than 80 MHz and for OFDMA transmission and for non-OFDMA transmission to multiple users. In this example, STAs 1441-1446 are operating on the primary 80 MHz channel, which is the lower 80MHz in this example, while STAs 1447-1451 are operating on the secondary 80 MHz channel, which is the upper 80 MHz in this example. The User fields for STAs 1441-1444 are in content channel 1 while the User fields for STAs 1445-1446 are in content channel 2 in the lower 80 MHz. The User fields for STAs 1448-1451 are in content channel 1 while the User fields for STA 1447 is in content channel 2 in the upper 80 MHz. The contents of the entire EHT-SIG field in the lower 80 MHz and upper 80 MHz for this example are shown in Table Z-26 and Z-27, respectively. The EHT-SIG content channels per 80 MHz can also be the same.

Table Z-26—EHT-SIG content in the lower 80 MHz for example 4

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+2 RA-1 subfields+CRC+Tail+2 RA-2 subfields+CRC+Tail) | 1111 11 110 1 10 0 1111 110000100 101110000 1000 000000 101110000 001110000 0110 000000 |  | 1111 11 110 1 10 0 1111 100001100 101110000 1011 000000 101110000 001110000 0110 000000 |  |
| User Specific field | STA 1441 | 10000101101 0001 0 000000 | STA 1445 | 10100101101 0101 1 001000 |
|  | STA 1442 | 01000101101 1110 0 000000 | STA 1446 | 01100101101 0010 1 001000 |
|  | CRC and Tail | 0011 000000 | CRC and Tail | 1100 000000 |
|  | STA 1443 | 11000101101 0110 0 000000 | Padding | 00000000 00000000 00000000 00000000 00000000 00000000 00000000 0 |
|  | STA 1444 | 00100101101 1010 0 000000 |  |  |
|  | CRC & Tail | 0010 000000 |  |  |
|  | Padding | 000 |  |  |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111111 01100111 11100001 00101110 00010000 00000101 11000000 11100000 11000000 01000010 11010001 00000000 10001011 01111000 00000001 10000001 10001011 01011000 00000001 00101101 10100000 00000100 00000000 |  | 11111111 01100111 11000011 00101110 00010110 00000101 11000000 11100000 11000000 01010010 11010101 10010000 11001011 01001010 01000110 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 11111111 11100110 10000111 01110100 00001000 10100000 00000011 00000111 00000011 01000010 10001011 00000000 11010001 00011110 10000000 10000001 11010001 00011010 10000000 10110100 00000101 00100000 00000000 |  | 11111111 11100110 11000011 01110100 01101000 10100000 00000011 00000111 00000011 01001010 10101011 00001001 11010011 01010010 01100010 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | FF E6 87 74 08 A0 03 07 03 42 8B 00 D1 1E 80 81 D1 1A 80 B4 05 20 00 |  | FF E6 C3 74 68 A0 03 07 03 4A AB 09 D3 52 62 00 00 00 00 00 00 00 00 |  |

Table Z-27—EHT-SIG content in the upper 80 MHz for example 4

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| Common field (U-SIG Overflow+2 RA-1 subfields+CRC+Tail+2 RA-2 subfields+CRC+Tail) | 1111 11 110 1 10 0 1111 001110000 1011100001101 000000 101110000 110000100 1011 000000 |  | 1111 11 110 1 10 0 1111 001110000 101110000 1101 000000 000011100 001110000 1010 000000 |  |
| User Specific field | STA 1448 | 00010101101 1110 1 000000 | STA 1447 | 11100101101 0010 1 1000 1 1 |
|  | STA 1449 | 10010101101 0110 1 000000 | CRC and Tail | 0000 000000 |
|  | CRC and Tail | 1110 000000 | Padding | 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 0000000 |
|  | STA 1450 | 01010101101 1010 1 000000 |  |  |
|  | STA 1451 | 11010101101 1010 1 000000 |  |  |
|  | CRC and Tail | 1010 000000 |  |  |
|  | Padding | 000 |  |  |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111111 01100111 10011100 00101110 00011010 00000101 11000011 00001001 01100000 00001010 11011110 10000001 00101011 01011010 00000111 00000000 10101011 01101010 00000110 10101101 10101000 00010100 00000000 |  | 11111111 01100111 10011100 00101110 00011010 00000000 01110000 11100001 01000000 01110010 11010010 11000110 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 11111111 11100110 00111001 01110100 01011000 10100000 11000011 10010000 00000110 01010000 01111011 10000001 11010100 01011010 11100000 00000000 11010101 01010110 01100000 10110101 00010101 00101000 00000000 |  | 11111111 11100110 00111001 01110100 01011000 00000000 00001110 10000111 00000010 01001110 01001011 01100011 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | FF E6 9C 2E 58 A0 C3 90 06 50 7B 81 D4 5A E0 00 D5 56 60 B5 15 28 00 |  | FF E6 9C 2E 58 00 0E 87 02 4E 4B 63 00 00 00 00 00 00 00 00 00 00 00 |  |

# Z.9  EHT-SIG Example 5

An example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for an 80 MHz DL non-OFDMA MU-MIMO transmission using EHT MU PPDU are shown in Table Z-28 and Table Z-29 respectively.

Table Z-28 U-SIG Overflow example 5

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 11 | 4x LTF + 3.2 μs GI |
|  | Number Of EHT-LTF Symbols | 010 | 4 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |
|  | Number of Non-OFDMA User | 010 | 3 non-OFDMA users |

Table Z-29 Resource allocation signaling example 5

| RU/MRU | 484+242-tone MRU 2 (242-[]-484) |
| --- | --- |
| SS1 | STA-ID 1441, EHT-MCS 10, LDPC, 2SS |
| SS2 |  |
| SS3 | STA-ID 1443, EHT-MCS 7, LDPC |
| SS4 | STA-ID 1445, EHT-MCS 5, LDPC |

The AP performs an equitable split of the User fields for the three MU-MIMO STAs on 484+242-tone MRU 2, with two User fields assigned to EHT-SIG content channel 1 and one User field assigned to EHT-SIG content channel 2. The User fields for STA 1441 and STA 1443 are in content channel 1 while User field for STA 1445 is in content channel 2. The content of the entire EHT-SIG field for this example is shown in Table Z-30.

Table Z-30—EHT-SIG content for example 5

|  | EHT-SIG content channel 1 |  | EHT-SIG content channel 2 |  |
| --- | --- | --- | --- | --- |
| First encoding block (U-SIG Overflow+Number of Non-OFDMA users + 1st User field +CRC+Tail) | 1111 11 010 1 10 0 1111 010 10000101101 0101 1 100000 1110 000000 |  | 1111 11 010 1 10 0 1111 010 10100101101 1010 1 100000 0101 000000 |  |
| User Specific field except for the 1st User field | STA 1443 | 11000101101 1110 1 100000 | Padding | 0000 0000 0000 0000 0000 0000 000 0000 |
|  | CRC&Tail | 0010 000000 |  |  |
|  | Padding | N/A |  |  |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11111101 01100111 10101000 01011010 10111000 00111000 00001100 01011011 11011000 00001000 0000 |  | 11111101 01100111 10101010 01011011 01011000 00010100 00000000 00000000 00000000 0000000 0000 |  |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 10111111 11100110 00010101 01011010 00011101 00011100 00110000 11011010 00011011 00010000 0000 |  | 10111111 11100110 01010101 11011010 00011010 00101000 00000000 00000000 00000000 0000000 000 |  |
| EHT-SIG field content in hexadecimal, organized as octets | BF E6 15 5A 1D 1C 30 DA 1B 10 0 |  | 2F F9 95 76 86 8A 00 00 00 00 0 |  |

# Z.10  EHT-SIG Example 6

An example of the EHT-SIG field with U-SIG Overflow and resource allocation signaling for an 80 MHz DL non-OFDMA SU transmission using EHT MU PPDU are shown in Table Z-31 and Table Z-32 respectively.

Table Z-31 U-SIG Overflow example 6

| U-SIG Overflow | Subfield | Indication | Meaning |
| --- | --- | --- | --- |
|  | Spatial Reuse | 1111 | PSR_AND_NON_SRG_OBSS_PD_PROHIBITED |
|  | GI+LTF Size | 10 | 4x LTF + 0.8 μs GI |
|  | Number Of EHT-LTF Symbols | 001 | 2 EHT-LTF symbols |
|  | LDPC Extra Symbol Segment | 1 | An LDPC extra symbol segment is present. |
|  | Pre-FEC Padding Factor | 01 | A pre-FEC padding factor of 1 |
|  | PE Disambiguity | 0 | The condition in Equation (36-80) is not met |
|  | Disregard | 1111 |  |
|  | Number of Non-OFDMA User | 000 | One non-OFDMA user |

Table Z-32 Resource allocation signaling example 6

| RU/MRU | 484+242-tone MRU 2 (242-[]-484) |
| --- | --- |
| SS1 | STA-ID 1441, EHT-MCS10, LDPC, 2SS, Tx beamforming |
| SS2 |  |

For non-OFDMA transmission to single user, there exists one content channel.The content of the entire EHT-SIG field for this example is shown in Table Z-30.

Table Z-30—EHT-SIG content for example 6

|  | EHT-SIG content channel 1 |
| --- | --- |
| First encoding block (U-SIG Overflow+Number of Non-OFDMA users + 1st User field +CRC+Tail) | 1111 01 100 1 10 0 1111 000 10000101101 0101 1 1000 1 1 0001 000000 |
| EHT-SIG field content in binary, organized as octets (LSB first) | 11110110 01100111 10001000 01011010 10111000 11000100 0000 |
| EHT-SIG field content in binary, organized as octets (MSB first within each octet) | 01101111 11100110 00010001 01011010 00011101 00100011 0000 |
| EHT-SIG field content in hexadecimal, organized as octets | 6F E6 11 5A 1D 23 0 |