IEEE P802.11
Wireless LANs

| EHT-LTF CR on P802.11be D0.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chenchen Liu | Huawei Technologies | Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | liuchenchen1@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

19 comments under 36.3.11.10:

CID 1413

CID 1568

CID 1569

CID 1570

CID 1584

CID 1630

CID 1979

CID 1998

CID 1999

CID 2000

CID 2001

CID 2230

CID 2663

CID 2675

CID 2816

CID 2938

CID 2939

CID 3068

CID 3075

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1413

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.20 | 36.3.11.10 | P282L15-18 is language specific to EHT MU with a single RU/MRU, then a sentence general to EHT MU is provided at P282L20 but the para continues at P282L21-26 with language specific to EHT MU with > 1 RU/MRU. This orderig is unnatural. | Move sentence at P282L20 to its own para above P282L15 | ACCEPTED Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4 No further changes are needed. |

Draft0.4 FYI

## CID 1568

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282 | 36.3.11.10 | the extra LTF symbol has been defined for OFDMA and non-OFDAMA transmission. to indicate using of extra LTF, it should be indciated by using the U-SIG or EHT-SIG | define the extra LTF indication in U-SIG or EHT-SIG field | REVISED Agreed in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. |

Draft0.4 FYI

## CID 1569

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.16 | 36.3.11.10 | the extra LTF symbol has been defined for OFDMA and non-OFDAMA transmission. and if the extra LTF is used, the number of LTF symbol is not a function of the total number of Nss | correct the sentence for the number of LTF symbol by referring to the MU-PPDU case | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 4.0. No further changes are needed. |

## CID 1570

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.31 | 36.3.11.10 | the extra LTF symbol has been defined for OFDMA and non-OFDAMA transmission. and if the extra LTF is used, the number of LTF symbol is not a function of the total number of Nss | correct the sentence for the number of LTF symbol by referring to the MU-PPDU case | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 4.0. No further changes are needed. |

## CID 1584

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 285.65 | 36.3.11.10 | Clarify EHT-LTF coefficients for the tones corresponding to punctured channels for non-OFDMA or for the tones which are not assigned to STAs for OFDMA. | See the comment. | REVISED Agree in principle. Note to the editor: This Please add the following text to Page285 L65 “For a non-OFDMA transmission when preamble puncturing is applied, a single large size MRU spans the non-punctured portions of the PPDU bandwidth. In the puncturing case, the values of the EHT-LTF sequence (defined in Equation (27-41) to Equation (27-52) and Equation (36-35) to Equation (36-37)) are replaced by zero for subcarriers that fall outside the aforementioned single large size MRU. This is also applicable to an EHT Sounding NDP with preamble puncturing. The mapping of the non-OFDMA puncturing pattern signaled in the U-SIG to the corresponding large size MRU is defined in Table 36-21. For an OFDMA transmission, the values of EHT-LTF sequence (defined in Equation (27-41) to Equation (27-52) and Equation (36-43) to Equation (36-45)) are replaced by zero, for subcarriers that belong to unassigned RUs as well as for DC tones or null sub-carriers. ” For OFDMA, This problem is already partialy revised in the PDT 11/21-114r4 for Draft 4.0, only the underlined need to be updated. |

Draft0.4 FYI

## CID 1630

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.12 | 36.3.11.10 | MAP is a Rel2 topic. Delete the sentence "This is applicable to multi-AP transmission modes as well". | See the comment. | ACCEPTED |

## CID 1979

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.11 | 36.3.11.10 | Multi-AP transmission is under the scope of Rel.2 features. | Suggest to delete the sentence "This is applicable to multi-AP transmission modes as well." at this stage of draft. (We can still tract the concept passed in SFD and bring it back in the proper stage of the draft for Rel.2) | ACCEPTED Note to the Editor: The resolution is the same as that of the CID1630. |

## CID 1998

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 283.53 | 36.3.11.10 | EHTLTF_{-2036:2036} is better to be modified as EHTLTF_{-2036, 2036} as like EHTLTF_{-1012, 1012} in 160 MHz transmission, since {-2036:2036} is not specifying what's the granularity of used LTF tones within given range(e.g. see the notation of EHTS_{-2032:16:2032} in 36.3.11.9). Otherwise, it may be better to express with the granularity(e.g. 4) according to 1x LTF tones. | As in comment. | REVISED Instructions to the editor Please replace EHTLTF-2036:2036 with EHTLTF-2036, 2036 in Page 283 L53 e.q.36-34, Page 284 L8 e.q.36-35 and Page 285 L5 e.q.36-36 |

## CID 1999

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 284.08 | 36.3.11.10 | EHTLTF_{-2036:2036} is better to be modified as EHTLTF_{-2036, 2036}, otherwise, it may be better to express by the granularity(e.g. 2) within the given range according to 2x LTF tones. | As in comment. | ACCEPTED Note to the Editor: The resolution is the same as that of the CID1998. |

## CID 2000

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 285.05 | 36.3.11.10 | EHTLTF_{-2036:2036} is better to be modified as EHTLTF_{-2036, 2036}, otherwise, it may be better to express by the granularity(e.g. 1) within the given range according to 4x LTF tones. | As in comment. | ACCEPTED Note to the Editor: The resolution is the same as that of the CID1998. |

## CID 2001

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 285.65 | 36.3.11.10 | The time domain representation of the signal for EHT MU PPDU and EHT TB PPDU are missing. | Add the description for time domain representation of the signal of EHT LTF for both EHT MU PPDU and EHT TB PPDU. | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. |

Draft 4.0 FYI

## CID 2230

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.11 | 36.3.11.10 | R2 feature is mentioned in R1 draft | Remove this sentence "This is applicable to multi-AP transmission modes as well." | ACCEPTED Note to the Editor: The resolution is the same as that of the CID1630 |

## CID 2663

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 285.65 | 36.3.11.10 | Missing description | Matematical description of EHT-LTF is missing, 11ax not only included that, it also included block diagrams on how to generate 1x and 2x LTF. Same diagrams can be referenced here, but mathematical description specifying various parameters that are present in EHT-LTF generation is a must | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. |

## CID 2675

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 282.28 | 36.3.11.10 | This paragraph conflicts with passed motions namely motions 137 and 141: | "In an EHT TB PPDU, is indicated in the Trigger frame that triggers the transmission of the PPDU. In a non-OFDMA EHT TB PPDU, the number of EHT-LTF symbols, , is a function of the total number of spatial streams, , as shown in Table TBD (EHT-LTF type and GI duration combinations for various EHT PPDU formats)." Motion 137: The number of EHT-LTF in non-OFDMA transmissions is larger than or equal to the initial number of EHT-LTFs determined by the number of total spatial streams. · The maximum number of EHT-LTFs supported by the receiving STA(s) can be claimed by capability. · The number of EHT-LTFs is signaled separately from Nss or Nss_total. · This feature is optional for receiver. · The allowed number of extra LTFs is TBD. · The support for NDP is TBD. [Motion 137, #SP258, [3] and [52]] Motion 141: The allowed values of maximum NLTF receive capability for multiple-user transmission are 4, 8, and 16. · NOTE 1 - This capability is for both OFDMA and non-OFDMA MU-MIMO transmission. · NOTE 2 - The value of maximum NLTF = 16 is available in R2. [Motion 141, #SP261, [3] and [52]] | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. |

## CID 2816

| Page. Line |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 285.65 |  | 36.3.11.10 | There is no mathematical expression for the time-domain representation of EHT-LTF | Add | REVISED Same as the CID2663 Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. The proposed resolution is also the same as that of CID 2663. |

## CID 2938

| Page. Line |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 282.20 |  | 36.3.11.10 | "In an EHT MU PPDU, N_EHT-LTF is indicated in the U-SIG field. In an EHT MU PPDU with more than one RU/MRU, N_EHT-LTF may take a value that is greater than or equal to the maximum value of the initial number of EHT-LTF symbols for each RU/MRU" The use of more LTFs is beneficial for channel estimation in all PPDUs regardless how may RU/MRUs are present. The case of one RU/MRU should therefore not be excluded. | remove "with more than one RU/MRU" and change related text in line number 15 to 18 | REJECTED The non-OFDMA transmission has more constrains on the number of extra LTF based on Motion 137. so the non-OFDMA transmission and OFDMA transmission are described separately. Please refer to the latest PDT 11/21-114r4. |

Discussion:

## CID 2939

| Page. Line |  |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 282.29 |  |  | 36.3.11.10 | "In an EHT TB PPDU, N_EHT-LTF is indicated in the Trigger frame that triggers the transmission of the PPDU. In a non-OFDMA EHT TB PPDU, the number of EHT-LTF symbols, N_EHT-LTF, is a function of the total number of spatial streams, N_ss, as shown in Table TBD (EHT-LTF type and GI duration combinations for various EHT PPDU formats). For an OFDMA EHT TB PPDU, N_EHT-LTF may be greater than or equal to the maximum value of the initial number of EHT-LTF symbols for each RU/MRU r, which is calculated as a function of N_ss,r,total, separately based on Table TBD"" With EHT MU PPDU, the use of more LTFs than the ones defined here is allowed. To harmonize and to reduce options, more LTFs than number of spatial streams should be considered for non-OFDMA EHT TB PPDU too." | remove references to non-OFDMA and OFDMA cases. That is to change "non-OFDMA EHT TB PPDU" to "EHT TB PPDU", and "OFDMA EHT TB PPDU" to "EHT TB PPDU" | REVISED The non-OFDMA transmission has more constrains on the number of extra LTF based on Motion 137. So they non-OFDMA transmission and OFDMA transmission are described separately. Please refer to the latest PDT 11/21-114r4. Instructions to the editor Please add the following sentence to Page 282, L33 If additional EHT-LTFs is supported which is indicated by extra LTFs for non-OFDMA PPDU capability fields, NEHT-LTF may be greater than or equal to the initial number of EHT-LTF symbols, with the limit that NEHT-LTF shall be no more than twice the initial number of EHT-LTFs and chosen from the set [2 4 8]. |

Disscussion:

## CID 3068

| Page. Line |  |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 281.56 |  |  | 36.3.11.10 | add a table to list the M/O for EHT-LTF similar as in HE-LTF | as commented | REVISED Agree in principle. Note to the editor: This problem is already revised in the PDT 11/21-114r4 for Draft 0.4. No further changes are needed. |

Dsicussion:

## CID 3075

| Page. Line |  |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 281.56 |  |  | 36.3.11.10 | missed NDP feedback tone set definition that already exsit in 11ax | add it back | REJECTED No EHT TB feedback NDP is defined yet. |