IEEE P802.11
Wireless LANs

| EHT scrambler CR on P802.11be D0.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chenchen Liu | Huawei Technologies | Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | liuchenchen1@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

14 comments under 36.3.12.2:

CID 1571

CID 1572

CID 1588

CID 3070

CID 2666

CID 3407

CID 2659

CID 1971

CID 2026

CID 2412

CID 2664

CID 2665

CID 3069

CID 3071

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1571

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.24 | 36.3.12.2 | following text " The octets of the PSDU are placed in the transmit serial bit stream, bit 0 first and bit 7 last" is not needed. | delete the this sentence. | REJECTED It is necessary for applying the scrambler properly |

Discussion:

## CID 1572

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.15 | 36.3.12.2 | The 11bit of service field is used for scramble seed in EHT PPDU. Why the 7bit of service field is considered when MU-RTS is transmitted by using the EHT-PPDU? Clarify it. | as in comment. | REVISED This is based on the Motion 124 #SP 124(802.11be supports using the first 11 LSB bits in SERVICE field for 802.11be scrambling seed initialization and keeping the remaining 5 bits as reserved), to guarantee all the CTS from different STAs are identical. Instructions to the editor: Please add the sentence” The SERVICE field of a CTS frame in response to an MU-RTS after scrambling shall have the identical first 7 bits to the SERVICE field of corresponding MU-RTS frame ” to the end of P288 L18 |

Discussion:

## CID 1588

| Page. Line |  | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 287.01 |  | 36.3.12 | Segment deparser block is defined in the transmitter block diagram. A new subsection which adresses this block is also needed in the Data field section. | Add a new subsection for segment deparser into the Data field section and describe its detail. | REVISED Not related to the scrambler subclause. Instructions to the editor: Consider reassigning this CID to, someone else. |

Discussion:

## CID 3070

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.42 | 36.3.12.2 | The text in the figure are not readable | suggest editor to re-import the figure | REVISED Instructions to the editor Please replace figure 36-51 of Page 287 with the following figure |

## CID 2666

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.34 | 36.3.12.2 | Texts in figure are not legible | Texts in figure 36-51 are barely legible | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 3407

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.00 | 36.3.12.1 | in Figure 36-51, letters are very blury. | regenerate the figure | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 2659

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.00 | 36.3.12.2 | Figure 36-51 quality is very low. Text on the figure is hard to read. | Replace with a better quality diagram that shows the texts on the figure. | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 1971

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.21 | 36.3.12.2 | Clear TBDs in 36.3.12.2 EHT PHY DATA scrambler and descrambler | As comment. There are quite a few TBDs and red colored paragarphs. | ACCEPTED Note to the Editor: The TBDs and red colored paragarphs in 36.3.12.2 are unnessary. |

Discussion:

## CID 2026

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.46 | 36.3.12.2 | We already decided to use 11bit Scrambler Initialization in Service field. So the Data scrambler is fine. | as in comment. Delete TBDs in Figure 36-51 and the following texts | ACCEPTED Note to the Editor: The proposed resolution is the same as that of CID1971. |

## CID 2412

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.23 | 36.3.12.2 | The sentence of "The DATA field, composed of SERVICE, PSDU, tail (if BCC is used), and pad parts, shall be scrambled with a length-2047 PPDU-synchronous scrambler" can make it possible to misunderstand that tail bits are scrambled. | Modify the sentence as follows: "The SERVICE field, PSDU, and pre-FEC PHY padding of the Data field shall be scrambled by the length-2047 PPDU-synchronous scrambler." | REJECTED The tail bits shall be scrambled and set to zeros after scrambling. |

Discussion:

## CID 2664

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.23 | 36.3.12.2 | Incorrect description | Scrambler is mandated to scramble pre-FEC padding and post-FEC padding sequence is designer's choice, so "and pad parts" is incorrect and should be replaced with pre-FEC padding (pad parts is copied from original description for 11a/g and not 11ax) | REVISED Agree with the commenter on the proposed change but with slight change in wordings. Instructions to the editor Please replace “pad parts” in Page 287 L23 with “pre-FEC pad parts” |

## CID 2665

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.12 | 36.3.12.2 | Incomplete description | Recommended to change paragraph as following similar to original description of 7-bit scrambler: "The same scrambler is used to scramble transmit data and to descramble receive data. When transmitting, the first 11 bits of scrambling sequence, which is also used to set the initial state of the scrambler, shall be set to a pseudorandom nonzero state. During reception by an EHT STA, the initial state can be estimated from the 11 LSB of the service field (TBD)." | REJECTED The initial state is not determined by the first 11 bits of scrambling sequence |

## CID 3069

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.15 | 36.3.12.2 | change "pad parts" to pre-FEC padding | as commented | REVISED “pad parts” in Page 287 L23 replaced with “pre-FEC pad parts” Note to the Editor: The proposed resolution is the same as that of CID 2664. |

## CID 3071

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.12 | 36.3.12.2 | The text here doesn't match figure 36-51. with the current figure the initial state of the scrambler, which is the value in the shift register, can be arbitrary value. | Remove "When transmitting, the initial state of the scrambler shall be set to a pseudorandom nonzero state" | REJECTED The initial state of the scrambler shall be set to a pseudorandom nonzero state, which is implemented by inputing the pseudorandom initialization bits |