IEEE P802.11
Wireless LANs

| EHT scrambler CR on P802.11be D0.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chenchen Liu | Huawei Technologies | Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | liuchenchen1@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

13 comments under 36.3.12.2:

CID 1571

CID 1572

CID 3070

CID 2666

CID 3407

CID 2659

CID 1971

CID 2026

CID 2412

CID 2664

CID 3069

CID 2665

CID 3071

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1571

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.24 | 36.3.12.2 | following text " The octets of the PSDU are placed in the transmit serial bit stream, bit 0 first and bit 7 last" is not needed. | delete the this sentence. | REJECTED It is necessary for applying the scrambler properly |

Discussion:

## CID 1572

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.15 | 36.3.12.2 | The 11bit of service field is used for scramble seed in EHT PPDU. Why the 7bit of service field is considered when MU-RTS is transmitted by using the EHT-PPDU? Clarify it. | as in comment. | REJECT This is based on the Motion 124 #SP 124(802.11be supports using the first 11 LSB bits in SERVICE field for 802.11be scrambling seed initialization and keeping the remaining 5 bits as reserved), to guarantee all the CTS from different STAs are identical. The CTS is transmitted in non-HT format with only seven scrambler initialization bits |

Discussion:

## CID 3070

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.42 | 36.3.12.2 | The text in the figure are not readable | suggest editor to re-import the figure | REVISED Instructions to the editor: Please replace figure 36-51 of Page 287 with the figure below CID 3070 in CND 11-21/416r3 and remove the TBD in the caption for the figure |

## CID 2666

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.34 | 36.3.12.2 | Texts in figure are not legible | Texts in figure 36-51 are barely legible | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 3407

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.00 | 36.3.12.1 | in Figure 36-51, letters are very blury. | regenerate the figure | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 2659

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.00 | 36.3.12.2 | Figure 36-51 quality is very low. Text on the figure is hard to read. | Replace with a better quality diagram that shows the texts on the figure. | REVISED Note to the Editor: The resolution is the same as that of the CID 3070 |

## CID 1971

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.21 | 36.3.12.2 | Clear TBDs in 36.3.12.2 EHT PHY DATA scrambler and descrambler | As comment. There are quite a few TBDs and red colored paragarphs. | REVISED Note to the Editor: Please replace P287 Line 49 to P288 Line 8 with the following text(no TBD, no red colored lines): “NOTE—The 2047-bit sequence generated repeatedly by the scrambler is (leftmost used first) 1111111111100000000011000000011110000011001100011111111011000000101110000100101100101100111100111110011110001111001101100111110111110001010001101000101110010100101110001100101101111100110100011111001011000111001110110111101011010010001100110101111111000100000110101000111000010110110010011011110111101001010010011000110111110111010001010100101000001100010001111010101100100000111101000110010010111110110010001011110101001001000011011010011101100111010111110100010001001010101011000000001110000001101100001110111001101010111110000010001100010101111010000100100100101101101100110110111111011010000101100100100111101101110010110101110011000101111110100100001001101001011110011001001111111011100000101011000100001110101001101000011110010011001110111111101010000010000100010100101010001100000101111000100100110101101111000110100110111001111010111100100010011101010111010000010100100010001101010101110000000101100000100111000101110110100101011001100001111111001100000111111000110000110111100111010011110100111001001110111011101010101010000000000100000000101000000100010000101010100100000001101000001110010001101110101110101000101000010100010010001010110101000011000010011110010111001110010111101110010010101110110000101011100100001011101001001010011011000111101110110010101011110000001001100001011111001001000111011010110101100011000111011110110101001011000011001110011111101111000010100110010001111110101100001000111001010110111000011010110011100011111011011000101101110100110101001111000011100110011011111111101000000010010000010110100010011001010111111000010000110010100111110001110001101101101110110110101011011000001101110001110101101101000110110010111011110010101001110000011101100011010111011100010101011010000001100100001111101001100010011111010111000100010110101010011000000111110000110001100111101111110010100001110001001101101011110110001001011101011001010001111000101100110100111111001110000111101100110010111111110010000001110100001101001001110011011101111101010100010000001010100001000001001010001011000101001110100011101001011010011001100 when the TXVECTOR parameter SCRAMBLER_INITIAL_VALUE is 2047 (all 1s). ” |

Discussion:

## CID 2026

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.46 | 36.3.12.2 | We already decided to use 11bit Scrambler Initialization in Service field. So the Data scrambler is fine. | as in comment. Delete TBDs in Figure 36-51 and the following texts | REVISED Note to the Editor: The proposed resolution is the same as that of CID1971. |

## CID 2412

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.23 | 36.3.12.2 | The sentence of "The DATA field, composed of SERVICE, PSDU, tail (if BCC is used), and pad parts, shall be scrambled with a length-2047 PPDU-synchronous scrambler" can make it possible to misunderstand that tail bits are scrambled. | Modify the sentence as follows: "The SERVICE field, PSDU, and pre-FEC PHY padding of the Data field shall be scrambled by the length-2047 PPDU-synchronous scrambler." | REJECTED The tail bits shall be scrambled and set to zeros after scrambling. |

Discussion:

## CID 2664

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.23 | 36.3.12.2 | Incorrect description | Scrambler is mandated to scramble pre-FEC padding and post-FEC padding sequence is designer's choice, so "and pad parts" is incorrect and should be replaced with pre-FEC padding (pad parts is copied from original description for 11a/g and not 11ax) | REVISED Agree with the commenter on the proposed change but with slight change in wordings. Instructions to the editor Please replace “pad parts” in Page 287 L23 with “pre-FEC pad parts” |

## CID 3069

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.15 | 36.3.12.2 | change "pad parts" to pre-FEC padding | as commented | REVISED “pad parts” in Page 287 L23 replaced with “pre-FEC pad parts” Note to the Editor: The proposed resolution is the same as that of CID 2664. |

## CID 2665

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.12 | 36.3.12.2 | Incomplete description | Recommended to change paragraph as following similar to original description of 7-bit scrambler: "The same scrambler is used to scramble transmit data and to descramble receive data. When transmitting, the first 11 bits of scrambling sequence, which is also used to set the state of the scrambler for the subsequent scrambling bits, shall be set to a pseudorandom nonzero state. During reception by an EHT STA, the initial state can be estimated from the 11 LSB of the service field (TBD)." | REVISED Note to the Editor: Please replace the paragraph from P288 Line10 to P288 Line 13 with the following text: "The same scrambler is used to scramble transmit data and to descramble receive data. When transmitting, the first 11 bits of scrambling sequence, which is also used to set the state of the scrambler for the subsequent scrambling bits, shall be set to a pseudorandom nonzero state. During reception by an EHT STA, the initial state can be estimated from the 11 LSBs of the scrambled service field." |

## CID 3071

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 288.12 | 36.3.12.2 | The text here doesn't match figure 36-51. with the current figure the initial state of the scrambler, which is the value in the shift register, can be arbitrary value. | Remove "When transmitting, the initial state of the scrambler shall be set to a pseudorandom nonzero state" | REVISED Note to the Editor: It has been solved in CID 2665. No further changes are needed. |