IEEE P802.11
Wireless LANs

| CR for Clause 36.3.2.3 Subcarriers and resource allocation for multiple RUs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Myeongjin Kim | Samsung |  |  | mj1108.kim@samsung.com |
| WookBong Lee | Samsung |  |  | wookbong.lee@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC34 on P802.11be D0.3

R0: The following 40 comments under “36.3.2.3 Subcarriers and resource allocation for multiple RUs” are resolved:

CID 1245, 1247, 1249, 1250, 1290, 1293, 1294, 1295, 1299, 1609, 1962, 1988, 1989, 1990, 1991, 2393, 2394, 2395, 2396, 2397, 2398, 2785, 2786, 2787, 2927, 2928, 2929, 2930, 2931, 3041, 3098, 3154, 3166, 3269, 3270, 3271, 3272, 3273, 3274, 3275

R1: The additional following 10 comments under “36.3.2.3 Subcarriers and resource allocation for multiple RUs” are resolved:

CID 1296, 1297, 1298, 2693, 2695, 2696, 2697, 2784, 2946, 3079

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Additional CIDs are resolved. |

## CIDs: 1245, 1247, 1249, 1250, 1290, 1293, 1294, 1295, 1299, 1609, 1962, 1988, 1989, 1990, 1991, 2393, 2394, 2395, 2396, 2397, 2398, 2785, 2786, 2787, 2927, 2928, 2929, 2930, 2931, 3041, 3098, 3154, 3166, 3269, 3270, 3271, 3272, 3273, 3274, 3275, 1296, 1297, 1298, 2693, 2695, 2696, 2697, 2784, 2946, 3079

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1245 | 184.59 | 36.3.2.3.1 | Should remove the reference to the HE RU tone indices definition in Clause 27 and refer to the EHT RU definition directly | as in comment | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1247 | 187.40 | 36.3.2.3.2 | Delete the paragraph of "Not every possible combination of small size MRUs and RUs is allowed in a DL OFDMA transmission. The allowed combinations of small size MRUs and RUs in a DL OFDMA transmission are restricted according to Table 36-26 (RU Allocation subfield)." since 1) allowed small MRU combinations have already discussed in the previous paragraphs. 2). Small MRU combination applies to both DL and UL OFDMA. Reference to the EHT-SIG RU allocation is not good. | as in comment | Accepted Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 1249 | 194.30 | 36.3.2.3.3 | "It is mandatory for non-AP STA to support the transmission and reception"'. Should also clarify the mandatory requirement for AP | as in comment | Revised. Agree in principle. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1250 | 194.34 | 36.3.2.3.3 | "unless the MRU size is larger than its supported bandwidth" should be changed "unless MRU is outside of the STA support BW" since a 484+242 MRU in seconday 80MHz has size less than an 80MHz STA's supportiing BW but can not be supported by this STA | as in comment | Revised. Agree in principle. But, it is better to write “unless MRU is outside of the STA operating channel width” rather than “unless MRU is outside of the STA support BW”. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1290 | 184.56 | 36.3.2.3.1 | Awkward English "An EHT STA shall be allowed to be assigned with more than one Resource Unit (RU). " | Try "An EHT STA shall support the reception of one RU or one Multiple Resource Unit (MRU) in a PPDU." | Revised. Agree in principle. But, it is better to write “An EHT STA shall support the transmission and reception of multiple resource unit (MRU) in a PPDU.” because this clause is about a description of MRU. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1293 | 185.14 | 36.3.2.3.2 | OFDMA is not a format | Change "DL and UL transmissions in OFDMA format" to "DL and UL OFDMA transmissions" | Accepted. |
| 1294 | 185.18 | 36.3.2.3.2 | "certain ... certain" is not clear since it doesn't express the notion of adjacency | Change the second "a certain" to "an adjacent". Ditto P185L25 | Accepted. |
| 1295 | 185.37 | 36.3.2.3.2 | It is hard for figures to be normative. | As well as these figures at P185L35-P187L39, identify the allowed small-RU MRUs via table(s) | Revised. Tables to identify the allowed small size MRUs are already included in 802.11be D0.4. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 1299 | 191.28 | 36.3.2.3.3 | "AP and STA" is WFA terminology not IEEE terminology | Change to "a STA" | Accepted. |
| 1609 | 185.46 | 36.3.2.3.2 | In Figure 36-5, MRU blocks are not consistent, two of them are using two colors and the other uses only one color. Figure 36-6 to Figure 36-9 also have the same issue. If there is no specific reason, make them consistent. | Make MRU blocks in figures consistent. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 1962 | 187.45 | 36.3.2.3.2 | "It is mandatory for a non-AP STA to support the transmission and reception of 26+52 tone and 26+106 tone MRUs in OFDMA." What about AP? The support of small-size MRUs should be mandatory at the AP too. | Add the mandaory support of small size MRUs for AP. At least add the mandatory receive of small size MRU at the AP. | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1988 | 185.14 | 36.3.2.3.2 | Suggest to modify DL and UL transmissions in OFDMA format as DL and UL OFDMA transmissions which is a generally used term in the spec. | As in comment. | Revised. Already revised by CID 1293. Note to the editor: the proposed resolutions for CID 1293, 1988 in 11/21-0417r1 are the same. |
| 1989 | 185.46 | 36.3.2.3.2 | Depicted blocks for MRU(some are expressed as one block, others not) needs to be consistent in Figure 36-5~9. | Update the Figure 36-5~9(MRUs). | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 1990 | 187.51 | 36.3.2.3.3 | Suggest to modify DL and UL transmissions in non-OFDMA format as DL and UL non-OFDMA transmissions which is a generally used term in the spec. | As in comment. | Accepted. |
| 1991 | 191.33 | 36.3.2.3.3 | Suggest to modigy DL and UL in OFDMA format as DL and UL OFDMA transmission(format is generally used with PPDU in the spec). | As in comment. | Revised. Modified the wording of “DL and UL in OFDMA format” as “DL and UL OFDMA transmissions”. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 2393 | 191.43 | 36.3.2.3 | Figures for MRUs in non-OFDMA and OFDMA can be merged. | "Modify the caption of Figure 36-11 (""Figure 36-11--Allowed 484+242 tone MRUs in non-OFDMA 80 MHz EHT PPDU"") as follows: Figure 36-11--Allowed 484+242 tone MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU | Rejected. Separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 has been made by CID2787. Therefoere, for the better understing, it is better to keep these figures separately. |
| 2394 | 192.06 | 36.3.2.3 | Figures for MRUs in non-OFDMA and OFDMA can be merged. | "Modify the caption of Figure 36-12 (""Figure 36-12--Allowed 996+484 tone MRUs in non-OFDMA 160 MHz EHT PPDU"") as follows: Figure 36-12--Allowed 996+484 tone MRUs in an 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU | Rejected. Separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 has been made by CID2787. Therefoere, for the better understing, it is better to keep these figures separately. |
| 2395 | 192.35 | 36.3.2.3 | Figures for MRUs in non-OFDMA and OFDMA can be merged. | "Modify the caption of Figure 36-14 (""Figure 36-14--Allowed 2x996+484 tone MRUs in non-OFDMA 320 MHz EHT PPDU"") as follows: Figure 36-14--Allowed 2x996+484 tone MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU | Rejected. Separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 has been made by CID2787. Therefoere, for the better understing, it is better to keep these figures separately. |
| 2396 | 193.43 | 36.3.2.3 | Figures for MRUs in non-OFDMA and OFDMA can be merged. | "Modify the caption of Figure 36-15 (""Allowed 3x996 tone MRUs in non-OFDMA 320 MHz EHT PPDU"") as follows: Figure 36-15--Allowed 3x996 tone MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU | Rejected. Separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 has been made by CID2787. Therefoere, for the better understing, it is better to keep these figures separately. |
| 2397 | 194.03 | 36.3.2.3 | Figures for MRUs in non-OFDMA and OFDMA can be merged. | "Modify the caption of Figure 36-16 (""Allowed 3x996+484 tone MRUs in non-OFDMA 320 MHz EHT PPDU "") as follows: Figure 36-16-- Allowed 3x996+484 tone MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU | Rejected. Separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 has been made by CID2787. Therefoere, for the better understing, it is better to keep these figures separately. |
| 2398 | 0.00 | 36.3.2.3 | Please refer the document of 11-21/0104r3. In Table 36-6, Table 36-7, and Table 36-8, each MRU combination can described by using RU indices. | For example, change "[empty-RU242 RU242 RU484]" in Table 36-X6 to "484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484]" The rest can be also changed in this way. I can submit a PDT document for this. | Revised. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 2785 | 185.32 | 36.3.2.3.2 | The allowed 26+52 tone RU tend to create "orphan" 26-tone RUs on the left and right. An allocation similar to Figure 36-8 would make more sense. | Define 26+52 tone RUs containing the center 26-tone RU. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2786 | 185.32 | 36.3.2.3.2 | The allowed 26+52 tone RU tend to create "orphan" 26-tone RUs on the left and right. An allocation similar to Figure 36-8 would make more sense. | Define 26+52 tone RUs containing the center 26-tone RU. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2787 | 187.49 | 36.3.2.3.3 | Having separate subsections for OFDMA and non-OFDMA inside "36.3.2.3.3 Large size multiple RUs" would be clearer. | Make separate subsections for OFDMA and non-OFDMA under 36.3.2.3.3 | Revised. Agree with the commenter. Having separate subsections for OFDMA and non-OFDMA looks better. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 2927 | 185.52 | 36.3.2.3.2 | In the Figure: The 26 tone RU which is part of right 26+52 tone MRU is not highlighted | Please highlight similar to the left 26+52 tone MRU. | Revised. Note to the editor: the comment has been resolved in 802.11be and no further changes are needed. |
| 2928 | 186.20 | 36.3.2.3.2 | In the Figure: The 26 tone RU which is part of two 26+52 tone MRUs are not highlighted | Please highlight. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2929 | 186.39 | 36.3.2.3.2 | In the Figure: The 26 tone RU which is part of four 26+52 tone MRUs are not highlighted | Please highlight. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2930 | 187.20 | 36.3.2.3.2 | In the Figure: The 26 tone RU which is part of two 26+106 tone MRUs are not highlighted | Please highlight. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2931 | 187.37 | 36.3.2.3.2 | In the Figure: None of the 26 tone RU which is part of four 26+106 tone MRUs are not highlighted | Please highlight. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 3041 | 185.52 | 36.3.2.3 | The color of the MRU in figure 36-5 to 36-10 should be consistent. | as commented | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 3098 | 184.59 | 36.3.2.3.1 | "The tones indices of the various RUs have been updated in relation to RUs defined for HE STAs (...) and are defined in 36.3.2 (Subcarrier and resource allocation)". It is not accurate to say that RUs in clause 27 are defined for HE STAs, since it is also defined for EHT STAs. Change to RUs defined in 27.3.2 (see Table ... ) and RUs definded in 36.3.2 | As in comment | Revised. Already revised by CID 1245. Note to the editor: the proposed resolutions for CID 1245, 3098 in 11/21-0417r1 are the same. |
| 3154 | 194.29 | 36.3.2.3 | This comment is on text in 11-21/0104r3. While the Table 36-X6 ~ Table 36-X8 are clearer in defining the large MRUs than the Figures in the subclause, the notation "[empty-RU242 RU242 RU484]" is still not precise enough. Note that small MRU had a more precise definition making use of the RU "numbers" - e.g. in Table 36-X1, 56+26 tones MRU 1 is defined as the union of 52-tone RU 2 and 26-tone RU 2. Large MRU should follow similar definition. | In Table 36-X6 ~ 36-X8 in 11-21/0104r3, change the MRU definition to make use of RU "numbers". For example, in Table 36-X6, the 484+242 tone MRU 1 should be defined as "242 tone RU 2 + 484 tone RU 2". | Revised. Already revised by CID 2398. Note to the editor: the proposed resolutions for CID 2398, 3154 in 11/21-0417r1 are the same. |
| 3166 | 185.01 | 36.3.2.3.1 | Given that we have explicitly defined what M-RUs are possible, there is no need to spell out the rules on small and large RUs not being allowed to be used together in an MRU. | Delete P185L1-10. | Rejected. For the better understing, it is better to keep these sentences. |
| 3169 | 184.58 | 36.3.2.3.1 | RU26 to RU4x996 should be 26-tone RU to 4x996-tone RU to be consistent through the spec | as in comment | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 3270 | 185.12 | 36.3.2.3.2 | add indices for allowed small size MRU for each figure. Those indices need to be refered in 36.3.2.5 | as in comment | Accepted. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 3271 | 185.12 | 36.3.2.3.2 | all 26+52 tone MRU and 26+106 tone MRU in this subclause should be 52+26-tone MRU and 106+26-tone MRU to be consistent throught the spec | as in comment | Accepted Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 3272 | 185.32 | 36.3.2.3.2 | "indicate" is usually used for signaling. Better to use "shown" instead of "indicated". Apply this modification for all figures in this subclause. | as in comment | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 3273 | 187.40 | 36.3.2.3.2 | "Not every possible combination of small size MRUs and RUs is allowed in a DL OFDMA transmission." seems redundant since it has alreadyexplained a first certain RU combined with a second certain RU with figures. | delete "Not every possible combination of small size MRUs and RUs is allowed in a DL OFDMA transmission." | Accepted Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 3274 | 187.49 | 36.3.2.3.3 | 484+242 tone MRU to 3x996+484 tone MRU should be 484+242-tone MRU to 3x996+484-tone MRU to be consistent through the spec. Don't forget dash (-) between RU size and tone | as in comment | Accepted Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 3275 | 194.33 | 36.3.2.3.3 | In "2×996+484 tone MRU, 3×996-tone MRU, and 3×996+484 tone MRU in 80/160/320 MHz PPDU in OFDMA", it should be 320 MHz PPDU in OFDMA. | delete 80/160/ from the sentence. | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1296 | 187.56 | 36.3.2.3.3 | Missing articles | Change to "in a non-OFDMA ...". Also P187L57.5 "any one of the four"; P188L1 "in the non-OFDMA 80MHz ...". And repeat these three fixes at P188L21-28 and again within P188L47-P189L2; ditto P189L34-P190L2, P190L36-44, etc etc. Also add an article to the figure captions in this section. | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1297 | 191.27 | 36.3.2.3.3 | Missing articles | "an AP and a STA"; "a 484+242 tone MRU"; "an 80 MHz PPDU" ... actually, review the whole section and add articles as required. | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 1298 | 191.31 | 36.3.2.3.3 | Unclear antecendent for "its" | "larger than the STA's supported bandwidth". Ditto fix the "its" at P194L34 | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 2693 | 184.58 | 36.3.2.3.1 | The following sentence is confusing: The tones indices of the various RUs have been updated in relation to RUs defined for HE STAs (see...) and are defined in 36.3.2 (Subcarrier and resource allocation) | Change it by list a set of Tables related to the tone indices for different bandwidth. For example, "The tones indices of the various RUs are defined in Table 27-7 (Data and pilot subcarrier indices for RUs in a 20 MHz HE PPDU and in a non-OFDMA 20 MHz HE PPDU), Table 27-8 (Data and pilot subcarrier indices for RUs in a 40 MHz HE PPDU and in a non-OFDMA 40 MHz HE PPDU)), Table ..." | Rejected. It looks better to modify the sentence as follows based on CID 1245 and 3098: “The tone indices of the various RUs for different EHT PPDU bandwidths are defined in 36.3.2.1 (Subcarriers and resource allocation for wideband).” |
| 2695 | 185.19 | 36.3.2.3.2 | Combine the following sentences into one: The data subcarriers of a 26+52 tone MRU consist of the data subcarriers of the 26-tone and 52-tone RUs that make up the 26+52 tone MRU. The pilot subcarriers of a 26+52 tone MRU consist of the pilot subcarriers of the 26-tone and 52-tone RUs that make up the 26+52 tone MRU. | Change the sentence to "The data and pilot subcarriers of a 26+52 tone MRU consist of the data and pilot subcarriers of the 26-tone and 52-tone RUs, respectively, that make up the 26+52 tone MRU. " The similar change can be made in L26 of this page. | Rejected. Combing the sentences doesn’t have any impact on the spec. Keeping the sentences separate looks better for consistency through this section. |
| 2696 | 185.46 | 36.3.2.3.2 | The right-most 26+52 tone MRU looks include the null tones. Shouldn't it be similar to the left-most 26+52 MRU? | See comment. Similar comment applies to other figures in this subclause. | Revised. Note to the editor: the comment has been resolved in 802.11be D0.4 and no further changes are needed. |
| 2697 | 185.38 | 36.3.2.3.2 | The MRU indices are shown in Figure 36-11 - Figure 36-21, but not in Figure 36-5 - Figure 36-10 | Add MRU indices on the right hand side of Figure 36-5 - Figure 36-10. And, by keeping those figures being consistent to Figure 36-11 - Figure 36-21, remove single RUs and null tones. | Revised. Already revised by CID 3270. Note to the editor: the proposed resolutions for CID 3270, 2697 in 11/21-0417r1 are the same. |
| 2784 | 185.14 | 36.3.2.3.2 | Change "are as follows: 26+52 tone MRU, and 26+106 tone MRU." to "are 26+52 tone MRU, and 26+106 tone MRU." | See comment | Rejected. Keeping the original sentence looks better for consistency through this section. |
| 2946 | 185.46 | 36.3.2.3.2 | The figure for MRUs are not consistent in figure 36-5, 36-6, 36-7 and 36-9. For example in figure 36-5, the first and second 26+52 tone MRU on the 3rd row looks different. | Make the MRUs consistent in the figures | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-0417r1 |
| 3079 | 184.57 | 36.3.2.3.1 | As described in Subclause 36.3.2, "The EHT tone plan and RU locations for a 20 MHz PPDU and 40 MHz PPDU are identical to that of HE PHY defined in Clause 27". The tone indices defined in Table 36-5,36-6,36-7 are for 80, 160, 320 MHz EHT PPDU. | The tones indices of the various RUs for different EHT PPDU bandwidths are defined in Table 27-7 for 20 MHz EHT PPDU, in Table 27-8 for 40 MHz EHT PPDU, in 3.6.2 for 80, 160, and 320 MHz EHT PPDU. | Rejected. It looks better to modify the sentence as follows based on CID 1245 and 3098: “The tone indices of the various RUs for different EHT PPDU bandwidths are defined in 36.3.2.1 (Subcarriers and resource allocation for wideband).” |

Proposed Changes:

## Instruction to the Editor: Modify texts in the subclause 36.3.2.3 as follows.

(The baseline for this text is 802.11be D0.4.)

36.3.2.3 Subcarriers and resource allocation for multiple RUs

36.3.2.3.1 General

An EHT STA shall support the transmission and reception of multiple resource unit (MRU) in a PPDU.(#1290) RUs in this context are RUs of 26-tone RU, 52-tone RU, 106-tone RU, 242-tone RU, 484-tone RU, 996-tone RU, 2×996-tone RU, and 4×996-tone RU. The tone indices of the various RUs for different EHT PPDU bandwidths are defined in 36.3.2.1 (Subcarriers and resource allocation for wideband).(#1245)

RUs with equal to or more than 242 tones are defined as large size RUs and RUs with less than 242 tones are defined as small size RUs.

Small size RUs can only be combined with small size RUs to form small size MRUs. The supported small size MRUs are defined in 36.3.2.3.2 (Small size multiple RUs).

Large size RUs can only be combined with large size RUs to form large size MRUs. The supported large size MRUs are defined in 36.3.2.3.3 (Large size multiple RUs).

36.3.2.3.2 Small size multiple RUs

The small size MRUs defined for DL and UL OFDMA transmissions are as follows: 52+26-tone MRU and 106+26-tone MRU. (#1293)

The 52+26-tone MRU is obtained by combining a certain 52-tone RU and an adjacent 26-tone RU that both fall within a 20 MHz channel boundary. The data subcarriers of a 52+26-tone MRU consist of the data subcarriers of the 52-tone and 26-tone RUs that make up the 52+26-tone MRU. The pilot subcarriers of a 52+26802.11be tone MRU consist of the pilot subcarriers of the 52-tone and 26-tone RUs that make up the 52+26-tone MRU. (#1294)

The 106+26-tone MRU is obtained by combining a certain 106-tone RU and an adjacent 26-tone RU that both fall within a 20 MHz channel boundary. The data subcarriers of a 106+26-tone MRU consist of the data subcarriers of the 106-tone and 26-tone RUs that make up the 106+26-tone MRU. The pilot subcarriers of a 106+26-tone MRU consist of the pilot subcarriers of the 106-tone and 26-tone RUs that make up the 106+26-tone MRU. (#1294)

The allowed 52+26-tone MRUs in a 20 MHz EHT PPDU are shown in Figure 36-5 (Allowed 52+26-tone MRUs in a 20 MHz EHT PPDU). (#3272)

Figure 36-5 – Allowed 52+26-tone MRUs in a 20 MHz EHT PPDU

The allowed 52+26-tone MRUs in a 40 MHz EHT PPDU are shown in Figure 36-6 (Allowed 52+26-tone MRUs in a 40 MHz EHT PPDU). (#3272)

Figure 36-6 – Allowed 52+26-tone MRUs in a 40 MHz EHT PPDU

The allowed 52+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz or 320 MHz EHT PPDU are shown in Figure 36-7 (Allowed 52+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz, or 320 MHz EHT PPDU).

Figure 36-7 – Allowed 52+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz, or 320 MHz EHT PPDU

The allowed 106+26-tone MRUs in a 20 MHz EHT PPDU are shown in Figure 36-8 (Allowed 106+26-tone MRUs in a 20 MHz EHT PPDU). (#3272)

Figure 36-8 – Allowed 106+26-tone MRUs in a 20 MHz EHT PPDU

The allowed 106+26-tone MRUs in a 40 MHz EHT PPDU are shown in Figure 36-9 (Allowed 106+26-tone MRUs in a 40 MHz EHT PPDU). (#3272)

Figure 36-9 – Allowed 106+26-tone MRUs in a 40 MHz EHT PPDU

The allowed 106+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz or 320 MHz EHT PPDU are shown in Figure 36-10 (Allowed 106+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz, or 320 MHz EHT PPDU). (#3272)

Figure 36-10 – Allowed 106+26-tone MRUs in each 80 MHz segment of an 80 MHz, 160 MHz, or 320 MHz EHT PPDU

The location of the small size MRUs are fixed as defined in Table 36-8 (Indices for small size MRUs in an OFDMA 20 MHz EHT PPDU), Table 36-9 (Indices for small-size MRUs in an OFDMA 40 MHz EHT PPDU), Table 36-10 (Indices for small size MRUs in an OFDMA 80 MHz EHT PPDU), Table 36-11 (Indices for small size MRUs in an OFDMA 160 MHz EHT PPDU), and Table 36-12 (Indices for small size MRUs in an OFDMA 320 MHz EHT PPDU).

For Table 36-8 (Indices for small size MRUs in an OFDMA 20 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 27-7 (Data and pilot subcarrier indices for RUs in a 20 MHz HE PPDU and in a non-OFDMA 20 MHz HE PPDU).

Table 36-8 - Indices for small size MRUs in an OFDMA 20 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 1 | 52-tone RU 2 + 26-tone RU 2 |
|  | MRU 2 | 52-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 52-tone RU 3 + 26-tone RU 8 |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | 106-tone RU 2 + 26-tone RU 5 |

For Table 36-9 (Indices for small size MRUs in an OFDMA 40 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 27-8 (Data and pilot subcarrier indices for RUs in a 40 MHz HE PPDU and in a non-OFDMA 20 MHz HE PPDU).

Table 36-9 - Indices for small size MRUs in an OFDMA 40 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 1 | 52-tone RU 2 + 26-tone RU 2 |
|  | MRU 2 | 52-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 52-tone RU 3 + 26-tone RU 8 |
|  | MRU 4 | 52-tone RU 6 + 26-tone RU 11 |
|  | MRU 5 | 52-tone RU 6 + 26-tone RU 14 |
|  | MRU 6 | 52-tone RU 7 + 26-tone RU 17 |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | 106-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 106-tone RU 3 + 26-tone RU 14 |
|  | MRU 4 | 106-tone RU 4 + 26-tone RU 14 |

For Table 36-10 (Indices for small size MRUs in an OFDMA 80 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-5 (Data and pilot subcarrier indices for RUs in an 80 MHz EHT PPDU).

Table 36-10 - Indices for small size MRUs in an OFDMA 80 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 1 | Not defined |
|  | MRU 2 | 52-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 52-tone RU 3 + 26-tone RU 8 |
|  | MRU 4 | 52-tone RU 6 + 26-tone RU 11 |
|  | MRU 5 | 52-tone RU 6 + 26-tone RU 14 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 52-tone RU 10 + 26-tone RU 24 |
|  | MRU 9 | 52-tone RU 11 + 26-tone RU 27 |
|  | MRU 10 | 52-tone RU 14 + 26-tone RU 30 |
|  | MRU 11 | 52-tone RU 14 + 26-tone RU 33 |
|  | MRU 12 | Not defined |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | Not defined |
|  | MRU 3 | Not defined |
|  | MRU 4 | 106-tone RU 4 + 26-tone RU 14 |
|  | MRU 5 | 106-tone RU 5 + 26-tone RU 24 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 106-tone RU 8 + 26-tone RU 33 |

For Table 36-11 (Indices for small size MRUs in an OFDMA 160 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-6 (Data and pilot subcarrier indices for RUs in a 160 MHz EHT PPDU).

Table 36-11 - Indices for small size MRUs in an OFDMA 160 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 1 | Not defined |
|  | MRU 2 | 52-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 52-tone RU 3 + 26-tone RU 8 |
|  | MRU 4 | 52-tone RU 6 + 26-tone RU 11 |
|  | MRU 5 | 52-tone RU 6 + 26-tone RU 14 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 52-tone RU 10 + 26-tone RU 24 |
|  | MRU 9 | 52-tone RU 11 + 26-tone RU 27 |
|  | MRU 10 | 52-tone RU 14 + 26-tone RU 30 |
|  | MRU 11 | 52-tone RU 14 + 26-tone RU 33 |
|  | MRU 12 | Not defined |
|  | MRU 13 | Not defined |
|  | MRU 14 | 52-tone RU 18 + 26-tone RU 42 |
|  | MRU 15 | 52-tone RU 19 + 26-tone RU 45 |
|  | MRU 16 | 52-tone RU 22 + 26-tone RU 48 |
|  | MRU 17 | 52-tone RU 22 + 26-tone RU 51 |
|  | MRU 18 | Not defined |
|  | MRU 19 | Not defined |
|  | MRU 20 | 52-tone RU 26 + 26-tone RU 61 |
|  | MRU 21 | 52-tone RU 27 + 26-tone RU 64 |
|  | MRU 22 | 52-tone RU 30 + 26-tone RU 67 |
|  | MRU 23 | 52-tone RU 30 + 26-tone RU 70 |
|  | MRU 24 | Not defined |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | Not defined |
|  | MRU 3 | Not defined |
|  | MRU 4 | 106-tone RU 4 + 26-tone RU 14 |
|  | MRU 5 | 106-tone RU 5 + 26-tone RU 24 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 106-tone RU 8 + 26-tone RU 33 |
|  | MRU 9 | 106-tone RU 9 + 26-tone RU 42 |
|  | MRU 10 | Not defined |
|  | MRU 11 | Not defined |
|  | MRU 12 | 106-tone RU 12 + 26-tone RU 51 |
|  | MRU 13 | 106-tone RU 13 + 26-tone RU 61 |
|  | MRU 14 | Not defined |
|  | MRU 15 | Not defined |
|  | MRU 16 | 106-tone RU 16 + 26-tone RU 70 |

For Table 36-12 (Indices for small size MRUs in an OFDMA 320 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-7 (Data and pilot subcarrier indices for RUs in a 320 MHz EHT PPDU).

Table 36-12 - Indices for small size MRUs in an OFDMA 320 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 1 | Not defined |
|  | MRU 2 | 52-tone RU 2 + 26-tone RU 5 |
|  | MRU 3 | 52-tone RU 3 + 26-tone RU 8 |
|  | MRU 4 | 52-tone RU 6 + 26-tone RU 11 |
|  | MRU 5 | 52-tone RU 6 + 26-tone RU 14 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 52-tone RU 10 + 26-tone RU 24 |
|  | MRU 9 | 52-tone RU 11 + 26-tone RU 27 |
|  | MRU 10 | 52-tone RU 14 + 26-tone RU 30 |
|  | MRU 11 | 52-tone RU 14 + 26-tone RU 33 |
|  | MRU 12 | Not defined |
|  | MRU 13 | Not defined |
|  | MRU 14 | 52-tone RU 18 + 26-tone RU 42 |
|  | MRU 15 | 52-tone RU 19 + 26-tone RU 45 |
|  | MRU 16 | 52-tone RU 22 + 26-tone RU 48 |
|  | MRU 17 | 52-tone RU 22 + 26-tone RU 51 |
|  | MRU 18 | Not defined |
|  | MRU 19 | Not defined |
|  | MRU 20 | 52-tone RU 26 + 26-tone RU 61 |
|  | MRU 21 | 52-tone RU 27 + 26-tone RU 64 |
|  | MRU 22 | 52-tone RU 30 + 26-tone RU 67 |
|  | MRU 23 | 52-tone RU 30 + 26-tone RU 70 |
|  | MRU 24 | Not defined |
|  | MRU 25 | Not defined |
|  | MRU 26 | 52-tone RU 34 + 26-tone RU 79 |
|  | MRU 27 | 52-tone RU 37 + 26-tone RU 82 |
|  | MRU 28 | 52-tone RU 38 + 26-tone RU 85 |
|  | MRU 29 | 52-tone RU 38 + 26-tone RU 88 |
|  | MRU 30 | Not defined |
|  | MRU 31 | Not defined |
|  | MRU 32 | 52-tone RU 42 + 26-tone RU 98 |
|  | MRU 33 | 52-tone RU 43 + 26-tone RU 101 |
|  | MRU 34 | 52-tone RU 46 + 26-tone RU 104 |
|  | MRU 35 | 52-tone RU 46 + 26-tone RU 107 |
|  | MRU 36 | Not defined |
|  | MRU 37 | Not defined |
|  | MRU 38 | 52-tone RU 50 + 26-tone RU 116 |
|  | MRU 39 | 52-tone RU 51 + 26-tone RU 119 |
|  | MRU 40 | 52-tone RU 54 + 26-tone RU 122 |
|  | MRU 41 | 52-tone RU 54 + 26-tone RU 125 |
|  | MRU 42 | 52-tone RU 55 + 26-tone RU 128 |
|  | MRU 43 | 52-tone RU 58 + 26-tone RU 132 |
|  | MRU 44 | 52-tone RU 58 + 26-tone RU 135 |
|  | MRU 45 | 52-tone RU 59 + 26-tone RU 138 |
|  | MRU 46 | 52-tone RU 62 + 26-tone RU 141 |
|  | MRU 47 | 52-tone RU 62 + 26-tone RU 144 |
|  | MRU 48 | Not defined |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | Not defined |
|  | MRU 3 | Not defined |
|  | MRU 4 | 106-tone RU 4 + 26-tone RU 14 |
|  | MRU 5 | 106-tone RU 5 + 26-tone RU 24 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 106-tone RU 8 + 26-tone RU 33 |
|  | MRU 9 | 106-tone RU 9 + 26-tone RU 42 |
|  | MRU 10 | Not defined |
|  | MRU 11 | Not defined |
|  | MRU 12 | 106-tone RU 12 + 26-tone RU 51 |
|  | MRU 13 | 106-tone RU 13 + 26-tone RU 61 |
|  | MRU 14 | Not defined |
|  | MRU 15 | Not defined |
|  | MRU 16 | 106-tone RU 16 + 26-tone RU 70 |
|  | MRU 17 | 106-tone RU 17 + 26-tone RU 79 |
|  | MRU 18 | Not defined |
|  | MRU 19 | Not defined |
|  | MRU 20 | 106-tone RU 20 + 26-tone RU 88 |
|  | MRU 21 | 106-tone RU 21 + 26-tone RU 98 |
|  | MRU 22 | Not defined |
|  | MRU 23 | Not defined |
|  | MRU 24 | 106-tone RU 24 + 26-tone RU 107 |
|  | MRU 25 | 106-tone RU 25 + 26-tone RU 116 |
|  | MRU 26 | Not defined |
|  | MRU 27 | Not defined |
|  | MRU 28 | 106-tone RU 28 + 26-tone RU 125 |
|  | MRU 29 | 106-tone RU 29 + 26-tone RU 135 |
|  | MRU 30 | Not defined |
|  | MRU 31 | Not defined |
|  | MRU 32 | 106-tone RU 32 + 26-tone RU 144 |

It is mandatory for a STA to support the transmission and reception of 52+26-tone and 106+26-tone MRUs for OFDMA EHT PPDU. (#1962)

36.3.2.3.3 Large size multiple RUs

36.3.2.3.3.1 Large size multiple RUs for non-OFDMA (#2787)

The large size MRUs defined for DL and UL non-OFDMA transmissions are as follows: 484+242-tone MRU, 996+484-tone MRU, 996+484+242-tone MRU, 2×996+484-tone MRU, 3×996-tone MRU, and 3×996+484-tone MRU. (#1990)

The 484+242-tone MRU is allowed in a non-OFDMA 80 MHz EHT PPDU. The 484+242-tone MRU is obtained by puncturing any one of four 242-tone RUs in the 80 MHz EHT PPDU. The data subcarriers of a 484+242-tone MRU consist of the data subcarriers of the 484-tone and 242-tone RUs that make up the 484+242-tone MRU. The pilot subcarriers of a 484+242-tone MRU consist of the pilot subcarriers of the 484-tone and 242-tone RUs that make up the 484+242-tone MRU. The four allowed 484+242-tone MRUs in a non-OFDMA 80 MHz EHT PPDU are shown in Figure 36-11 (Allowed 484+242-tone MRUs in a non-OFDMA 80 MHz EHT PPDU).

Figure 36-11 – Allowed 484+242-tone MRUs in a non-OFDMA 80 MHz EHT PPDU

The 996+484-tone MRU is allowed in a non-OFDMA 160 MHz EHT PPDU. The 996+484-tone MRU is obtained by puncturing any one of four 484-tone RUs in the 160 MHz EHT PPDU. The data subcarriers of a 996+484-tone MRU consist of the data subcarriers of the 996-tone and 484-tone RUs that make up the 996+484-tone MRU. The pilot subcarriers of a 996+484-tone MRU consist of the pilot subcarriers of the 996-tone and 484-tone RUs that make up the 996+484-tone MRU. The four allowed 996+484-tone MRUs in a non-OFDMA 160 MHz EHT PPDU are shown in Figure 36-12 (Allowed 996+484-tone MRUs in a non-OFDMA 160 MHz EHT PPDU).

Figure 36-12 – Allowed 996+484-tone MRUs in a non-OFDMA 160 MHz EHT PPDU

The 996+484+242-tone MRU is allowed in a non-OFDMA 160 MHz EHT PPDU. The 996+484+242 tone MRU is obtained by puncturing any one of eight 242-tone RUs in the 160 MHz EHT PPDU. The data subcarriers of a 996+484+242-tone MRU consist of the data subcarriers of the 996-tone, 484-tone, and 242-tone RUs that make up the 996+484+242-tone MRU. The pilot subcarriers of a 996+484+242-tone MRU consist of the pilot subcarriers of the 996-tone, 484-tone, and 242-tone RUs that make up the 996+484+242-tone MRU. The eight allowed 996+484+242-tone MRUs in a non-OFDMA 160 MHz EHT PPDU are shown in Figure 36-13 (Allowed 996+484+242-tone MRUs in a non-OFDMA 160 MHz EHT PPDU).

Figure 36-13 – Allowed 996+484+242-tone MRUs in a non-OFDMA 160 MHz EHT PPDU

The 2×996+484-tone MRU is allowed in a non-OFDMA 320 MHz EHT PPDU. The 2×996+484-tone MRU is obtained by puncturing either the first or the fourth 996-tone RU in a 320 MHz EHT PPDU and puncturing any one of six 484-tone RUs in the remaining 240 MHz. The data subcarriers of a 2×996+484-tone MRU consist of the data subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484-tone MRU. The pilot subcarriers of a 2×996+484-tone MRU consist of the pilot subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484-tone MRU. The twelve allowed 2×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU are shown in Figure 36-14 (Allowed 2×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

Figure 36-14 – Allowed 2×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU

The 3×996-tone MRU is allowed in a non-OFDMA 320 MHz EHT PPDU. The 3×996-tone MRU is obtained by puncturing any one of four 996-tone RUs in the 320 MHz EHT PPDU. The data subcarriers of a 3×996-tone MRU consist of the data subcarriers of the three 996-tone RUs that make up the 3×996-tone MRU. The pilot subcarriers of a 3×996-tone MRU consist of the pilot subcarriers of the three 996-tone RUs that make up the 3×996-tone MRU. The four allowed 3×996-tone MRUs in a non-OFDMA 320 MHz EHT PPDU are shown in Figure 36-15 (Allowed 3×996-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

Figure 36-15 – Allowed 3×996-tone MRUs in a non-OFDMA 320 MHz EHT PPDU

The 3×996+484-tone MRU is allowed in a non-OFDMA 320 MHz EHT PPDU. The 3×996+484-tone MRU is obtained by puncturing any one of eight 484-tone RUs in the 320 MHz EHT PPDU. The data subcarriers of a 3×996+484-tone MRU consist of the data subcarriers of the three 996-tone RUs and 484-tone RU that make up the 3×996+484-tone MRU. The pilot subcarriers of a 3×996+484-tone MRU consist of the pilot subcarriers of the three 996-tone RUs and 484-tone RU that make up the 3×996+484-tone MRU. The eight allowed 3×996+484-MRUs in a non-OFDMA 320 MHz EHT PPDU are shown in Figure 36-16 (Allowed 3×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

Figure 36-16 – Allowed 3×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU

It is mandatory for a STA to support the transmission and reception of a 484+242-tone MRU in a non-OFDMA 80 MHz EHT PPDU, a 996+484-tone MRU and a 996+484+242-tone MRU in a non-OFDMA 160 MHz EHT PPDU, 2×996+484-tone, 3×996-tone, and 3×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU unless the MRU size is larger than STA’s supported bandwidth. (#1299, #1297, #1298)

36.3.2.3.3.2 Large size multiple RUs for OFDMA (#2393)

The large size MRU defined for DL and UL OFDMA transmissions are as follows: 484+242-tone MRU, 996+484-tone MRU, 2×996+484-tone MRU, 3×996-tone MRU, and 3×996+484-tone MRU. (#1991)

The 484+242-tone MRU is allowed in OFDMA 80 MHz, 160 MHz, and 320 MHz EHT PPDU. The 484+242-tone MRU is obtained by combining a 484-tone RU and a 242-tone RU. The data subcarriers of a 484+242-tone MRU consist of the data subcarriers of the 484-tone and 242-tone RUs that make up the 484+242-tone MRU. The pilot subcarriers of a 484+242-tone MRU consist of the pilot subcarriers of the 484-tone and 242-tone RUs that make up the 484+242-tone MRU. For an OFDMA 80 MHz EHT PPDU, the four allowed 484+242-tone MRUs are shown in Figure 36-17 (Allowed 484+242-tone MRUs in an OFDMA 80 MHz EHT PPDU).

Figure 36-17 – Allowed 484+242-tone MRUs in an OFDMA 80 MHz EHT PPDU

For OFDMA transmission in 160 MHz and 320 MHz, the allowed combinations for a 484+242-tone MRU in an OFDMA 80 MHz EHT PPDU are allowed in each 80 MHz segment of OFDMA 160 MHz and 320 MHz EHT PPDU.

The 996+484-tone MRU is allowed in OFDMA 160 MHz and 320 MHz EHT PPDU. The 996+484-tone MRU is obtained by combining a 996-tone RU and a 484-tone RU. The data subcarriers of a 996+484-tone MRU consist of the data subcarriers of the 996-tone and 484-tone RUs that make up the 996+484-tone MRU. The pilot subcarriers of a 996+484-tone MRU consist of the pilot subcarriers of the 996-tone and 484-tone RUs that make up the 996+484-tone MRU. For an OFDMA 160 MHz EHT PPDU, the four allowed 996+484 tone MRUs are shown in Figure 36-18 (Allowed 996+484-tone MRUs in an OFDMA 160 MHz EHT PPDU).

Figure 36-18 – Allowed 996+484-tone MRUs in an OFDMA 160MHz EHT PPDU

For OFDMA transmission in 320 MHz, the allowed combinations for a 996+484-tone MRU in OFDMA 160 MHz EHT PPDU are allowed only within primary 160 MHz channel or secondary 160 MHz channel, respectively.

The 2×996+484-tone MRU is allowed in an OFDMA 320 MHz EHT PPDU. The 2×996+484-tone MRU is obtained by combining two 996-tone RUs and a 484-tone RU. The data subcarriers of a 2×996+484-tone MRU consist of the data subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484-tone MRU. The pilot subcarriers of a 2×996+484-tone MRU consist of the pilot subcarriers of the two 996-tone RUs and 484-tone RU that make up the 2×996+484-tone MRU. The twelve allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU are shown in Figure 36-19 (Allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU).

Figure 36-19 – Allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU

The 3×996-tone MRU is allowed in an OFDMA 320 MHz EHT PPDU. The 3×996-tone MRU is obtained by combining three 996-tone RUs. The data subcarriers of a 3×996-tone MRU consist of the data subcarriers of the three 996-tone RUs that make up the 3×996-tone MRU. The pilot subcarriers of a 3×996-tone MRU consist of the pilot subcarriers of the three 996-tone RUs that make up the 3×996-tone MRU. The four allowed 3×996-tone MRUs in an OFDMA 320 MHz EHT PPDU are shown in Figure 36-20 (Allowed 3×996-tone MRUs in an OFDMA 320 MHz EHT PPDU).

Figure 36-20 – Allowed 3×996-tone MRUs in an OFDMA 320 MHz EHT PPDU

The 3×996+484-tone MRU is allowed in an OFDMA 320 MHz EHT PPDU. The 3×996+484-tone MRU is obtained by combining three 996-tone RUs and a 484-tone RU. The data subcarriers of a 3×996+484-tone MRU consist of the data subcarriers of the three 996-tone RUs and 484-tone RU that make up the 3×996+484-tone MRU. The pilot subcarriers of a 3×996+484-tone MRU consist of the pilot subcarriers of the three 996-tone RUs and 484-tone RU that make up the 3×996+484 tone MRU. The eight allowed 3×996+484-MRUs in an OFDMA 320 MHz EHT PPDU are shown in Figure 36-21 (Allowed 3×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU).

Figure 36-21 – Allowed 3×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU

It is mandatory for a STA to support the transmission and reception of 484+242-tone MRU in each 80 MHz segment, 996+484-tone MRU in the primary 160 MHz channel and the secondary 160 MHz channel, 2×996+484-tone MRU, 3×996-tone MRU, and 3×996+484-tone MRU in 320 MHz channel for an OFDMA EHT PPDU unless the MRU is outside of the STA operating channel width. (#1249, #1250, #3275)

36.3.2.3.3.3 Location of large size multiple RUs

The location of the large size MRUs are fixed as defined in Table 36-13 (Indices for large size MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU), Table 36-14 (Indices for large size MRUs in a 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU), and Table 36-15 (Indices for large size MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU).

For Table 36-13 (Indices for large size MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-5 (Data and pilot subcarrier indices for RUs in an 80 MHz EHT PPDU).

Table 36-13 - Indices for large size MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484] (#3154) |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 empty-RU242 RU484] |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 empty-RU242 RU242] |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 empty-RU242] |

For Table 36-14 (Indices for large size MRUs in a 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-6 (Data and pilot subcarrier indices for RUs in a 160 MHz EHT PPDU).

Table 36-14 - Indices for large size MRUs in a 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484] in lower 80 MHz channel |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 empty-RU242 RU484] in lower 80 MHz channel |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 empty-RU242 RU242] in lower 80 MHz channel |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 empty-RU242] in lower 80 MHz channel |
|  | MRU 5 | 484-tone RU 4 + 242-tone RU 6; [empty-RU242 RU242 RU484] in upper 80 MHz channel |
|  | MRU 6 | 484-tone RU 4 + 242-tone RU 5; [RU242 empty-RU242 RU484] in upper 80 MHz channel |
|  | MRU 7 | 484-tone RU 3 + 242-tone RU 8; [RU484 empty-RU242 RU242] in upper 80 MHz channel |
|  | MRU 8 | 484-tone RU 3 + 242-tone RU 7; [RU484 RU242 empty-RU242] in upper 80 MHz channel |
| 996+484-tone MRU | MRU 1 | 996-tone RU 2 + 484-tone RU 2; [empty-RU484 RU484 RU996] |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 1; [RU484 empty-RU484 RU996] |
|  | MRU 3 | 996-tone RU 1 + 484-tone RU 4; [RU996 empty-RU484 RU484] |
|  | MRU 4 | 996-tone RU 1 + 484-tone RU 3; [RU996 RU484 empty-RU484] |
| 996+484+242-tone MRU (Only for non-OFDMA) | MRU 1 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484 RU996] |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 1; [RU242 empty-RU242 RU484 RU996] |
|  | MRU 3 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 4; [RU484 empty-RU242 RU242 RU996] |
|  | MRU 4 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 empty-RU242 RU996] |
|  | MRU 5 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 6; [RU996 empty-RU242 RU242 RU484] |
|  | MRU 6 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 5; [RU996 RU242 empty-RU242 RU484] |
|  | MRU 7 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 8; [RU996 RU484 empty-RU242 RU242] |
|  | MRU 8 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 7; [RU996 RU484 RU242 empty-RU242] |

For Table 36-15 (Indices for large size MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU), the indices for MRUs are defined based on the indices for RUs in Table 36-7 (Data and pilot subcarrier indices for RUs in a 320 MHz EHT PPDU).

Table 36-15 - Indices for large size MRUs in a 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 empty-RU242 RU484] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 empty-RU242 RU242] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 empty-RU242] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 5 | 484-tone RU 4 + 242-tone RU 6; [empty-RU242 RU242 RU484] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 6 | 484-tone RU 4 + 242-tone RU 5; [RU242 empty-RU242 RU484] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 7 | 484-tone RU 3 + 242-tone RU 8; [RU484 empty-RU242 RU242] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 8 | 484-tone RU 3 + 242-tone RU 7; [RU484 RU242 empty-RU242] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 9 | 484-tone RU 6 + 242-tone RU 10; [empty-RU242 RU242 RU484] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 10 | 484-tone RU 6 + 242-tone RU 9; [RU242 empty-RU242 RU484] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 11 | 484-tone RU 5 + 242-tone RU 12; [RU484 empty-RU242 RU242] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 12 | 484-tone RU 5 + 242-tone RU 11; [RU484 RU242 empty-RU242] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 13 | 484-tone RU 8 + 242-tone RU 14; [empty-RU242 RU242 RU484] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 14 | 484-tone RU 8 + 242-tone RU 13; [RU242 empty-RU242 RU484] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 15 | 484-tone RU 7 + 242-tone RU 16; [RU484 empty-RU242 RU242] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 16 | 484-tone RU 7 + 242-tone RU 15; [RU484 RU242 empty-RU242] in upper 80 MHz channel in upper 16 0MHz |
| 996+484-tone MRU | MRU 1 | 996-tone RU 2 + 484-tone RU 2; [empty-RU484 RU484 RU996] in lower 160 MHz |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 1; [RU484 empty-RU484 RU996] in lower 160 MHz |
|  | MRU 3 | 996-tone RU 1 + 484-tone RU 4; [RU996 empty-RU484 RU484] in lower 160 MHz |
|  | MRU 4 | 996-tone RU 1 + 484-tone RU 3; [RU996 RU484 empty-RU484] in lower 160 MHz |
|  | MRU 5 | 996-tone RU 4 + 484-tone RU 6; [empty-RU484 RU484 RU996] in upper 160 MHz |
|  | MRU 6 | 996-tone RU 4 + 484-tone RU 5; [RU484 empty-RU484 RU996] in upper 160 MHz |
|  | MRU 7 | 996-tone RU 3 + 484-tone RU 8; [RU996 empty-RU484 RU484] in upper 160 MHz |
|  | MRU 8 | 996-tone RU 3 + 484-tone RU 7; [RU996 RU484 empty-RU484] in upper 160 MHz |
| 996+484+242-tone MRU (Only for non-OFDMA) | MRU 1 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 2; [empty-RU242 RU242 RU484 RU996] in lower 160 MHz |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 1; [RU242 empty-RU242 RU484 RU996] in lower 160 MHz |
|  | MRU 3 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 4; [RU484 empty-RU242 RU242 RU996] in lower 160 MHz |
|  | MRU 4 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 empty-RU242 RU996] in lower 160 MHz |
|  | MRU 5 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 6; [RU996 empty-RU242 RU242 RU484] in lower 160 MHz |
|  | MRU 6 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 5; [RU996 RU242 empty-RU242 RU484] in lower 160 MHz |
|  | MRU 7 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 8; [RU996 RU484 empty-RU242 RU242] in lower 160 MHz |
|  | MRU 8 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 7; [RU996 RU484 RU242 empty-RU242] in lower 160 MHz |
|  | MRU 9 | 996-tone RU 4 + 484-tone RU 6 + 242-tone RU 10; [empty-RU242 RU242 RU484 RU996] in upper 160 MHz |
|  | MRU 10 | 996-tone RU 4 + 484-tone RU 6 + 242-tone RU 9; [RU242 empty-RU242 RU484 RU996] in upper 160 MHz |
|  | MRU 11 | 996-tone RU 4 + 484-tone RU 5 + 242-tone RU 12; [RU484 empty-RU242 RU242 RU996] in upper 160 MHz |
|  | MRU 12 | 996-tone RU 4 + 484-tone RU 5 + 242-tone RU 12; [RU484 RU242 empty-RU242 RU996] in upper 160 MHz |
|  | MRU 13 | 996-tone RU 3 + 484-tone RU 8 + 242-tone RU 14; [RU996 empty-RU242 RU242 RU484] in upper 160 MHz |
|  | MRU 14 | 996-tone RU 3 + 484-tone RU 8 + 242-tone RU 13; [RU996 RU242 empty-RU242 RU484] in upper 160 MHz |
|  | MRU 15 | 996-tone RU 3 + 484-tone RU 7 + 242-tone RU 16; [RU996 RU484 empty-RU242 RU242] in upper 160 MHz |
|  | MRU 16 | 996-tone RU 3 + 484-tone RU 7 + 242-tone RU 15; [RU996 RU484 RU242 empty-RU242] in upper 160 MHz |
| 2×996+484-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 2; [empty-RU484 RU484 RU996 RU996 empty-RU996] |
|  | MRU 2 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 1; [RU484 empty-RU484 RU996 RU996 empty-RU996] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 3 + 484-tone RU 4; [RU996 empty-RU484 RU484 RU996 empty-RU996] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 3 + 484-tone RU 3; [RU996 RU484 empty-RU484 RU996 empty-RU996] |
|  | MRU 5 | 996-tone RU 1 + 996-tone RU 2 + 484-tone RU 6; [RU996 RU996 empty-RU484 RU484 empty-RU996] |
|  | MRU 6 | 996-tone RU 1 + 996-tone RU 2 + 484-tone RU 5; [RU996 RU996 RU484 empty-RU484 empty-RU996] |
|  | MRU 7 | 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 4; [empty-RU996 empty-RU484 RU484 RU996 RU996] |
|  | MRU 8 | 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 3; [empty-RU996 RU484 empty-RU484 RU996 RU996] |
|  | MRU 9 | 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 6; [empty-RU996 RU996 empty-RU484 RU484 RU996] |
|  | MRU 10 | 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 5; [empty-RU996 RU996 RU484 empty-RU484 RU996] |
|  | MRU 11 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 8; [empty-RU996 RU996 RU996 empty-RU484 RU484] |
|  | MRU 12 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 7; [empty-RU996 RU996 RU996 RU484 empty-RU484] |
| 3×996-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4; [empty-RU996 RU996 RU996 RU996] |
|  | MRU 2 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4; [RU996 empty-RU996 RU996 RU996] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4; [RU996 RU996 empty-RU996 RU996] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3; [RU996 RU996 RU996 empty-RU996] |
| 3×996+484-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 2; [empty-RU484 RU484 RU996 RU996 RU996] |
|  | MRU 2 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 1; [RU484 empty-RU484 RU996 RU996 RU996] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 4; [RU996 empty-RU484 RU484 RU996 RU996] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 3; [RU996 RU484 empty-RU484 RU996 RU996] |
|  | MRU 5 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 6; [RU996 RU996 empty-RU484 RU484 RU996] |
|  | MRU 6 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 5; [RU996 RU996 RU484 empty-RU484 RU996] |
|  | MRU 7 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 8; [RU996 RU996 RU996 empty-RU484 RU484] |
|  | MRU 8 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 7; [RU996 RU996 RU996 RU484 empty-RU484] |

------------- Visio files for the figures--------------------