IEEE P802.11
Wireless LANs

| CR for Clause 36.3.22 Channel Numbering and Annex E |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ruchen Duan | Samsung |  |  | r.duan@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC34 on P802.11be D0.3

The following 2 comments under “36.3.22 Channel Numbering”  and Annex E are resolved:

CID 1577, 1956

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CIDs: 1577, 1956

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1577 | 355.51 | 36.3.22 | for the indication of 320MHz, 320_1, and 320_2 has been defined. But, 320_1 and 320_2 are not applied in 27.3.23.2. | define the channel number by considering the 320_1 and 320_2 | Rejected. Channel numbering is not related with BW. Definition for center frequency of 320_1 and 320_2 are defined in Annex E as in the CR for CID 1956. |
| 1956 | 385.18 | Annex E | Define 320-1 and 320-2 | as in comment | Rejected 320-1 and 320-2 is only different in location/center frequency. Giving them different operating class seems not necessary. In the scenario where operating class will be used, channel numbering will also be provided, I believe that there is no ambiguity if we put 320-1 and 320-2 in a same class. The purpose of mentioning 320-1 and 320-2 is to remove ambiguity in BW location for a PPDU, and this differentiation is only needed in PHY preamble. Defnition for 320-1 and 320-2 is required in CID 3175. And the definition can be provided based on current Annex E. “Set to 4 for 320 MHz EHT PPDU occupying one of the channels defined in the Operating class 137 with center frequency 31, 95, 159. Set to 5 for 320 MHz EHT PPDU occupying one of the channels defined in the Operating class 137 with center frequency 63, 127, 191” |

Proposed Changes: None