IEEE P802.11
Wireless LANs

| CR for Clause 36.3.22 Channel Numbering and Annex E |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ruchen Duan | Samsung |  |  | r.duan@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC34 on P802.11be D0.3

The following 2 comments under “36.3.22 Channel Numbering”  and Annex E are resolved:

CID 1577, 1956

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CIDs: 1577, 1956

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1577 | 355.51 | 36.3.22 | for the indication of 320MHz, 320_1, and 320_2 has been defined. But, 320_1 and 320_2 are not applied in 27.3.23.2. | define the channel number by considering the 320_1 and 320_2 | Revised. Change subclause name to “36.3.22 Channel numbering and channelization”, and add subclause “36.3.22.1 Cnannelization for 320MHz” |
| 1956 | 385.18 | Annex E | Define 320-1 and 320-2 | as in comment | Rejected. In CR for CID 1577, 320-1 and 320-2 are defined in 36.3.22. |

Proposed Changes:

## Instruction to the Editor: Modify texts in the 36.3.22 as follows.

(The baseline for this text is D0.3)

- 36.3.22 Channel numbering and channelization

The STA may operate in the 2.4 GHz band, 5 GHz band or 6 GHz band. The set of valid operating channel numbers by regulatory domain is defined in Annex E. Channel allocation for each band is defined as follows:

— In 19.3.15.2 (Channel allocation in the 2.4 GHz band) for the 2.4 GHz band

— In 19.3.15.3 (Channel allocation in the 5 GHz band) for the 5 GHz band

— In 27.3.23.2 (Channel allocation in the 6 GHz band) for the 6 GHz band

36.3.22.1 Cnannelization for 320MHz

The 320 MHz channel can be any two adjacent 160MHz channel. Two type of channelizatins for 320MHz channel are defined: 320-1 and 320-2. 320-1 is defined as 320MHz channel with channel center frequency numbered 31, 95, 159, and 320-2 is defined as 320MHz channel with channel center frequency numbered 63, 127, 191.