IEEE P802.11
Wireless LANs

| Segment Parser CR on P802.11be D0.3 – part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bo Gong | Huawei Technologies |  |  | gongbo8@hisilicon.com |
| Jianhan Liu | Mediatek |  |  | Jianhan.Liu@mediatek.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission shows

- Resolution for a comment received from TGbe comment collection (TGbe Draft D0.3)

- The proposed changes are based on 11be D0.4.

The submission provides resolutions to 10 CIDs, including 1587, 2442, 2443, 2672, 2673, 2817, 2951, 2952, 2953 and 3072.

## CID 1587

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 287.1 | 36.3.12 | Frequency domain duplication for EHT-MCS14 block is defined in the transmitter block diagram. A new subsection which adresses this block is also needed in the Data field section. | Add a new subsection for frequency domain duplication into the Data field section and describe its detail. | Revised The details for frequency domain duplication for EHT-MCS 14 block and the related changes in the context are included in PDT 11/21-0139r3, which has been added in D0.4 as 36.3.13.10 Frequency domain duplication. Note to the editor: the proposed changes have been reflected in P802.11be D0.4 and no further changes are needed. |

## CID 2442

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.35 | 36.3.12.5 | correct a parameter in an expression | Correct to k=(Ncbpss,l,u-nl*44*Nbpscs,l,u),...,Ncbpss,l,u-1 (at the end of the expression there should be Ncbpss and not Nbpscs) | Revised Agreed in principle. Reflect the detailed explanation Instructions to the editor Please make the changes as shown in 11/21-0443r1 |

Instructions to the editor:

In Line35 Page 296 of P802.11be D0.3 or Line26 P400 of P802.11be D0.4, the maximum value of k is  instead of ,the expression

should be revised as

.

## CID 2672

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 (36-61) | sum range for getting offset of m with-in each (summation m_l) block should be sum of m for all previous segments | the second term in the equation should be summation m_i from i=0 to i=l-1 and not from i=0 to i=L-1, i.e. summation upto previous segment | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r1 are the same |

## CID 2817

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 | sum over m_i in (36-61) should be from 0 to l-1, not from 0 to L-1. For example for m_i=1, i=0:, .., 3, we should have: y_k,l = x_4k+l. | Correct. Also correct in (36-62) | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r1 are the same. |

## CID 2951

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 295.59 | 36.3.12.5 | In the equation \sum_{i=0}^{L-1}m_i should be \sum_{i=0}^{l-1}m_i | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2672 CID 2817 and CID 2951 in 11/21-0443r1 are the same. |

Instructions to the editor:

In Line 59 Page 295 of P802.11be D0.3 or Line50 P399 of P802.11be D0.4, the second term of equation (36-61) of P802.11be D0.3 or equation (36-68) of P802.11be D0.4,, should be revised as , in which,  is revised as .

## CID 2443

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 | correct a parameter in an expression | The last sum (sum(mi)) should be until "l-1" and not "L-1" and also for "i!=lo" and not "i!=i0" | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r1 are the same. |

## CID 2673

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 (36-62) | sum range for getting offset of m with-in each (summation m_l) block should be sum of m for all previous segments | same comment as 36-61 but here it applies to third term | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r1 are the same. |

## CID 2953

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.30 | 36.3.12.5 | The last "L-1" should be "l-1" | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 Note to the editor: The resolutions for CID 2443, CID 2673, CID 2953 in 11/21-0443r1 are the same. |

Instructions to the editor:.

In Line 30 Page 296 of P802.11be D0.3 or Line 21 Page 400 of P802.11be D0.4, the third term of the equation (36-62) of P802.11be D0.3 or the equation (36-69) of P802.11be D0.4,, should be revised as , in which,  is revised as  and  is revised as .

- CID 2952

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.20 | 36.3.12.5 | "L-1" should be "l-1" | Correct the equation. | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 |

## CID 3072

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 296.20 | 36.3.12.5 | change "for subblock l=0 " to "for frequency subblock l=0" | as commented | Revised Agreed in principle. Reflect the detailed explanation. Instructions to the editor Please make the changes as shown in 11/21-0443r1 |

Instructions to the editor:

In Line 20 Page 296 of P802.11be D0.3 or Line 11 Page 400 of P802.11be D0.4, the expression “ for subblock  ” should be revised as

“ for frequency subblock ”, where  is revised as  and “subblock” is changed to “frequency subblock”.