IEEE P802.11
Wireless LANs

| EHT-SIG CR on P802.11be D0.3 – part 7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3. The instruction to the editor are based on P802.11be D0.4 for Editor’s convenience.

3 comments:

CID 2198 2644 2645

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 2198

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 263.11 | 36.3.11.8.5 | the description for STA-ID in table 36-32 (and 36-31) says: "Indicate the STA-ID related information." I assume this is the ID for the intended STA recipient or a special value like 2046 as in 11ax? If so, then perhaps this could be clarified by borrowing the description from 11ax. If not, then where does the reader find more information on what the 'related information' is? | Change "Indicate the STA-ID related information." to "Set to a value indicated from TXVECTOR parameter STA_ID (see 26.11.1 (STA_ID))." or provide further details on the related information for this subfield. | Revised Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0507r0 |

Instructions to the editor

Please make the changes to L10, P362 of P802.11 D0.4 as shown below:

Table 36-39—User field format for a non-MU-MIMO allocation

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B0–B10 | STA-ID | 11 | Set to a value of the TXVECTOR parameter STA_ID (see 26.11.1 (STA_ID)). |

Please make the changes to L10, P363 of P802.11 D0.4 as shown below:

Table 36-40—User field format for a MU-MIMO allocation

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B0–B10 | STA-ID | 11 | Set to a value of the TXVECTOR parameter STA_ID (see 26.11.1 (STA_ID)). |

## CID 2644

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 0.00 | 36.3.11.8.5 | User field when STA ID = 2046 | In the case of STA ID = 2046, table 36-31 suggests that NSS is not arbitrary, but doesn't give an indication what the transmitter should fill in here. | Revised Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0507r0 |

Instructions to the editor

Please make the changes to L30, P362 of P802.11 D0.4 as shown below:

Table 36-39—User field format for a non-MU-MIMO allocation

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B16–B19 | NSS | 4 | Indicate the number of spatial streams. Set to the number of spatial streams minus 1 for up to 8 spatial streams. If the STA-ID matches, other values are Validate. If the STA-ID doesn’t match, other values are Disregard. Set to an arbitrary value if the STA-ID subfield is 2046. |

## CID 2645

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 0.00 | 36.3.11.8.5 | User field when STA ID = 2046 | In the case of STA ID = 2046, table 36-32 suggests that Spatial Configuration is not arbitrary, but doesn't give an indication what the transmitter should fill in here. | Revised Agreed in principle. Reflect the detailed changes. Instructions to the editor Please make the changes as shown in 11/21-0507r0 |

Please make the changes to L38, P363 of P802.11 D0.4 as shown below:

Table 36-40—User field format for a MU-MIMO allocation

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B16–B21 | Spatial Configuration | 6 | Indicates the number of spatial streams for a user in an MU-MIMO allocation (see Table 36-41 (Spatial Configuration subfield encoding)). Set to an arbitrary value if the STA-ID subfield is 2046. |