IEEE P802.11
Wireless LANs

| EHT CR 2661 and 2671 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.03.23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Edward Au |  |  |  |  |
| Mengshi Hu |  |  |  |  |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3. The instruction to the editor are based on P802.11be D0.4 for Editor’s convenience.

2 comments:

CID 2661 and 2671

are resolved in this document.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 2661

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 197.60 | 36.3.4 | Incorrect name | Change "triggering frame" to "Trigger frame" | Rejected triggering frame is a correct name: “triggering frame: A Trigger frame or a frame carrying a TRS Control subfield.” |

Discussion

In Line 60 page 197, it is

“The format of the EHT TB PPDU is defined as Figure 36-23 (EHT TB PPDU format). This format is used for a transmission that is a response to a triggering frame from an AP.”

Triggering frame is a correct name. In P802.11ax D8.0, page 47, L48, it shows:

“triggering frame: A Trigger frame or a frame carrying a TRS Control subfield.”

End of discussion

## CID 2671

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 313.(36-78),(36-79) | 36.3.13 | Equations (36-78) and (36-79) should be TBD since the value of parameter LENGTH is TBD in the description | Mark (36-78) and (36-79) as TBD. | Revised The LENGTH is defined in 11/21-0259r4 now. Instead of adding more TBDs, we can remove the TBD now. Instructions to the editor Please make the changes as shown in 11/21-0527r0. |

Discussion

In 11/21-0259r4, the passed PDT of trigger frame, the meaning of LENGTH is already clear.

An EHT non-AP STA that responds to a Trigger frame that solicits an EHT TB PPDU shall set the TXVECTOR parameters below as follows:

- The FORMAT parameter is set to EHT_TB

- The L_LENGTH parameter is set to the value indicated by the UL Length subfield in the Common Info field of the Trigger frame.(#M7)

Hence we can remove the TBD now and simply follows 11ax:

End of discussion

Instructions to the editor

Please make the changes to L41, P417 of P802.11 D0.4 as shown below:

LENGTH is the value indicated by UL Length subfield in the Common Info field of the Trigger frame.