IEEE P802.11
Wireless LANs

| CR for CID 1249, 1250, 1962, 3275 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.04.29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Myeongjin Kim | Samsung |  |  | mj1108.kim@samsung.com |
| WookBong Lee | Samsung |  |  | wookbong.lee@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC34 on P802.11be D0.3

R0:

- The following 4 comments under “36.3.2.3 Subcarriers and resource allocation for multiple RUs” are resolved:

- CID 1249, 1250, 1962, 3275

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CIDs: 1249, 1250, 1962, 3275

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1249 | 194.30 | 36.3.2.3.3 | "It is mandatory for non-AP STA to support the transmission and reception"'. Should also clarify the mandatory requirement for AP | as in comment | Rejected. Based on the passed motion (Motion 115, #SP72) in SFD, large size MRU combinations in OFDMA are mandatory for non-AP STA only. |
| 1250 | 194.34 | 36.3.2.3.3 | "unless the MRU size is larger than its supported bandwidth" should be changed "unless MRU is outside of the STA support BW" since a 484+242 MRU in seconday 80MHz has size less than an 80MHz STA's supportiing BW but can not be supported by this STA | as in comment | Rejected. 36.3.2.2 (Support of wide bandwidth OFDM operation) specifies that RU/MRUs which are not fully within the STA’s operating location do not need to be supported. |
| 1962 | 187.45 | 36.3.2.3.2 | "It is mandatory for a non-AP STA to support the transmission and reception of 26+52 tone and 26+106 tone MRUs in OFDMA." What about AP? The support of small-size MRUs should be mandatory at the AP too. | Add the mandaory support of small size MRUs for AP. At least add the mandatory receive of small size MRU at the AP. | Rejected. Base on the passed motion (Motion 115, #SP71) in SFD, small size MRU combinations in OFDMA are mandatory for non-AP STA only. |
| 3275 | 194.33 | 36.3.2.3.3 | In "2×996+484 tone MRU, 3×996-tone MRU, and 3×996+484 tone MRU in 80/160/320 MHz PPDU in OFDMA", it should be 320 MHz PPDU in OFDMA. | delete 80/160/ from the sentence. | Revised. Agree with the commenter. But, for the better description, “for an OFDMA EHT PPDU” is changed to “for an OFDMA 80/160/320 MHz EHT PPDU” in this sentence. Instructions to the editor: Please make the changes as shown in 21/0748r0 |

Instructions to the editor

Please make the changes to P194 L33 as shown below: (The baseline for this text is 21/0417r3.)

It is mandatory for a non-AP STA to support the transmission and reception of 484+242-tone MRU in each 80 MHz frequency subblock, 996+484-tone MRU in the primary 160 MHz channel and the secondary 160 MHz channel, 2×996+484-tone MRU, 3×996-tone MRU, and 3×996+484-tone MRU in 320 MHz channel for an OFDMA 80/160/320 MHz EHT PPDU unless the MRU size is larger than its supported bandwidth.