IEEE P802.11
Wireless LANs

| CR for EHT Default PE Duration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.05.12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mengshi Hu | Huawei Technologies | F3-6-A118, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | humengshi@huawei.com |
| Ross Yu | Huawei Technologies |  |  |  |
| Yuchen Guo | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D0.3.

2 comments realted to EHT Default PE Duration:

CID 1651 and 1652

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 1651

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 146.47 | 35.4.1.1 | The maximum PE duration is 20 us for EHT TB PPDU as well as EHT MU PPDU, but the current default PE duration signaling does not support 20 us. | Define the method to set DEFAULT_PE_DURATION parameter as 20 us | REVISED Define the method to signal 20 us by the Default PE Duration subfield. Instructions to the editor: Please make the changes as shown in 11/21-0827r0 |

## CID 1652

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 313.58 | 36.3.13 | The maximum PE duration is 20 us for EHT TB PPDU as well as EHT MU PPDU, but the current default PE duration signaling does not support 20 us. | Define the method to signal 20 us as the default PE duration, or define a restriction regarding the default PE duration of 20 us | REVISED Define the method to signal 20 us by the Default PE Duration subfield. Note to the Editor: the resolutions for CIDs 1651 and 1652 are the same. |

Instructions to the editor (for CID 1651 and 1652)

Please make the changes to P114, L1 in D0.4 as shown below:

The format of the EHT Operation element is shown in Figure 9-788ee (EHT Operation element format).

|  | Element ID | Length | Element ID Extension | EHT Operation Parameters | EHT Operation Information |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | Y | X |

Figure 9-788ee – EHT Operation element format

Please make the changes to P114, L38 in D0.4 as shown below:

The format of the EHT Operation Parameters field is defined in Figure 9-xxx (EHT Operation Parameters field format).

|  | EHT Default PE Duration | .,.. |
| --- | --- | --- |
| Bits: | 1 | 8Y-1 |

Figure 9-xxx – EHT Operation Parameters field format

The EHT Default PE Duration subfield is set to 1 to indicate that the PE field duration for an EHT TB PPDU solicited by a TRS Control subfield is 20 µs and set to 0 to indicate that the PE field duration is the same as that indicated in the HE Operation Parameters field in the HE Operation element. The use of the PE field duration indicated in the EHT Default PE Duration subfield is defined in 35.4.2.3 (Non-AP STA behavior for UL MU operation).

End of the proposed change

Discussion:

(The following shows another design of the EHT Default PE Duration subfield)

The format of the EHT Operation Parameters field is defined in Figure 9-xxx (EHT Operation Parameters field format).

|  | EHT Default PE Duration | .,.. |
| --- | --- | --- |
| Bits: | 3 | 8Y-3 |

Figure 9-xxx – EHT Operation Parameters field format

The EHT Default PE Duration subfield indicates the PE field duration in units of 4 µs for an EHT TB PPDU that is solicited by a TRS Control subfield and its use is defined in 35.4.2.3 (Non-AP STA behavior for UL MU operation). Values 0-5 of the EHT Default PE Duration subfield indicates 0, 4, 8, 12, 16, 20 µs, respectively, and values 6-7 of the Default PE Duration subfield are reserved.

Appendix: