Wireless LANs

| CC36 CR on Spatial Configuration Subfield |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

3 comments are resolved.

CID 5483, 5484, 4952

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Add if dot11EHTBaseLineFeaturesImplementedOnly equals true. |

## CID 5483

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 449.60 | 36.3.12.8.5 | Remove entries that with NSS_total larger than 8 in Table 36-42 | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Instruction to the editor, please making the changes as shown in 11/21-1042r1, under CID 5483 and CID 4952. |

## CID 4952

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 450.29 | 36.3.12.8.5 | Many places in the spec the total number of spatial streams is limited to 8, e.g. 36.1.1 "total across all users not exceeding eight spatial streams". So why are there entries where Total N_SS is greater than 8? Seems like those should be invalid entries | as in comment | REVISED Agreed in principle. Reflect the detailed changes. Instruction to the editor, please making the changes as shown in 11/21-1042r1, under CID 5483 and CID 4952. |

Discussion:

11be R1 supports up to 8 spatial streams for now. Hence it is reasonable to remove the entries with Total Nss larger than 8. And the columns Nss[1] to Nss[8] and total entries will be the same as those in Table 27-30—Spatial Configuration subfield encoding of 802.11ax-2021. Whilst the difference is the bitwidth of the Spatial Configuration subfield (6 bit in 11be and 4 bit in 11ax). The indications (B5-B0) for those remained entries will not change. Later in R2, we will go back to the original table as shown in P802.11be D1.01.

Discussion ends

Instruction to the editor, please making the changes to Table 36-42—Spatial Configuration subfield encoding in Page 472, line 29 of P802.11be D1.01

Table 36-42—Spatial Configuration subfield encoding

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 000000– 000011 | 1–4 | 1 |  |  |  |  |  |  | 2–5 | 10 |
|  | 000100– 000110 | 2–4 | 2 |  |  |  |  |  |  | 4–6 |  |
|  | 000111– 001000 | 3–4 | 3 |  |  |  |  |  |  | 6–7 |  |
|  | 001001 | 4 | 4 |  |  |  |  |  |  | 8 |  |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 000000– 000011 | 1–4 | 1 | 1 |  |  |  |  |  | 3–6 | 13 |
|  | 000100– 000110 | 2–4 | 2 | 1 |  |  |  |  |  | 5–7 |  |
|  | 000111– 001000 | 3–4 | 3 | 1 |  |  |  |  |  | 7–8 |  |
|  | 001010– 001100 | 2–4 | 2 | 2 |  |  |  |  |  | 6–8 |  |
|  | 001101 | 3 | 3 | 2 |  |  |  |  |  | 8 |  |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 000000– 000011 | 1–4 | 1 | 1 | 1 |  |  |  |  | 4–7 | 11 |
|  | 000100– 000110 | 2–4 | 2 | 1 | 1 |  |  |  |  | 6–8 |  |
|  | 000111 | 3 | 3 | 1 | 1 |  |  |  |  | 8 |  |
|  | 001010– 001011 | 2–3 | 2 | 2 | 1 |  |  |  |  | 7–8 |  |
|  | 010100 | 2 | 2 | 2 | 2 |  |  |  |  | 8 |  |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 000000– 000011 | 1–4 | 1 | 1 | 1 | 1 |  |  |  | 5–8 | 7 |
|  | 000100– 000101 | 2–3 | 2 | 1 | 1 | 1 |  |  |  | 7–8 |  |
|  | 001010 | 2 | 2 | 2 | 1 | 1 |  |  |  | 8 |  |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 |  |  |  |  |  |  |  |  |  |  | 7 |
| 6 | 000000– 000010 | 1–3 | 1 | 1 | 1 | 1 | 1 |  |  | 6–8 | 4 |
|  | 000100 | 2 | 2 | 1 | 1 | 1 | 1 |  |  | 8 |  |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 |  |  |  |  |  |  |  |  |  |  | 4 |
| 7 | 000000– 000001 | 1–2 | 1 | 1 | 1 | 1 | 1 | 1 |  | 7–8 | 2 |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 011101– 011110 | 3–4 | 3 | 3 | 3 | 1 | 1 | 1 |  | 15–16 | 2 |
|  | 011111– 100001 | 2–4 | 2 | 2 | 2 | 2 | 1 | 1 |  | 12–14 |  |
|  | 100010– 100011 | 3–4 | 3 | 2 | 2 | 2 | 1 | 1 |  | 14–15 |  |
|  | 100100 | 4 | 4 | 2 | 2 | 2 | 1 | 1 |  | 16 |  |
|  | 100101– 100110 | 3–4 | 3 | 3 | 2 | 2 | 1 | 1 |  | 15–16 |  |
|  | 100111 | 3 | 3 | 3 | 3 | 2 | 1 | 1 |  | 16 |  |
|  | 101000– 101010 | 2–4 | 2 | 2 | 2 | 2 | 2 | 1 |  | 13–15 |  |
|  | 101011– 101100 | 3–4 | 3 | 2 | 2 | 2 | 2 | 1 |  | 15–16 |  |
|  | 101101 | 3 | 3 | 3 | 2 | 2 | 2 | 1 |  | 16 |  |
|  | 101110– 110000 | 2–4 | 2 | 2 | 2 | 2 | 2 | 2 |  | 14–16 |  |
|  | 110001 | 3 | 3 | 2 | 2 | 2 | 2 | 2 |  | 16 |  |
| 8 | 000000 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 8 | 1 |

Table 36-42—Spatial Configuration subfield encoding (continued)

| Nuser | B5…B0 | NSS [1] | NSS [2] | NSS [3] | NSS [4] | NSS [5] | NSS [6] | NSS [7] | NSS [8] | Total NSS | Total entries |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  |  |  |  |  |  |  |  |  |  | 1 |

## CID 5484

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 449.39 | 36.3.12.8.5 | Undefiend values are Validate if STA-ID matches and Disregard if STA-ID doesn't match | Add the description as in comment | REVISED Agreed in principle. Reflect the detailed changes. Instruction to the editor, please making the changes as shown in 11/21-1042r1, under CID 5484. |

Instruction to the editor, please making the changes to Page 471, line 42 of P802.11be D1.01

Table 36-41—User field format for a MU-MIMO allocation

| B16–B21 | Spatial Configuration | 6 | Indicates the number of spatial streams for a user in an MU-MIMO allocation (see Table 36-42 (Spatial Configuration subfield encoding)). If STA-ID matches, the values that don’t exist in Table 36-42 (Spatial Configuration subfield encoding)) are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true. If STA-ID does not match, the values that do not exist in Table 36-42 (Spatial Configuration subfield encoding)) are Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true.. |
| --- | --- | --- | --- |