Wireless LANs

| CC36 CR on Spatial Configuration Subfield |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

3 comments are resolved.

CID 5483, 5484, 4952

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Add if dot11EHTBaseLineFeaturesImplementedOnly equals true. |
| R2 | For CID 5484, all values shall be Disregard if STA-ID does not match |
| R3 | Changes during presentation |

## CID 5484

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 449.39 | 36.3.12.8.5 | Undefiend values are Validate if STA-ID matches and Disregard if STA-ID doesn't match | Add the description as in comment | REVISED Agreed in principle. Reflect the detailed changes. Instruction to the editor, please making the changes as shown in 11/21-1042r3, under CID 5484. |

Instruction to the editor, please making the changes to Page 471, line 42 of P802.11be D1.01

Table 36-41—User field format for a MU-MIMO allocation

| B16–B21 | Spatial Configuration | 6 | Indicates the number of spatial streams for a user in an MU-MIMO allocation (see Table 36-42 (Spatial Configuration subfield encoding)). If STA-ID matches, the values that are reserved or don’t exist in Table 36-42 (Spatial Configuration subfield encoding)) are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true. If STA-ID does not match, all values are Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true. |
| --- | --- | --- | --- |