IEEE P802.11
Wireless LANs

| CC36 CR on 36.3.12.8.3 Common field for OFDMA transmission – Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Stephen McCann |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

24 comments are resolved.

CID 4670, 4854, 4855, 4856, 5416, 5417, 5418, 5420, 5478, 5479, 5528, 6153, 6438, 6439, 6467, 6468, 6469, 7002, 8110, 8111, 8112, 8113, 8114, 8115

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Remove “Number Of Non-OFDMA Users” row in Common field for EHT sounding NDP, which was a typo caused by copy and paste |
| R2 | Change RU/MRU to RU or MRU |
| R3 | Changes during presentation |

## CID 6438

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8 | Is the value of Spatial Resue subfield different between 80 MHz frequency subblocks? We may clarify whether the value of Spatial Resue subfield is same in every EHT-SIG content channel or not. | Clarify whether the value of Spatial Resue subfield is same in every EHT-SIG content channel or not. | Revised The spec already has the following in which spatial reuse is part of U-SIG Overflow bits: B0–B16 of Table 36-33 (Common field for OFDMA transmission) are U-SIG Overflow bits for OFDMA transmission and are duplicated in each content channel. On the other hand, there is no need to mention the specific subfields after Tables 36-33, 36-36, and 36-37 because these are duplicated. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID 4670. |

## CID 5418

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8.3 | This paragraph is redundant since it's been said that the U-SIG Overflow bits are duplicated in each content channel. Ditto P443L25, P445L1. | Remove this paragraph. | Revised Also reflect the changes for P443L25, P445L1. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID 4670. |

## CID 8114

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8.3 | discrepancy between the first paragraph and the third paragraph. U-SIG overflow bits include Spatial Reuse but in the third paragraph, it is missed. Moreover the third paragraph itself is redundant. Better to delete it. | as in comment | Revised Also reflect the changes for P443L25, P445L1. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID 4670. |

## CID 7002

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8.3 | Remove the 'Value for' at the beginning, it is redundant. | Remove 'Value for' at the beginning. | REVISED The paragraph has been removed. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID 4670 |

## CID 4670

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8.3 | "Value" lacks an article | Change to "Each value of ... has" | REVISED The paragraph has been removed. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID 4670 |

Instructions to the editor, please make the following changes to P455, L50 of P802.11be D1.01:

Instructions to the editor, please make the following changes to P465, L24 of P802.11be D1.01:

Instructions to the editor, please make the following changes to P467, L1 of P802.11be D1.01:

## CID 6439

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 435.21 | 36.3.12.8.3 | Remove a space before a comma in this sentence. | As in comment | ACCEPTED |

## CID 4854

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 432.08 | 36.3.12.8.3 | When BW is smaller than 160MHz, the RU Allocation-2 subfield in table 36-33 is not present. So, to apply this, change "M" with "0 or M" | As in comment | Revised Propose to set M = 0, when RU Allocation-2 subfield does not exist. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID5528. |

## CID 5528

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 432.08 | 36.3.12.8.3 | RU Allocation-2 is conditionally present, so the number of subfields should be changed as "0 or M", or modify the text in Description as ".. where M is equal to 2 or 6 as follows and the subfields are not present (i.e. M = 0) otherwise:" | As in comment | Revised Reflect the detailed changes Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID5528. |

Instructions to the editor, please make the following changes to P454, L15 of P802.11be D1.01:

(#2810)M RU Allocation-2 subfields are present in an EHT-SIG content channel if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU where M is equal to 2 or 6 as follows

M is set to 2 if the Bandwidth field in the U-SIG field is 3 (indicating a 160 MHz EHT MU PPDU).

M is set to 6 if the Bandwidth field in the U-SIG field is 4 or 5(#3189).

The subfields are not present otherwise (i.e., M is equal to 0):

## CID 4855

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.01 | 36.3.12.8.3 | B13-B16 are Disregards bits and are not U-SIG overflow bits. change B0-B16 with B0-B12 | As in comment | Rejected Disregard bits are also part of U-SIG Overflow bits. |

## CID 4856

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 441.48 | 36.3.12.8.3 | The dynamic split is performed by AP. So, the text "(on a per case )" does not need, delete (on a per case ) | As in comment | Rejected “on a per case” is to clarify the split is implementation specific. |

## CID 5417

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 430.60 | 36.3.12.8.3 | "set to 1" is at Tx and "Disregard" is at Rx. Ditto P443L10, P444L47. | Change the description to "Set to all 1s and Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true." | Revised. Reflect the changes for all fields/subfields in EHT-SIG Common. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID5416 |

## CID 5416

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 430.38 | 36.3.12.8.3 | Should specify clearly that other values of the Number of EHT-LTF symbols are Validate for R1 devices. Ditto P442L38, P444L34 | Change to "Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true." | Revised. Reflect the changes for all fields/subfields in EHT-SIG Common. Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID5416 |

Instructions to the editor, please make the following changes to P452, L31 of P802.11be D1.01:

Table 36-33—Common field for OFDMA transmission

| Bit | Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- | --- |
| B6–B8 | Number Of EHT-LTF Symbols | 1 | 3 | Indicate the number of EHT-LTF symbols: Set to 0 to indicate 1 EHT-LTF symbol. Set to 1 to indicate 2 EHT-LTF symbols. Set to 2 to indicate 4 EHT-LTF symbols. Set to 3 to indicate 6 EHT-LTF symbols. Set to 4 to indicate 8 EHT-LTF symbols. Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true.(#2735)(#3188) |
| B13-B16 | Disregard | 1 | 4 | Set to all 1s. Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true. |

Table 36-36—Common field for non-OFDMA transmission to a single user and non-OFDMA transmission to multiple users

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B6–B8 | Number Of EHT-LTF Symbols | 3 | Indicate the number of EHT-LTF symbols: Set to 0 to indicate 1 EHT-LTF symbol. Set to 1 to indicate 2 EHT-LTF symbols. Set to 2 to indicate 4 EHT-LTF symbols. Set to 3 to indicate 6 EHT-LTF symbols. Set to 4 to indicate 8 EHT-LTF symbols. Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true.(#2736)(#3188) |
| B13-B16 | Disregard | 4 | Set to all 1s. Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true. |
| B17-B19 | Number Of Non-OFDMA Users | 3 | (#3004)Indicates the total number of non-OFDMA users. Set to n to indicate n+1 non-OFDMA users. Set to 0 for non-OFDMA transmission to a single user and set to a value larger than 0 for non-OFDMA transmission to multiple users. Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true. |

Table 36-37—Common field for EHT sounding NDP

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B4-B5 | GI+LTF Size | 2 | Indicates the GI duration and EHT-LTF size: Set to 0 to indicate 2x LTF + 0.8 μs GI. Set to 1 to indicate 2x LTF + 1.6 μs GI. Value 2 is Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true. Set to 3 to indicate 4x LTF + 3.2 μs GI.(#3188)(#2737) |
| B6–B8 | Number Of EHT-LTF Symbols | 3 | Indicate the number of EHT-LTF symbols: Set to 0 to indicate 1 EHT-LTF symbol. Set to 1 to indicate 2 EHT-LTF symbols. Set to 2 to indicate 4 EHT-LTF symbols. Set to 3 to indicate 6 EHT-LTF symbols. Set to 4 to indicate 8 EHT-LTF symbols. Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true.(#2737)(#3188) |
| B9-B12 | NSS | 4 | (#3081)Indicates the number of spatial streams of the EHT sounding NDP: Set to the number of spatial streams minus 1 for up to 8 spatial streams. Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true.(#2737) |
| B14-B15 | Disregard | 2 | Set to both 1s. Disregard if dot11EHTBaseLineFeaturesImplementedOnly equals true. |

## CID 5420

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 434.34 | 36.3.12.8.3 | It's weird to have the discussion on how to label and determine the number of user fields here without first introduce the RU allocation subfield mapping (Table 36-35 and the description paragraphs, e.g., explaining "Punctured 242-tone RU" and "Unassigned 242-tone RU", etc.). | Suggest to move contents from P437L1 to P441L44 to before this paragraph. | ACCEPTED |

## CID 5478

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 431.19 | 36.3.12.8.3 | Add MRU too in the descriptions | as in comment | Revised Reflect the detailed changes Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID8110 |

## CID 5479

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 432.22 | 36.3.12.8.3 | Add MRU too in the descriptions | as in comment | Revised Reflect the detailed changes Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID8110 |

## CID 8113

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 432.16 | 36.3.12.8.3 | add indicating a320MHz EHT MU PPDU at the end of ... U-SIG field is 4 or 5 or delete the text of (indicating a 160 MHz EHT MU PPDU) | as in comment | Revised Choose the second method and reflect the detailed changes Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID8110 |

## CID 8110

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 431.11 | 36.3.12.8.3 | add indicating a 80MHz, 160MHz, or 320MHz EHT MU PPDU at the end of ... 2, 3, 4, or 5. or delete the text of (indicating a 20 MHz or 40 MHz EHT MU PPDU) | as in comment | Revised Choose the second method and reflect the detailed changes Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID8110 |

Instructions to the editor, please make the following changes to P452, L31 of P802.11be D1.01:

Table 36-33—Common field for OFDMA transmission

| Bit | Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- | --- |
| B17–B16+9N | RU Allocation-1 | N | 9 | N RU Allocation-1 subfields are present in an EHT-SIG content channel, where: N is set to 1 if the Bandwidth field in the U-SIG field is equal to 0 or 1. N is set to 2 if the Bandwidth field in the U-SIG field is equal to 2, 3, 4, or 5(#3189). (#1394)(#1395)(#1279)Each RU Allocation-1 subfield in an EHT-SIG content channel corresponding to a 20 MHz frequency subchannel indicates the RU or MRU assignment, including the size of the RU(s) or MRU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain, where the subcarrier indices of the RU(s) meet the conditions in Table 36-34 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth). Each RU Allocation-1 subfield also indicates information needed to compute the number of users allocated to each of these RU(s) or MRU(s). |
| B27+9N –B26+9N+9M | RU Allocation-2 | M | 9 | (#2810)M RU Allocation-2 subfields are present in an EHT-SIG content channel if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU where M is equal to 2 or 6 as follows and the subfields are not present otherwise: M is set to 2 if the Bandwidth field in the U-SIG field is 3. M is set to 6 if the Bandwidth field in the U-SIG field is 4 or 5(#3189). (#1394)(#1395)(#1279)Each RU Allocation-2 subfield in an EHT-SIG content channel corresponding to a 20 MHz frequency subchannel indicates the RU or MRU assignment, including the size of the RU(s) or MRU(s) and their placement in the frequency domain, to be used in the EHT modulated fields of the EHT MU PPDU in the frequency domain, where the subcarrier indices of the RU(s) meet the conditions in Table 36-34 (RUs associated with each RU Allocation subfield for each EHT-SIG content channel and PPDU bandwidth). Each RU Allocation-2 subfield also indicates information needed to compute the number of users allocated to each of these RU(s) or MRU(s). |

## CID 8111

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 431.16 | 36.3.12.8.3 | move the text from "Each RU Allocation 1 to each of these RU(s)" below the Table 36-33. | as in comment | Rejceted The text cited is to describe the function of RU Allocation-1 subfield. |

## CID 8112

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 432.21 | 36.3.12.8.3 | move the text from "Each RU Allocation 2 to each of these RU(s)" below the Table 36-33. | as in comment | Rejceted The text cited is to describe the function of RU Allocation-2 subfield. |

## CID 6153

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 434.20 | 36.3.12.8.3 | "RU allocation subfield" should be "RU Allocation subfield" | RU allocation subfield should be "RU Allocation subfield" | Accepted |

## CID 6467

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.41 | 36.3.12.8.3 | the text "An RU Allocation subfield shall not indicate an RU which spans the entire PPDU bandwidth" should refer to MRU as well | change "...indicate an RU which..." to "...indicate an RU/MRU which..." | Revised Instructions to the editor: please reflect the proposed change, but change RU/MRU to RU or MRU |

## CID 6469

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 434.42 | 36.3.12.8.3 | This line should refer to MRU as well | Change "RU" to "RU/MRU" | Revised Also revise similar place in the previous paragraph Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID6468 |

## CID 6468

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 434.40 | 36.3.12.8.3 | This line should refer to MRU as well | Change the beginning of the sentence to "For an RU/MRU that is..." | Revised Also revise similar place in the previous paragraph Instructions to the editor: Please make the changes to the spec as shown in 11/21-1057r3, under CID6468 |

Instructions to the editor, please make the following changes to P456, L33 of P802.11be D1.01:

For an RU or MRU that is referred to by a first or only one RU Allocation subfield in an EHT-SIG content channel, the RU Allocation subfield encodes the number of User fields per RU contributed to the User Specific field in the same EHT-SIG content channel as the RU Allocation subfield. This number is labeled for RU r and EHT-SIG content channel c as described in Table 36-35 (RU Allocation subfield).

For an RU or MRU that is referred to by two or more RU Allocation subfields in an EHT-SIG content channel (e.g., a 996-tone RU in a 160 MHz PPDU), the RU Allocation subfield other than the first one in the EHT-SIG content channel encodes zero additional User fields per RU or MRU contributed to the User Specific field in the same EHT-SIG content channel as the RU Allocation subfield.

In an EHT MU PPDU, an RU or MRU that is not allocated to a user can be indicated as follows:

—The RU Allocation subfield in the EHT-SIG Common field is set to 26 or 27 (see Table 36-35 (RU Allocation subfield)).

—The STA-ID subfield in the EHT-SIG User field is set to 2046 (#1399)for RU or MRU with fewer than 242 tones (see 35.8.1.1 (STA_ID) and 36.3.12.8.5 (User Specific field)).

—The RU Allocation subfield in the EHT-SIG Common field is set to 24 (see Table 36-35 (RU Allocation subfield)). In this case, the middle 26-tone RU is not allocated.

## CID 8115

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 433.08 | 36.3.12.8.3 | only RU Allocation subfield and a single RU Allocation subfield are used in mix. Make it consistent in this subclause. | as in comment | Revised Instructions to the editor: Please change “only RU Allocation subfield” to “single RU allocation subfield” in page 456, line 33 of P802.11be D1.01. |