IEEE P802.11
Wireless LANs

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Stephen McCann | Huawei Technologies Co., Ltd | Southampton, United Kingdom |  | stephen.mccann@ieee.org |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.2.

3 comment resolutions are proposed:

CIDs 4095, 4291 and 5072

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Updates to clause references based on P802.11be D1.2 |

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4095 | 74.24 | 9.2.4.7.1 | Updated Table 9-25 to include a column for EHT | As in comment | REVISED Agreed in principle. Instructions to the editor: Please make the changes as shown in 11/21-1072r1, with the reference CID 4095. |
| 4291 | 0.0 | 9.2.4.7.1 | Update table to add EHT PPDU limits.(references relative to TGax 8.0) | As in comment. | REVISED Agreed in principle. Instructions to the editor: Please make the changes as shown in 11/21-1072r1, with the reference CID 4291. |
| 5072 | 71.07 | 9.2.4 | Add Table 9-25 from baseline 802.11ax-2021 spec and extend it to include a column for EHT PHY. | As in comment. | REVISED Agreed in principle. Instructions to the editor: Please make the changes as shown in 11/21-1072r1, with the reference CID 5072. |

Instructions to the editor, please add the following new table to a new clause 9.2.4.8.1 at P96, L24 of P802.11be D1.2.

The indicated changes show the new additional column for EHT from IEEE 802.11 REVme D0.2 (e.g. from the rolled in IEEE 802.11ax-2021)

- Frame Body field

- General

|  |  | - Maximum data unit sizes (in octets) and durations (in microseconds)input.doc |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Non-HT non-VHT non-HE(11ax) non-S1G non-DMG PPDU and non-HT duplicate PPDU |  | HT PPDU | VHT PPDU | HE PPDU | EHT PPDU | S1G PPDU | DMG PPDU |
| MMPDU size | 2304 |  | 2304 | See NOTE 1 | See NOTE 1 | See NOTE 1 | See NOTE 1 | 2304 |
| MSDU size | 2304 |  | 2304 | 2304 | 2304 | 2304 | 2304 | 7920 |
| A-MSDU size | 3839 or 4065 (see NOTE 2) (HT STA, see also Table 9-215 (Subfields of the HT Capability Information field)), or N/A (non-HT STA, see also 10.11 (A-MSDU operation)) |  | 3839 or 7935 (see also Table 9-215 (Subfields of the HT Capability Information field)) | See NOTE 3 | 2.4 GHz band: 3839 or 7935 (see also Table 9-215 (Subfields of the HT Capability Information field)) Otherwise: see NOTE 3 | 2.4 GHz band: 3839 or 7935 (see also Table 9-215 (Subfields of the HT Capability Information field)) Otherwise: see NOTE 3 | See NOTE 3 | 7935 |
| MPDU size | See NOTE 4 |  | See NOTE 5 | 3895 or 7991 or 11 454 (see also Table 9-303 (Subfields of the VHT Capabilities Information field)) | 2.4 GHz band: see NOTE 5 Otherwise: 3895 or 7991 or 11 454 (see also Table 9-303 (Subfields of the VHT Capabilities Information field)) See NOTE 7 | 2.4 GHz band: see NOTE 5 6 GHz band: 3895, 7991 or 11 454 (see also 9.4.2.263 (HE 6 GHz Band Capabilities element)) Otherwise: 3895 or 7991 or 11 454 (see also Table 9-303 (Subfields of the VHT Capabilities Information field)) See NOTE 7 | 3895 or 7991 (see also Table 9-335 (Subfields of the S1G Capabilities Information field)) | See NOTE 5 |
| PSDU size | 212–1 (see Table 15-5 (DSSS PHY characteristics), Table 16-4 (HR/DSSS PHY characteristics), Table 17-21 (OFDM PHY characteristics), Table 18-5 (ERP characteristics)) |  | 216–1 (see Table 19-25 (HT PHY characteristics)) | 4 692 480 (~222.16) (see Table 21-28 (VHT PHY characteristics)) | 6 500 631 (~222.63) (see Table 27-54 (HE PHY characteristics)) | 15 523 200 (~223.88) (see Table 36-69 (EHT PHY characteristics)) | 797 160 (~219.60) (see Table 23-40 (S1G PHY characteristics)) | 218–1 (see Table 20-30 (DMG PHY characteristics)) |
| PPDU duration | See NOTE 6 |  | 5484 (HT_MF; see 10.27.4 (L_LENGTH and L_DATARATE parameter values for HT-mixed format PPDUs)) or 10 000 (HT_GF; see Table 19-25 (HT PHY characteristics)) | 5484 (see Table 21-28 (VHT PHY characteristics)) | 5484 (see Table 27-54 (HE PHY characteristics)) | 5484 (see Table 36-69 (EHT PHY characteristics)) | 27 840 (see Table 23-40 (S1G PHY characteristics)) | 2000 (see Table 20-30 (DMG PHY characteristics)) |
| NOTE 1—No direct constraint on the maximum MMPDU size; indirectly constrained by the maximum MPDU size (see Error: Reference source not found). NOTE 2—Indirect constraint from the maximum PSDU size: 212–1 octets minus the minimum QoS Data frame overhead (26 octets for the MAC header and 4 octets for the FCS). NOTE 3—No direct constraint on the maximum A-MSDU size; indirectly constrained by the maximum MPDU size. NOTE 4—No direct constraint on the maximum MPDU size; indirectly constrained by the maximum MSDU/MMPDU or (for HT STAs only) A-MSDU size. NOTE 5—No direct constraint on the maximum MPDU size; indirectly constrained by the maximum A-MSDU size. NOTE 6—No direct constraint on the maximum duration, but an L_LENGTH value above 2332 might not be supported by some receivers (see NOTE 2 in 10.27.4 (L_LENGTH and L_DATARATE parameter values for HT-mixed format PPDUs)). NOTE 7—The maximum MPDU size might be greater than the size declared as supported by the recipient if the MPDU is an HE Compressed Beamforming/CQI frame. |  |  |  |  |  |  |  |  |