IEEE P802.11
Wireless LANs

| CR for Section 9.4.1.67 on P802.11be D1.01 part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.01.

8 comments under subclause 9.4.1.67 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 4331

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 112.30 | 9.4.1.67a | The term "EHT NDP" is not clear - does it refer to EHT sounding NDP or to EHT NDP Announcement frame? | Please clarify to which of the PPDUs this term refers: EHT sounding NDP PPDU or to EHT NDP Announcement frame. | ACCEPTED Note to the Editor: Change the text as following: Indicates the channel width used to determine the starting and ending subcarrier indices when interpreting the Partial BW Info subfields. The value of the BW subfield corresponds to the bandwidth of EHT Sounding NDP PPDU. |

## CID 4594

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 116.44 | 9.4.1.67b | In D1.0, it says "The Average SNR of Space-Time Stream i subfield in Table 9-91b (HE Compressed Beamforming Report information) is an 8-bit 2s complement integer defined in Table 9-77 (Average SNR of Space-Time Stream i subfield)." This implies following HE Compressed Beamforming Report information table. However, with multiple-RUs, a single SNR value will not reflect the channel status (considering small scale fading) if non-adjacent RUs are assigned to STA. Therefore, MCS will be mismatched to multiple-RU configuration (the two extremes are too aggressive leading to decoding failure in week channels or two pessimistic leading to unutilizing the power of multiple RUs which aims to increasing spectral efficiency). Even if SNR is averaged out with different RUs in MRU operation, the expected large fluctuation (due to non-adjacent RUs) may be problematic. | Introduce a new table with new extended signaling to provide SNR segmented levels on different RU assignment. Hence, SNR in each spatial stream can be segmented into N segments where each segment reports SNR for one RU segment. However, this will lead to some other change in MCS selection as MCS is selected as a single value but there may be either different MCS levels for different RUs in MRU operation or some unified selection based on SNR segmentation. | REJECTED Several reasons: Average SNR provides average value to define single MCS for the entire RU/MRU assigned to STA. As long as same MCS is used for all the subcarriers there is no reason to spleat average SNR value to multiple value Non-adjacent RUs (large MRU) are not supposed have less significant fluctuations in average SNR between RUs as BW is large MU Feedback type provides average SNR per subcarrier and may be used for single STA |

## CID 4889

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 112.28 | 9.4.1.67a | the values 5-7 are not used. so, add the following text in the description. " values 5-7 are reserved" | As in comment | REVISED Note to the Editor: Add the text at the end of the BW description: The values 5-7 are reserved |

## CID 4890

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 114.38 | 9.4.1.67b | In the title of table 9-91j-k. it is unclear what means the term "not cover the entire 80MHz " and "cover the entire 80MHz ". To clarify it, modify the above title of the table based on the 242 tone RU and based on 996 tones RU. | As in comment | REVISED Note to the Editor: Change table title as following: Table 9-91j—Subcarrier indices when not all bits in Partial BW Info Subfield corresponding 80 MHz Subblock are set to 1 Table 9-91k—Subcarrier indices when all bits in Partial BW Info Subfield corresponding 80 MHz subblock are set to 1 for Ng= 4 Table 9-91l—Subcarrier indices when all bits in Partial BW Info Subfield corresponding 80 MHz subblock are set to 1 for Ng= 16 Additionaly modify text on page 120.36 as following: For an EHT NDP Announcement frame of bandwidth larger than or equal to 80 MHz, in each 80 MHz subblock(#1279), if the Partial BW Info subfield indicates the feedback of the entire 80 MHz (all the bits corresponding 80MHz subblock are set to 1), the subcarrier indices of the corresponding 996-tone RU is included in the feedback, otherwise, the subcarrier indices of 242-tone RU for each 20 MHz indicated by Partial BW Info subfieldare included in the feedback report. |

## CID 4891

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 117.61 | 9.4.1.67d | Reference is wrong, correct it | Modify the wrong reference with Table 9-91g | ACCEPTED Note to the Editor: Change reference as proposed in comment |

## CID 4892

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 118.13 | 9.4.1.67d | Reference is wrong, correct it | Modify the wrong reference with Table 9-91g | ACCEPTED Note to the Editor: Change reference as proposed in comment |

## CID 5025

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 112.25 | 9.4.1.67a | The value 0 is reserved. The values 0 and 8-15 are reserved. 1) Remove duplication, 2) why 8-15 are reserved | Clarify the range of values | REVISED Note to the Editor: Change the text as following: The values 0 and 8–15 are reserved. |

## CID 5397

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 112.22 | 9.4.1.67a | The sentence "The value 0 is reserved." is redundant. | Remove the sentence | REVISED Note to the Editor: Change the text as following: The values 0 and 8–15 are reserved. Same resolution as in CID 5025. |