IEEE P802.11
Wireless LANs

| CR for Section 9.4.1.67 on P802.11be D1.01 part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.01.

10 comments under subclause 9.4.1.67 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Modifed Instrcution to editor |

## CID 5085

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 116.26 | 9.3.4.67b | Phrasing "subcarrier indices are included in feedback report" is not accurate | Change text to compressed beamfroming information corresponding subcarrier indices is included in fedback report | REVISED Instruction to the Editor: change the text as following: For an EHT NDP Announcement frame of bandwidth larger than or equal to 80 MHz, in each 80 MHz subblock(#1279), if the Partial BW Info subfield indicates the feedback of the entire 80 MHz, the compressed beamforming information related to subcarrier indices of the corresponding 996-tone RU is included in the feedback; otherwise the compressed beamforming information related to subcarrier indices of 242-tone RU for each 20 MHz indicated by Partial BW Info subfield is included in the feedback report. |

## CID 5398

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 114.15 | 9.4.1.67b | The notations of b_psi and b_phi are not explicitly defined in 9.4.1.67a. | Replace the notations by words here. Alternatively, use the same notations b_psi and b_phi in the Codebook Information subfield definition in Table 9-91i. | REVISED Instruction to the Editor: change the text as following: The angles are quantized as defined in Table 9-74 (Quantization of angles) with (bits for ) and (bits for ϕ) |

## CID 5676

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 112.28 | 9.4.1.67a | In the following "BW Indicates the channel width used to determine the starting and ending subcarrier indices when interpreting the Partial BW Info subfields", the BW subfield is used to indicate the resolution of the Partial BW subfield indication, not to indicate the starting and ending subcarrier indices. | Should be changed as follows...."BW Indicates the channel width used to determine the resolution of the Partial BW Info subfield indication when interpreting the Partial BW Info subfield" | REVISED Instruction to the Editor: Remove the following text: Indicates the channel width used to determine the starting and ending subcarrier indices when interpreting the Partial BW Info subfields. |

## CID 6083

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 116.44 | 9.4.1.67b | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | REJECTED The term is copied from the specified reference, no change is required |

## CID 6084

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 116.45 | 9.4.1.67b | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | REJECTED The term is copied from the specified reference, no change is required |

## CID 6085

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 116.49 | 9.4.1.67b | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | REJECTED The term is copied from the specified reference, no change is required |

## CID 6086

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 117.39 | 9.4.1.67c | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | REJECTED The term is copied from the specified reference, no change is required |

## CID 6087

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 118.13 | 9.4.1.67d | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | ACCEPTED Instruction to the Editor: change the text as suggested |

## CID 6088

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 118.16 | 9.4.1.67d | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | REJECTED The term is copied from the specified reference, no change is required |

## CID 6089

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 118.19 | 9.4.1.67d | The use of the term "space-time streams" is no longer correct | change "space-time streams" to "spatial streams" | ACCEPTED Instruction to the Editor: change the text as suggested |