IEEE P802.11
Wireless LANs

| CR for Section 9.3.1.19 on P802.11be D1.01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |
| Wook Bong Lee | Samsung |  |  |  |
| Edward Au | Huawei |  |  |  |
| Alfred Asterjadhi | Qualcomm Inc. |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.01.

7 comments under subclause 9.3.1.19 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | CIDs 4148 and 8262 are added. Editorial modifications on proposed text. |
| R2 | Added additional change on P80L59 |
| R3 | Change to align to P802.11az™/D3.2 |
| R4 | List of authors updated |
| R5 | Editorial changes |

## CID 4146

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.28 | 9.3.1.19 | I think there are 4 variants of NDPA (randing is missing). Please check and fix. | As in comment. | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

## CID 8262

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.28 | 9.3.1.19 | IEEE 802.11az define a type of ranging NDP Announcement frame. So there are four types. | as in the comment | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5. Same resolution as for CID #4146 |

## CID 4148

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.62 | 9.3.1.19 | Maybe a good idea to rename these bits since it is odd that a settin of ranging and HE of 1 identify an EHT variant. Either that or add a table with the different combos. Also probably a good idea to have a different subclause for each of these variants? Getting very difficult to read. | As in comment. | REVISED Instruction to the Editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

## CID 5083

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.31 | 9.3.1.19 | No Reference to Sounding Dialog Token | Add reference to section/table where Sounding Dialog Token Field is defined | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

## CID 5084

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.61 | 9.3.1.19 | Fourth option of HE Subfield and Ranging Subfield is not specified | Add specification for the option when HE Subfield and Ranging Subfield are set to 0 and 1 respectively | REVISED See discussion below Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

## CID 6005

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.28 | 9.3.1.19 | Ranging NDP Announcement varient should be mentioed here | as in the comment | REVISED See discussion below Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

## CID 5787

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 78.23 | 9.3.1.19 | NDPA frame has four variants. For readability, it is better to divide this clause into five subclauses: 1) overview; 2) VHT variant; 3) HE Variant; 4) Ranging variant; 5) EHT variant | as in the comment | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/21-1105r5, under CID 5787 |

Discussion:

802.11ax defined two subfields of Sounding Dialog Token field as HE Subfield and Reserved.

802.11az (P802.11az™/D3.2) replaced the HE and Reserved Subfields by single NDP Announcement Type subfield of two bits to indicate three variants of NDPA frame (HE/VHT/Ranging).

11be may accept the change of 11az and further modify it to add forth variant of EHT NDP Announcment frame.

Discussion Ends here.

Instructions to the editor: please make the following changes on page 80, L 59

The NDP  (#1732, #1767)  Announcement frame has three four variants, the VHT  NDP  Announcement  frame,  the  HE  NDP  Announcement  frame, and  the  Ranging  NDP Announcement  frame and EHT NDP Announcement  frame.  The  three four  formats  are  distinguished  by  the  setting  of  the  NDP Announcement Type Variant subfield in the Sounding Dialog Token field.

The  NDP  Announcement  frame  contains  at  least  one  STA  Info  field.  If the NDP Announcement frame contains only one STA Info field, then in the case of VHT or HE NDP Announcement frames the RA field is set to the address of the STA that can provide feedback (see 10.37.5.2 (Rules for VHT sounding protocol sequences)), while in the case of Ranging NDP Announcement frames, the RA address is set to the address of the RSTA or ISTA that is the intended recipient of the frame. If the NDP Announcement frame contains more than one STA Info field, then the RA field is set to the broadcast address.

The  TA  field  is  set  to  the  address  of  the  STA  transmitting  the  NDP Announcement  frame  or  the  bandwidth  signaling  TA  of  the  STA  transmitting  the NDP Announcement frame. In an EHT NDP Announcement frame transmitted by an EHT STA that is a STA 6G with 320 MHz bandwidth support in a non-HT or non-HT duplicate format and where the scrambling sequence and SERVICE field carry the TXVECTOR parameter CH_BANDWIDTH_IN_NON_HT, the TA field is a set to a bandwidth signaling TA. Otherwise, in a NDP Announcement frame transmitted  by  a  VHT,HE or  EHT STA  in  a  non-HT  or  non-HT  duplicate  format  and  where  the scrambling sequence carries the TXVECTOR parameter CH_BANDWIDTH_IN_NON_HT, the TA field is set to a bandwidth signaling TA

Change Figure 9-59 (Sounding Dialog Token field format) as follows:

|  | B0 B1 | B2 B7 |
| --- | --- | --- |
|  | NDP Announcement Variant | Sounding Dialog Token Number |
| Bits: | 2 | 6 |

## Figure 9-59 — Sounding Dialog Token field format

Instructions to the editor: please make the following changes on page 81

The setting of the NDP Announcement Type Variant subfield in the Sounding Dialog Token field identifies the variant of the NDP Announcement frame, refer to Table 9-28d NDP Announcement frame variant encoding.

| NDP Announcement Type Variant subfield |  | NDP Announcement frame Variant |  |
| --- | --- | --- | --- |
| B1 | B0 |  |  |
| 0 | 0 | VHT NDP Announcement frame |  |
| 0 | 1 | Ranging NDP Announcement frame |  |
| 1 | 0 | HE NDP Announcement frame |  |
| 1 | 1 | EHT NDP Announcement frame |  |

## Table 9-28d — NDP Announcement frame variant encoding

Change the second paragraph after Fig 9-61a

Instructions to the editor: please make the following changes on page 81

The frame format of the EHT NDP Announcement frame is the same as the HE NDP Announcement frame shown in Figure 9-61a (HE NDP Announcement frame format).

The Duration, RA, and TA fields are set as in a VHT NDP Announcement frame.

The NDP Announcement Variant subfield isset to 3 to identify the frame as an EHT NDP Announcement frame.

The Sounding Dialog Token Number field in the Sounding Dialog Token field contains a value selected by the beamformer to identify the EHT NDP Announcement frame.