IEEE P802.11
Wireless LANs

| CR for Section 36.3.5 on P802.11be D1.01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

2 comments under subclause 36.3.5 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 5093

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 375.17 | 36.3.5 | First and second RU are not clear definition | Better to use specific RU index or to define it through lower and higher frequency | REVISED Note to the Editor: Change the text as following: Please make the changes to the spec as shown in 11/21-1107r1, under CID 7971 |

## CID 7971

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 375.18 | 36.3.5 | Of the two RU484s, which one is the 'first' one? E.g., the one in lower frequency? | At P375L18, change "first 484-tone RU" to "lower 484-tone RU" in two locations. At P375L19, change "second 484-tone RU" to "higher 484-tone RU" At P375L22, change "first 996-tone RU" to "lower 996-tone RU" in two locations. At P375L24, change "second 996-tone RU" to "higher 996-tone RU" At P375L26, change "first 2x996-tone RU" to "lower 2x996-tone RU" in two locations. At P375L27, change "second 2x996-tone RU" to "higher 2x996-tone RU" | ACCEPTED Note to the Editor: Change the text as following: Please make the changes to the spec as shown in 11/21-1107r1, under CID 7971 |

Suggested change:

In EHT DUP mode, the encoding and modulation are described as follows:

—For an 80 MHz EHT MU PPDU transmitted in EHT DUP mode, encoding and BPSK-DCM

modulation are done for the lower 484-tone RU, and then the lower 484-tone RU is duplicated to the higher 484-tone RU along with a partial sign change to reduce PAPR.

— For a 160 MHz EHT MU PPDU transmitted in EHT DUP mode, encoding and BPSK-DCM

modulation are done for the lower 996-tone RU, and then the lower 996-tone RU is duplicated to the higher 996-tone RU along with a partial sign change to reduce PAPR

— For a 320 MHz EHT MU PPDU transmitted in EHT-DUP mode, encoding and BPSK-DCM

modulation are done for lower 2x996-tone RU, and then the lower 2x996-tone RU is duplicated to the higher 2x996-tone RU along with a partial sign change to reduce PAPR.