IEEE P802.11
Wireless LANs

| PDT Reference for EHT-SIG CRC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed draft text to P802.11be D1.01. It aims to add/correct reference for CRC to align with the resolution for CID 4848, 5002, 8105 as shown in 11/21-1078r0

# Revision Notes

| R0 | Initial revision |
| --- | --- |

Instructions to the editor, please make the following changes to P453, L33 of P802.11be D1.01:

Table 36-33—Common field for OFDMA transmission

| Bit | Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- | --- |
| B17+9N –B20+9N. | CRC | 1 | 4 | The CRC is calculated over bits 0 to 16+9N (see 27.3.11.7.3 (CRC computation)).. |
| B27+9N+9M –B30+9N+9M | CRC | 0 or 1 | 4 | The CRC subfield is present if the Bandwidth subfield in the U-SIG field indicates a 160 MHz, 320 MHz-1, or 320 MHz-2 EHT MU PPDU and not present otherwise. If present, the CRC is calculated over bits 27+9N to 26+9N+9M (see 27.3.11.7.3 (CRC computation)). NOTE—N=2 when the CRC subfield exists. |

Instructions to the editor, please make the following changes to P468, L29 of P802.11be D1.01:

Table 36-38—The first Encoding Block in EHT-SIG for non-OFDMA transmission to a single user and multiple users

| Bit | Subfield | Number of bits | Description |
| --- | --- | --- | --- |
| B42–B45 | CRC | 4 | The CRC is calculated over bits 0 to 41 (see 27.3.11.7.3 (CRC computation)). |

Instructions to the editor, please make the following changes to P469, L24 of P802.11be D1.01:

Table 36-39—User Block field

| Subfield | Number of subfields | Number of bits per subfield | Description |
| --- | --- | --- | --- |
| CRC | 1 | 4 | The CRC is calculated over bits 0 to 21 for a User Block field that contains one User field, and bits 0 to 43 for a User Block field that contains two User fields (see 27.3.11.7.3 (CRC computation)). |