IEEE P802.11
Wireless LANs

| CR for Section 36.3.11.12 on P802.11be D1.0 – Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following CIDs on P802.11be D1.2:

4695, 4908, 5010, 5488, 6472, 7007, 7008, 7235, 7317, 7751, 8130

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Update references to D1.2 |
| R2 | Some modification by the group and remove 7237, 7238, |

## CID 4695

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 476.60 | 36.3.12.11 | Preamble puncturing resolution for 320M PPDU shall be 40 MHz based on the passed motion | As in comment | Revised Note to the Editor: Change the text in 575.60-61 in D1.2 as follows: Preamble puncturing resolution shall be 20 MHz for an EHT MU PPDU in OFDMA transmission for bandwidth larger than 40 MHz and in non-OFDMA transmission for 80 MHz and 160 MHz bandwidth. In other words, puncturing a subchannel smaller than a 242-tone RU is not allowed in the cases above. Preamble puncturing resolution shall be 40 MHz for an EHT MU PPDU in non-OFDMA transmission for 320 MHz bandwidth. In other words, puncturing a subchannel smaller than a 484-tone RU is not allowed in 320 MHz bandwidth. |

## CID 4908

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.02 | 36.3.12.11.1 | What is the primary 20 MHz channel unmodulated? it is unclear. clarify it. | As in comment | REVISED Note to the Editor: Change the text in 576.1-2 in D1.2 as follows: In case of EHT TB PPDU, a non-AP STA might not be assigned the primary 20 MHz channel(#2709). This non-AP STA leaves the primary 20 MHz channel unoccupiedfor the entire duration of the EHT TB PPDU. |

## CID 5010

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 476.51 | 36.3.12.11.1 | Delete "For MU PPDU, U-SIG and EHT-SIG include information on the preamble puncturing." since this is also specified in 36.3.12.11.2. Section 36.3.12.11.1 which is just for a general description doesn't have to include details. | As in comment | Rejected The current wording is still general. It is not getting into details of the U-SIG/EHT-SIG content. |

## CID 5488

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.22 | 36.3.12.11.2 | Add description that one 80 MHz subblock may be punctured. And there is no signaling within that 80 MHz subblock. | As in comment | Revised Instructions to the Editor: Please add the following text in D1.2 576.22 “The puncturing pattern ‘0000’ represents a fully unoccupied 80 MHz subblock and is also a valid pattern, however contrary to the other puncturing patterns, there is no signalling in such an 80 MHz subblock.” Note to the Editor: Same resolution for CIDs 5488, 6472 |

## CID 6472

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.18 | 36.3.12.11 | There is no reference for the case where an entire 80 MHz sub-block is punctured | Add a note or a new paragraph in line 22 that says: "Puncturing pattern of 0000 is a valid one for a full punctured sub-block, however contrary to the above patterns, it cannot be signaled" | Revised Instructions to the Editor: Please add the following text in D1.2 576.22 “The puncturing pattern ‘0000’ represents a fully unmodulated 80 MHz subblock and is also a valid pattern, however contrary to the other puncturing patterns, there is no signalling in such a 80 MHz subblock.” Note to the Editor: Same resolution for CIDs 5488, 6472 |

## CID 7007

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.20 | 36.3.12.11.2 | Replace 'an "1" ' with 'a "1" '. | Replace 'an' with 'a'. | Accepted Note to the Editor: Please refer to D1.2 576.22 |

## CID 7008

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.28 | 36.3.12.11.2 | Replace 'signaled in U-SIG' by 'signaled in the U-SIG field' | Replace 'signaled in U-SIG' by 'signaled in the U-SIG field' | Accepted Note to the Editor: Please refer to D1.2 576.28 |

## CID 7235

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 476.56 | 32.3.12.11.1 | Move paragraph on lines 56-58 (definition of preamble puncturing) to beginning of section. | See comment | Accepted Note to the Editor: Please refer to D1.2 575.56 |

## CID 7317

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 476.63 | 36.3.12.11.1 | "Primary 20 MHz channel shall not be punctured in any PPDU.". This leads to a related issue: Is it OK for a PPDU to not allocate RUs in the primary 20 MHz (while sending a preamble in the primary 20 MHz)? Also, it is OK to use a (large) MRU with the "hole" coinciding with the primary 20 MHz? If allowed, add a note here to clarify that. If not allowed, let's make it explicit in the appropriate section of the draft. | See comment | Revised Instructions to the Editor: Please update the text in D1.2 575.63 Preamble puncturing shall not be applied in the primary 20 MHz channel(#2709) in an EHT MU PPDU. |

## CID 7751

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 476.51 | 36.3.12.11.1 | Replace "For MU PPDU, " with "For EHT MU PPDU" | as in comment | Accepted Note to the Editor: Please refer to D1.2 576.28 |

## CID 8130

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 477.18 | 36.3.12.11.2 | the fourth paragragh isthe details of the second paragraph. The fourth paragraph is better to be moved after the second paragraph and/or merged. | as in comment | Revised Instructions to the Editor: In D1.2, switch between the 4th paragraph (576.17-21) and the 3rd paragraph (576.14-15) |