IEEE P802.11
Wireless LANs

| Segment Deparser CR on P802.11be D1.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.11.1 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bo Gong | Huawei Technologies |  |  | gongbo8@huawei.com |
| Jianhan Liu | Mediatek |  |  | Jianhan.Liu@mediatek.com |
| Ross Jian Yu | Huawei Technologies |  |  | ross.yujian@huawei.com |

Abstract

This submission shows

- resolution for a comment received from TGbe comment collection (based on TGbe Draft D1.0)

- the proposed changes based on TGbe Draft D1.2.

The submission provides resolutions to CIDs 4646, 4647, 5096, 7479, and 8139.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

- CID 4646

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 485.31 | 36.3.13.5 | The name "Segment Parser" has created much confusion in 11ax and 11be | Insert "NOTE - The output of the operation described in this subclause is named a frequency subblock rather than a segment because the output is one or more frequency subblocks yet one frequency segment. A more precise name for this operation would be frequency subblock parser." | Revised Instructions to the editor Please make the changes as shown in 11/21-1515r4 |

## CID 4647

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 494.32 | 36.3.13.9 | The name "Segment De/parser" has created much confusion in 11ax and 11be | Insert "NOTE - The input of the operation described in this subclause is named a frequency subblock rather than a segment because the input is one or more frequency subblocks yet one frequency segment. A more precise name for this operation would be frequency subblock deparser." | Revised Instructions to the editor Please make the changes as shown in 11/21-1515r4 |

Discussion:

For details about the difference between ‘segment’ and ‘frequency subblock’, see document 11-21-0489-03-00be-cr-on-cid-1279.

Instructions to the editor:

Insert the following sentence as the first paragraph of Section 36.3.13.5

The segment parser operation is applied to each 80MHz frequency subblock.

Insert the following sentence as the first paragraph of Section 36.3.13.9

The Segment deparser combines 80MHz frequency sublocks from the output of the LDPC tone mapper into a single frequency segment.

## CID 5096

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 494.52 | 36.3.13.9 | Name of frequency subblock is not consisent in the text | Change block to subblock | ACCEPTED |

Background:

## CID 7479

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 494.52 | 36.3.13.9 | What is the definition of "N_{SD_q}" ? Add a definition for this variable. | As in comment. | REJECTED The notations NSD_q and NSD_l have the same defination, which indicate the number of data tones in the q-th and l-th frequency subblocks. The definition has been pointed out in Line 18 and 55, Page 493 of D1.0. As to their usage, the difference lies in the ranges of q and l, where l ∈[0, L-1] denotes one of all the frequency subblocks and q∈[0, l] denotes one of the first to the l+1-th frequency subblocks. |

## CID 8139

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 494.32 | 36.3.13.9 | make space between "242-," and "484-" | as in comment | REVISED Instructions to the editor Please make the changes as shown in 11/21-1515r4 |

Instructions to the editor:

Please insert a space between ‘484+242-,’ and ‘and’ in Line 32 Page 593 of TGbe Draft D1.2.