IEEE P802.11
Wireless LANs

| CC36 CR on EHT Operation element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.01.24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guogang Huang | Huawei Technologies | F3-6-A124, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | huangguogang1@huawei.com |
| Yunbo Li |  |  |  |  |
| Ming Gan |  |  |  |  |
| Yuchen Guo |  |  |  |  |
| Yiqing Li |  |  |  |  |
| Yuxin Lu |  |  |  |  |
| Mengyao Ma |  |  |  |  |
| Stephen MaCann |  |  |  |  |

Abstract

This submission proposes resolutions for the following CIDs:

4261, 4262, 4263, 5124, 5125, 6012, 6234, 6602, 6603, 6702, 6703, 8278, 8279

Revisions:

- Rev 0: Initial version of the document.

| CID | Commenter | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 4261 | Alfred Asterjadhi | 126.58 | 9.4.2.295a | Length of the EHT Operation element is missing. Add the length. | As in comment | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4261) in 11/21-1533r1 |
| 5124 | Geonjung Ko | 127.10 | 9.4.2.295a | Need to specify the size of each subfield. | As in comment | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4261) in 11/21-1533r1 |
| 6702 | Rojan Chitrakar | 126.60 |  | Size of the EHT Operation Information field is missing. | Specify the size of the EHT Operation Information field. | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4261) in 11/21-1533r1 |
| 8278 | Zhiqiang Han | 126.57 |  | How long is the EHT Operation Information field? Please clarify it. | as in comment. | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4261) in 11/21-1533r1 |
| 4263 | Alfred Asterjadhi | 127.26 | 9.4.2.295a | The encoding and the length for the CCFS is missing. Please add it. | As in comment | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4263) in 11/21-1533r1 |
| 6234 | Ming Gan | 127.12 |  | Please specify the number of CCFS such that it is aligned with that in 802.11ax | as in the comment | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4263) in 11/21-1533r1 |
| 6602 | Po-Kai Huang | 127.26 |  | The description of the CCFS field is missing. | Add the description for either one CCFS field or two CCFS fields to align with the baseline. | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4263) in 11/21-1533r1 |
| 6703 | Rojan Chitrakar | 127.23 |  | The encoding information for CCFS is missing | Specify the encoding information for CCFS. | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#4263) in 11/21-1533r1 |
| 8279 | Zhiqiang Han | 127.15 |  | How long is the Channel Width? 3bits? | as in comment. | Revised Agree in principle with the commenter. TGbe editor to make the changes with the CID tag (#8279) in 11/21-1533r1 |
| 6012 | Liwen Chu | 127.4 |  | 5GHz band also needs channel configuration information given the static channel puncture is allowed in 5GHz band. | As in comment | Revised Agree with the commenter in principle. TGbe editor to make the changes with the CID tag (#6012) in 11/21-1533r1 |
| 6603 | Po-Kai Huang | 127.26 |  | Channel Width and CCFS field are not needed for 2.4 and 5 GHz band because there is no need for additional information that conflicts with the baseline indication. | "Create a control field that indicates BSS configuration information field exist or not and the BSS configuration information includes CCFS and channel width field. Put the Disabled Subchannel Bitmap Present bit in the control field as well." | Revised Agree in principle with the commenter. The static channel puncture is also allowed in 5GHz band TGbe editor to make the changes with the CID tag (#6012) in 11/21-1533r1 |
| 4262 | Alfred Asterjadhi | 127.5 | 9.4.2.295a | If this is the case then this field needs to be called EHT 6G Operation Information field. | As in comment | Rejected Considering the static channel puncture has been allowed in 5GHz band, there is no need to change it. |
| 5125 | Geonjung Ko | 127.28 | 9.4.2.295a | Change "BBS" to "BSS" | As in comment | Revised Agree in principle with the commenter. Since this table is deleted, no change is needed. |

---------------------------------------------Option 1: one CCFS subfield -----------------------------------------------------------

TGbe editor: Modify 9.4.2.311 EHT Operation element as follows:

9.4.2.311 EHT Operation element

The operation of EHT STAs in an EHT BSS is controlled by the following:

- The HT Operation element, HE Operation element, and EHT Operation element if operating in the

2.4 GHz band

- The HT Operation element, VHT Operation element (if present), HE Operation element, and EHT Operation element if operating in the 5 GHz band

- The HE Operation element and EHT Operation element if operating in the 6 GHz band

The format of the EHT Operation element is shown in Figure 9-1002a (EHT Operation element format).

|  | Element ID | Length | Element ID Extension | EHT Operation Information | Disabled Subchannel Bitmap |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 0 or 2 |

Figure 9-1002a – EHT Operation element format (#1086)(#1667)(#2148)(#2147)(#4261)

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The EHT STA obtains the channel configuration information from the EHT Operation element if operating in the (#6012)5 or 6 GHz band. The subfields of EHT Operation Information field are defined in  Figure 9-xxxb (EHT Operation Information fieldformat(#1086)(#1667)(#2148)(#2147)).

| Channel Width |  |  |
| --- | --- | --- |
| Disabled Subchannel Bitmap Present |  |  |

|  | Control | CCFS |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure 9-xxxb – EHT Operation Information field format(#6603)

The Control subfield is defined in Figure 9-xxxc (Control subfield).

|  | B0 B2 | B3 B6 | B7 |
| --- | --- | --- | --- |
|  | Channel Width | Reserved | Disabled Subchannel Bitmap Present |
| Bits: | 3 | 4 | 1 |

Figure 9-xxxc – Control subfield format(#6603)(#8279)

The Channel Width subfield indicates the EHT BSS channel width and is set to 0 for 20 MHz, 1 for 40 MHz, 2 for 80 MHz, 3 for 160 MHz and 4 for 320 MHz. Values in the range 5 to 7 are reserved.

The Disabled Subchannel Bitmap Present subfield is set to 1 if the Disabled Subchannel Bitmap field is present and set to 0 otherwise.

The CCFS subfield indicates the channel center frequency index for the 20, 40, 80, 160, or 320 MHz channel on which the EHT BSS operates.(#4263)

(#1086)(#1667)(#2148)(#2147)The Disabled Subchannel Bitmap field is present if the Disabled Subchannel Bitmap Present subfield is equal to 1 and provides a list of subchannels that are punctured within the BSS bandwidth; otherwise it is not present.

The Disabled Subchannel Bitmap field is a 16-bit bitmap where the lowest numbered bit corresponds to the 20 MHz subchannel that lies within the BSS bandwidth and that has the lowest frequency of the set of all 20 MHz subchannels within the BSS bandwidth. Each successive bit in the bitmap corresponds to the next higher frequency 20 MHz subchannel. A bit in the bitmap is set to 1 to indicate that the corresponding 20 MHz subchannel is punctured and is set to 0 to indicate that the corresponding 20 MHz subchannel is not punctured.

---------------------------------------------Option 2: two CCFS subfields-----------------------------------------------------------

TGbe editor: Modify 9.4.2.311 EHT Operation element as follows:

9.4.2.311 EHT Operation element

The operation of EHT STAs in an EHT BSS is controlled by the following:

- The HT Operation element, HE Operation element, and EHT Operation element if operating in the

2.4 GHz band

- The HT Operation element, VHT Operation element (if present), HE Operation element, and EHT Operation element if operating in the 5 GHz band

- The HE Operation element and EHT Operation element if operating in the 6 GHz band

The format of the EHT Operation element is shown in Figure 9-1002a (EHT Operation element format).

|  | Element ID | Length | Element ID Extension | EHT Operation Information | Disabled Subchannel Bitmap |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 3 | 0 or 2 |

Figure 9-1002a – EHT Operation element format (#1086)(#1667)(#2148)(#2147)(#4261)

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The EHT STA obtains the channel configuration information from the EHT Operation element if operating in the (#6012)5 or 6 GHz band. The subfields of EHT Operation Information field are defined in  Figure 9-xxxb (EHT Operation Information fieldformat(#1086)(#1667)(#2148)(#2147)).

| Channel Width |  |  |
| --- | --- | --- |
| Disabled Subchannel Bitmap Present |  |  |

|  | Control | CCFS0 | CCFS1 |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 9-xxxb – EHT Operation Information field format(#6603)

The Control subfield is defined in Figure 9-xxxc (Control subfield).

|  | B0 B2 | B3 B6 | B7 |
| --- | --- | --- | --- |
|  | Channel Width | Reserved | Disabled Subchannel Bitmap Present |
| Bits: | 3 | 4 | 1 |

Figure 9-xxxc – Control subfield format(#6603)(#8279)

The Channel Width subfield indicates the EHT BSS channel width and is set to 0 for 20 MHz, 1 for 40 MHz, 2 for 80 MHz, 3 for 160 MHz and 4 for 320 MHz. Values in the range 5 to 7 are reserved.

The Disabled Subchannel Bitmap Present subfield is set to 1 if the Disabled Subchannel Bitmap field is present and set to 0 otherwise.

The CCFS0 subfield indicates the channel center frequency index for the 20, 40, 80, 160, or 320 MHz channel on which the EHT BSS operates. If the EHT BSS channel width is 160 MHz, the CCFS0 subfield indicates the channel center frequency index of the primary 80 MHz channel. If the EHT BSS channel width is 320 MHz, the CCFS0 subfield indicates the channel center frequency index of the primary 160 MHz channel.(#4263)

The CCFS1 subfield indicates the channel center frequency index for 160, or 320 MHz channel on which the EHT BSS operates. If the EHT BSS channel width is 160 MHz, the CCFS1 subfield indicates the channel center frequency index of the 160 MHz channel. If the EHT BSS channel width is 320 MHz, the CCFS1 subfield indicates the channel center frequency index of the 320 MHz channel.(#4263)

(#1086)(#1667)(#2148)(#2147)The Disabled Subchannel Bitmap field is present if the Disabled Subchannel Bitmap Present subfield is equal to 1 and provides a list of subchannels that are punctured within the BSS bandwidth; otherwise it is not present.

The Disabled Subchannel Bitmap field is a 16-bit bitmap where the lowest numbered bit corresponds to the 20 MHz subchannel that lies within the BSS bandwidth and that has the lowest frequency of the set of all 20 MHz subchannels within the BSS bandwidth. Each successive bit in the bitmap corresponds to the next higher frequency 20 MHz subchannel. A bit in the bitmap is set to 1 to indicate that the corresponding 20 MHz subchannel is punctured and is set to 0 to indicate that the corresponding 20 MHz subchannel is not punctured.