IEEE P802.11
Wireless LANs

| CR for Clause 36.3.2.2 Subcarriers and resource allocation for multiple RUs - part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.11.19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Myeongjin Kim | Samsung |  |  | mj1108.kim@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC36 on P802.11be D1.0

Baseline documents: TGbe D1.2.

R0:

- The following 24 comments under “36.3.2.2 Subcarriers and resource allocation for multiple RUs” are resolved:

- CID 4983, 6426, 4984, 4985, 5405, 5464, 5524, 5811, 6424, 6425, 6427, 6783, 6784, 7143, 7134, 7135, 7136, 7137, 7141, 7746, 8092, 7138, 7139, 7140

R1:

- Fixed the typos

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 4983, 6426

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4983 | 36.3.2.2.2.2 | 347.06 | In Figure 36-7, 52+26-tone MRU indices are shown. It seems that MRU indices are always 2 to 11 in all 80 MHz subblocks in 80/160/320 MHz but in 160/320 MHz other MRU indices are also defined. | Add other MRU indices into the figure or add a sentence, for example, "For 160 MHz and 320 MHz EHT PPDU, MRU indices are different in each 80 MHz subblock.". | Revised. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |
| 6426 | 36.3.2.2 | 347.01 | Add a dash (-) between 52+26 and tone | As in comment | Revised. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |

Proposed Changes:

Modify P447 L1-L3 of D1.2 as follows: (The baseline for this text is 802.11be D1.2.)

(#3272)The allowed 52+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU are shown in Figure 36-7 (Allowed 52+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU(#1279)(#3270)). For 160 MHz and 320 MHz EHT PPDU, MRU indices are different in each 80 MHz subblock and defined in Table 36-11 (Indices for small size MRUs in an OFDMA 160 MHz EHT PPDU) and Table 36-12 (Indices for small size MRUs in an OFDMA 320 MHz EHT PPDU).

## CID 4984

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4984 | 36.3.2.2.2 | 348.07 | In Figure 36-10, 106+26-tone MRU indices are shown. It seems that MRU indices are always 1 to 8 in all 80 MHz subblocks in 80/160/320 MHz but in 160/320 MHz other MRU indices are also defined. | Add other MRU indices into the figure or add a sentence, for example, "For 160 MHz and 320 MHz EHT PPDU, MRU indices are different in each 80 MHz subblock.". | Revised. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |

Proposed Changes:

Modify P448 L1-L3 of D1.2 as follows: (The baseline for this text is 802.11be D1.2.)

(#3272)The allowed 106+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU are shown in Figure 36-10 (Allowed 106+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU(#1279)(#3270)). For 160 MHz and 320 MHz EHT PPDU, MRU indices are different in each 80 MHz subblock and defined in Table 36-11 (Indices for small size MRUs in an OFDMA 160 MHz EHT PPDU) and Table 36-12 (Indices for small size MRUs in an OFDMA 320 MHz EHT PPDU).

## CID 4985

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4985 | 36.3.2.2.2 | 355.15 | For a 20MHz operating non-AP STA, only some 52+26-tone MRUs are mandatory. | Add "except for a 20 MHz operating non-AP STA". | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P455 L15. |

## CID 5405, 5464, 5524, 5811, 6424

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5405 | 36.3.2.2.1 | 345.43 | 4x996-tone RU is not one of the RU components in an MRU. | Remove 4x996-tone RU in this sentence. | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P445 L43. |
| 5464 | 36.3.2.2.1 | 345.43 | An MRU cannot consist of multiple RUs of 4*996-tone RU | Change ",2x996-tone RU, and 4x996-tone RU" to "and 2x996-tone RU" | Accepted Already accepted by CID 5405. Note to the editor: No further changes are needed. |
| 5524 | 36.3.2.2.1 | 345.43 | 4x996-tone RU is not used by composing multiple RUs for an MRU. | Modify the sentence as "An MRU consists of multiple RUs of 26-tone RU, 52-tone RU, 106-tone RU, 242-tone RU, 484-tone RU, 996-tone RU, and 2x996-tone RU." | Accepted Already accepted by CID 5405. Note to the editor: No further changes are needed. |
| 5811 | 36.3.2.2.1 | 345.43 | 4x996-tone RU is not a part of any MRU. | change "An MRU consists of multiple RUs of 26-tone RU, 52-tone RU, 106-tone RU, 242-tone RU, 484-tone RU, 996-tone RU, 2x996-tone RU, and 4x996-tone RU." to "An MRU consists of multiple RUs of 26-tone RU, 52-tone RU, 106-tone RU, 242-tone RU, 484-tone RU, 996-tone RU, and 2x996-tone RU." | Accepted Already accepted by CID 5405. Note to the editor: No further changes are needed. |
| 6424 | 36.3.2.2 | 345.43 | 4x996-tone RU is not used to make up a MRU. | Delete the "4x996-tone RU" | Accepted Already accepted by CID 5405. Note to the editor: No further changes are needed. |

## CID 6425

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6425 | 36.3.2.2 | 346.38 | Add a dash (-) between 52+26 and tone | As in comment | Revised. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |

Proposed Changes:

Modify P446 L38-L39 of D1.2 as follows: (The baseline for this text is 802.11be D1.2.)

(#3272)The allowed 52+26-tone MRUs in an OFDMA 40 MHz EHT PPDU are shown in Figure 36-6 (Allowed 52+26-tone MRUs in an OFDMA 40 MHz EHT PPDU(#3270)).

## CID 6427

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6427 | 36.3.2.2 | 349.03 | "non-OFDMA 20 MHz HE PPDU" should be "non-OFDMA 40 MHz HE PPDU". | As in comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P449 L3. |

## CID 6783, 6784, 7143

,

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6783 | 36.3.2.2.2 | 353.35 | In Table 36-12, 52+26 tone MRU 27 should be "52-tone RU 35 + 26-tone RU 82" | As in comment | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |
| 6784 | 36.3.2.2.2 | 353.62 | In Table 36-12, 52+26 tone MRUs 42 and 43 should be undefined. | As in comment | Revised. Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |
| 7143 | 36.3.2.2.2 | 353.62 | For consistency with other 80 MHz subbblocks, it looks like MRU 42 and MRU 43 should be "Not defined". | Change to "Not defined" | Revised Already revised by CID 6784. Note to the editor: No further changes are needed. |

Proposed Changes:

Modify Table 36-12 as follows: (The baseline for this text is 802.11be D1.2.)

| MRU type | MRU index | MRU combination |
| --- | --- | --- |
| 52+26-tone MRU | MRU 11 | 52-tone RU 14 + 26-tone RU 33 |
|  | MRU 12 | Not defined |
|  | MRU 13 | Not defined |
|  | MRU 14 | 52-tone RU 18 + 26-tone RU 42 |
|  | MRU 15 | 52-tone RU 19 + 26-tone RU 45 |
|  | MRU 16 | 52-tone RU 22 + 26-tone RU 48 |
|  | MRU 17 | 52-tone RU 22 + 26-tone RU 51 |
|  | MRU 18 | Not defined |
|  | MRU 19 | Not defined |
|  | MRU 20 | 52-tone RU 26 + 26-tone RU 61 |
|  | MRU 21 | 52-tone RU 27 + 26-tone RU 64 |
|  | MRU 22 | 52-tone RU 30 + 26-tone RU 67 |
|  | MRU 23 | 52-tone RU 30 + 26-tone RU 70 |
|  | MRU 24 | Not defined |
|  | MRU 25 | Not defined |
|  | MRU 26 | 52-tone RU 34 + 26-tone RU 79 |
|  | MRU 27 | 52-tone RU 35 + 26-tone RU 82 |
|  | MRU 28 | 52-tone RU 38 + 26-tone RU 85 |
|  | MRU 29 | 52-tone RU 38 + 26-tone RU 88 |
|  | MRU 30 | Not defined |
|  | MRU 31 | Not defined |
|  | MRU 32 | 52-tone RU 42 + 26-tone RU 98 |
|  | MRU 33 | 52-tone RU 43 + 26-tone RU 101 |
|  | MRU 34 | 52-tone RU 46 + 26-tone RU 104 |
|  | MRU 35 | 52-tone RU 46 + 26-tone RU 107 |
|  | MRU 36 | Not defined |
|  | MRU 37 | Not defined |
|  | MRU 38 | 52-tone RU 50 + 26-tone RU 116 |
|  | MRU 39 | 52-tone RU 51 + 26-tone RU 119 |
|  | MRU 40 | 52-tone RU 54 + 26-tone RU 122 |
|  | MRU 41 | 52-tone RU 54 + 26-tone RU 125 |
|  | MRU 42 | Not defined |
|  | MRU 43 | Not defined |
|  | MRU 44 | 52-tone RU 58 + 26-tone RU 135 |
|  | MRU 45 | 52-tone RU 59 + 26-tone RU 138 |
|  | MRU 46 | 52-tone RU 62 + 26-tone RU 141 |
|  | MRU 47 | 52-tone RU 62 + 26-tone RU 144 |
|  | MRU 48 | Not defined |
| 106+26-tone MRU | MRU 1 | 106-tone RU 1 + 26-tone RU 5 |
|  | MRU 2 | Not defined |
|  | MRU 3 | Not defined |
|  | MRU 4 | 106-tone RU 4 + 26-tone RU 14 |
|  | MRU 5 | 106-tone RU 5 + 26-tone RU 24 |
|  | MRU 6 | Not defined |
|  | MRU 7 | Not defined |
|  | MRU 8 | 106-tone RU 8 + 26-tone RU 33 |
|  | MRU 9 | 106-tone RU 9 + 26-tone RU 42 |
|  | MRU 10 | Not defined |
|  | MRU 11 | Not defined |
|  | MRU 12 | 106-tone RU 12 + 26-tone RU 51 |
|  | MRU 13 | 106-tone RU 13 + 26-tone RU 61 |
|  | MRU 14 | Not defined |
|  | MRU 15 | Not defined |
|  | MRU 16 | 106-tone RU 16 + 26-tone RU 70 |
|  | MRU 17 | 106-tone RU 17 + 26-tone RU 79 |
|  | MRU 18 | Not defined |
|  | MRU 19 | Not defined |
|  | MRU 20 | 106-tone RU 20 + 26-tone RU 88 |
|  | MRU 21 | 106-tone RU 21 + 26-tone RU 98 |
|  | MRU 22 | Not defined |
|  | MRU 23 | Not defined |
|  | MRU 24 | 106-tone RU 24 + 26-tone RU 107 |
|  | MRU 25 | 106-tone RU 25 + 26-tone RU 116 |
|  | MRU 26 | Not defined |
|  | MRU 27 | Not defined |
|  | MRU 28 | 106-tone RU 28 + 26-tone RU 125 |
|  | MRU 29 | 106-tone RU 29 + 26-tone RU 135 |
|  | MRU 30 | Not defined |
|  | MRU 31 | Not defined |
|  | MRU 32 | 106-tone RU 32 + 26-tone RU 144 |

## CID 7134, 7135, 7136, 7137, 7141, 7746

,

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 7134 | 36.3.2.2.1 | 345.42 | "An MRU consists of multiple RUs". This is too generic. | Replace with "An MRU consists of selected combinations of " | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P445 L41. |
| 7135 | 36.3.2.2.2 | 346.02 | For clarity: Change "within a 20 MHz channel" to "within the same 20 MHz channel". | See comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P446 L2. |
| 7136 | 36.3.2.2.2 | 346.02 | For clarity: Change "consist of the data subcarriers of" to "consist of the union of the data subcarriers of". Make same change in other places where the same language is used. | See comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P446 L2-L3. |
| 7137 | 36.3.2.2.2 | 346.09 | For clarity: Change "within a 20 MHz channel" to "within the same 20 MHz channel". | See comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P446 L9. |
| 7141 | 36.3.2.2.2 | 348.23 | After "Table 36-12", add "for 20, 40, 80, 160 and 320 MHz respectively". (Note that Table captions in references are not included in the published version) | See comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P448 L23. |
| 7746 | 36.3.2.2.1 | 345.47 | Rephrase "RUs that are the same size or larger than ..." as "RUs that are the same size as or larger than ..." | as in comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.2 are P445 L48. |

## CID 8092

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 8092 | 36.3.2.2.2 | 346.34 | add the meaning or descirption of the dotted line in Figure 36-5, 36-6, 36-7, Figure 36-8, 36-9 and 36-10 somewhere. | as in comment | Revised. Agree that the dotted line in figures may cause a confusion. So, the figures are revised. Instructions to the editor: Please make the changes as shown in 11/21-1600r1 |

Proposed Changes:

Modify Figure 36-5, 36-6, 36-7, Figure 36-8, 36-9 and 36-10 as follows: (The baseline for this text is 802.11be D1.2.)

Figure 36-5—Allowed 52+26-tone MRUs in an OFDMA 20 MHz EHT PPDU

Figure 36-6—Allowed 52+26-tone MRUs in an OFDMA 40 MHz EHT PPDU

Figure 36-7—Allowed 52+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU

Figure 36-8—Allowed 106+26-tone MRUs in an OFDMA 20 MHz EHT PPDU

Figure 36-9—Allowed 106+26-tone MRUs in an OFDMA 40 MHz EHT PPDU

Figure 36-10—Allowed 106+26-tone MRUs in each 80 MHz frequency subblock of an OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU

## CID 7138, 7139, 7140

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 7138 | 36.3.2.2.2 | 346.15 | Is it "OFDMA 20 MHz EHT PPDU" or "20 MHz OFDMA EHT PPDU". The latter looks more sensible. | Change "OFDMA 20 MHz EHT PPDU" to "20 MHz OFDMA EHT PPDU" (all occurences) | Rejected. We follow the convention used in IEEE 802.11ax. |
| 7139 | 36.3.2.2.2 | 346.38 | Is it "OFDMA 40 MHz EHT PPDU" or "40 MHz OFDMA EHT PPDU". The latter looks more sensible. | Change "OFDMA 40 MHz EHT PPDU" to "40 MHz OFDMA EHT PPDU" (all occurences) | Rejected. We follow the convention used in IEEE 802.11ax. |
| 7140 | 36.3.2.2.2 | 347.01 | Change "OFDMA 80 MHz, 160 MHz, or 320 MHz EHT PPDU" to "80 MHz, 160 MHz, or 320 MHz OFDMA EHT PPDU" (all occurences) | See comment | Rejected. We follow the convention used in IEEE 802.11ax. |

------------- Visio files for the figures--------------------