IEEE P802.11
Wireless LANs

| CC36 CR on CID 4296 ESS Report Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.01.25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guogang Huang | Huawei Technologies | F3-6-A124, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | huangguogang1@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0.

CID 4296 is resolved.

| CID | Commenter | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 4296 | Alfred Asterjadhi |  | 9.4.2.256 | Any updates to ESS Report element for 11be? Align with concept of MLD APs for example (references relative to TGax D8.0) | As in comment | REVISED Agreed in principle. Instructions to the editor: Please make the changes to the spec as shown in 11/21-xxxxr0 |

Discussion

This CR document is prepared based on the assumption that all the APs affiliated with an AP MLD shall belong to the same ESS.

TGbe editor: Modify the following subclause as follows:

- ESS Report element(11ax)

The format of the ESS Report element is shown in Error: Reference source not found.

|  | Element ID | Length | Element ID Extension | ESS Information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 |
| - ESS Report element format(11ax) |  |  |  |  |

The Element ID, Length and Element ID Extension fields are defined in 9.4.2.1 (General).

The format of the ESS Information field is defined in Error: Reference source not found.

|  | Planned ESS | Edge Of ESS | Recommended BSS Transition RSSI Threshold Within ESS | Planned ESS For MLDs | Edge Of ESS For MLDs | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: | 1 | 1 | 6 | 1 | 1 | 6 |
| - ESS Information field format(11ax) |  |  |  |  |  |  |

The Planned ESS subfield indicates whether the BSS is part of an ESS that is planned with several BSSs in an overlapping configuration. This subfield is set to 1 to indicate that the ESS is deployed to ensure blanket coverage over the Extended Service Area (ESA). Otherwise, this subfield is set to 0 and the Edge Of ESS and Recommended BSS Transition RSSI Threshold Within ESS subfields are reserved.

The Edge Of ESS subfield indicates whether the BSS is at the edge of the ESS. This subfield is set to 1 to indicate the BSS is at the edge of the ESS. Otherwise, this subfield is set to 0.

The Recommended BSS Transition RSSI Threshold Within ESS subfield indicates the RSSI below which an associated STA is recommended to initiate BSS transition to a neighbor BSS belonging to the ESS.

The resolution for the Recommended BSS Transition RSSI Threshold Within ESS subfield is 1 dB. The encoding is defined in Error: Reference source not found.

| - Recommended BSS Transition RSSI Threshold Within ESS subfield encoding(11ax)input.doc |  |
| --- | --- |
| Value | Description |
| 0–62 | –100 dBm to –38 dBm |
| 63 | No recommendation |

The Planned ESS For MLDs subfield indicates whether the AP MLD is part of an ESS that is planned with several BSSs or AP MLDs in an overlapping configuration.

The Edge Of ESS For MLDs subfield indicates whether the AP MLD is at the edge of the ESS. This subfield is set to 1 to indicate the AP MLD is at the edge of the ESS. Otherwise this subfield is set to 0.

The use of the ESS Report element is described in 11.21.7.5 (Planned ESS(11ax)) and 35.3.24.1 (Planned ESS for MLDs).

TGbe editor:Add the following subclause as follows:

35.3.24.1 Planned ESS for MLDs

The AP affiliated with an AP MLD may transmit an ESS Report element (see 9.4.2.256 (ESS Report element)) to indicate whether or not the corresponding AP MLD is in a planned ESS, to assist associated STAs and non-AP MLDs' roaming.

If the AP affiliated with the AP MLD transmits an ESS Report element, the AP affiliated with the AP MLD shall set the Planned ESS subfield in the ESS Information field to 1. And AP affiliated with the AP MLD shall set the Planned ESS For MLDs subfield in the ESS Information field to 1 if it is part of an ESS that is planned with several BSSs or AP MLD in overlapping configuration, whereby an associated non-AP MLD may adjust its BSS transition algorithms accordingly. Otherwise, it shall set the Planned ESS For MLDs subfield to 0.

If the Planned ESS For MLDs subfield is 1, then the AP affiliated with the AP MLD shall set the Edge Of ESS For MLDs subfield in the ESS Information field of the ESS Report element to 1 if the associated AP MLD is at the edge of an ESS. Otherwise, it shall set the Edge Of ESS For MLDs subfield to 0.

For a non-AP MLD, if the beacon RSSI of a link is below the Recommended BSS Transition RSSI Threshold Within ESS subfield in the ESS Information field of the ESS Report element, then it should not use that link for transmission. If the beacon RSSIs of all setup links are below the Recommended BSS Transition RSSI Threshold Within ESS subfield in the ESS Information field of the ESS Report element, respectively, then it should initiate a BSS transition.

The value of the Edge Of ESS For MLDs subfield may be changed by the AP MLD if conditions in the ESS change. An AP MLD shall not change the value of the Planned ESS For MLDs subfield over the lifetime of the AP MLD.