IEEE P802.11
Wireless LANs

| CC36 Comment Resolutions for CID 4985 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.12.13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Myeongjin Kim | Samsung |  |  | mj1108.kim@samsung.com |

Abstract

This submission proposes a resolution for CID 4985 from the CC36 on P802.11be D1.0

Baseline documents: TGbe D1.3.

R0:

- Initial version of the document.

## CID 4985

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4985 | 36.3.2.2.2 | 355.15 | For a 20MHz operating non-AP STA, only some 52+26-tone MRUs are mandatory. | Add "except for a 20 MHz operating non-AP STA". | Revised. Instructions to the editor: Please make the changes as shown in 11/21-2003r0 |

Proposed Changes:

Modify P477 L15-L16 of D1.3 as follows: (The baseline for this text is 802.11be D1.3.)

It is mandatory for a non-AP STA to support the transmission and reception of 52+26-tone and 106+26-tone MRUs in an OFDMA EHT PPDU(#1293).

NOTE — For a 20 MHz operating non-AP EHT STA, the transmission and reception of 52+26-tone and 106+26-tone MRUs which are allowed in 36.3.2.6 (RU and MRU restrictions for 20 MHz operation) shall be supported.