IEEE P802.11
Wireless LANs

| CC36 CR on CID 4296 ESS Report Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.01.25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guogang Huang | Huawei Technologies | F3-6-A124, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | huangguogang1@huawei.com |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.5.

The following CIDs are resolved.

4262, 6012, 4284, 4309, 4310

| CID | Commenter | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 4262 | Alfred Asterjadhi | 127.5 | 9.4.2.295a | If this is the case then this field needs to be called EHT 6G Operation Information field. | As in comment | Reject The EHT Operation Information field can be also present in 5 GHz band. |
| 6012 | Liwen Chu | 127.4 | 9.4.2.295a | 5GHz band also needs channel configuration information given the static channel puncture is allowed in 5GHz band. | As in comment | Revised Agree with the commenter. The related text has been removed by doc. 11/21-1533r5. No further actions are needed for this CID. |
| 4284 | Alfred Asterjadhi |  | 35.9 | Normative behaviors for EHT BSS Operation are generally missing (add related behaviors inline with what was done in 11ax (wherein the EHT Operation element is providing the necessary signaling. | As in comment | Revised Agree with the commenter. This whole subclause has been rewritten in draft 1.5. No further actions are needed for this CID. |
| 4309 | Alfred Asterjadhi |  | 35.9.1 | Need to add references to the 11ax subclauses regarding how to set the fields of the HE Operation element and so on (VHT etc). We also need to specify what fields are there in the EHT Operation eleement and once that is done specify here of the expected behavior from the STA transmitting and receiving these fields. | As in comment | Revised Agree with the commenter. This whole subclause has been rewritten in draft 1.5. No further actions are needed for this CID. |
| 4310 | Alfred Asterjadhi |  | 35.9.1 | Disallowed, inactive, punctured... different terminolgies. Suggest chosing one and use throughout when referring to the same functionality. | As in comment | Revised Agree with the commenter. To align with the subclause 35.16 EHT BSS operation, the word ‘disabled’ is replaced with ‘punctured’ in subclause 9.4.2.311. Instructions to the editor: Please make the changes with the CID tag (#4310) to the spec as shown in 11/22-0545r0 |

9.4.2.311 EHT Operation element

TGbe editor: Modify the following figure as follows:

|  | B0 | B1 | B2 B7 |
| --- | --- | --- | --- |
|  | EHT Operation Information Present | Punctured Subchannel Bitmap Present | Reserved |
| Bits: | 1 | 1 | 6 |

Figure 9-1002b—EHT Operation Parameters field format(#6603)(#4310)

TGbe editor: Modify the following paragraph as follows:

(#4310) (#6603) The Punctured Subchannel Bitmap Present subfield is set to 1 if the Punctured Subchannel Bitmap field is present and set to 0 otherwise. The Punctured Subchannel Bitmap Present subfield is valid only when the HT Operation Information Present subfield is set to 1.

TGbe editor: Modify the following figure as follows:

| Control | CCFS0 | CCFS1 | Punctured Subchannel Bitmap |
| --- | --- | --- | --- |

Octets:	1	1	1	0 or 2

Figure 9-1002c—EHT Operation Information field format(#6603)(#4310)

TGbe editor: Modify the following paragraphs as follows:

(#4310)(#1086)(#1667)(#2148)(#2147)(#6603)The Punctured Subchannel Bitmap subfield is present if the Punctured Subchannel Bitmap Present subfield is equal to 1 and provides a list of subchannels that are punctured within the BSS bandwidth; otherwise it is not present.

(#4310)(#6603)The Punctured Subchannel Bitmap subfield is a 16-bit bitmap where the lowest numbered bit corresponds to the 20 MHz subchannel that lies within the BSS bandwidth and that has the lowest frequency of the set of all 20 MHz subchannels within the BSS bandwidth. Each successive bit in the bitmap corresponds to the next higher frequency 20 MHz subchannel. A bit in the bitmap is set to 1 to indicate that the corresponding 20 MHz subchannel is punctured and is set to 0 to indicate that the corresponding 20 MHz subchannel is not punctured.