IEEE P802.11
Wireless LANs

| CR for CIDs 6144 and 7990 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.04.01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mengshi Hu | Huawei Technologies | F3-6-A118, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | humengshi@huawei.com |
| Ross Yu |  |  |  |  |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |
| Stephen McCann |  |  |  |  |

Abstract

This submission contains the proposed comment resolutions for the following CIDs related to the EHT Default PE Duration in D1.0.

CIDs 6144 and 7990

Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 6144

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 126.44 | 9.4.2.295a | 20 us Default PE Duration should be defined in EHT Operation element. (0, 4, 8, 12, 16 are indicated in the Default PE Duration subfield in HE Operation element). | 20 us Default PE Duration should be defined in EHT Operation element. | REJECTED Because there is no consensus on TRS Control subfield for EHT, the padding requirement for an EHT TB PPDU solicited by the TRS Control should not be considered at this time. |

## CID 7990

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 126.44 | 9.4.2.295a | HE Operation element has the "Default PE Duration" subfield to indicate the PE field duration for an HE TB PPDU that is solicited with a TRS Control subfield. EHT has one more PE value than HE, specifically the 20 usec PE. Hence, we need a "Default PE Duration" subfield in the EHT Operation element to support 20 usec PE. | Define and add a "Default PE Duration" subfield in the EHT Operation element. | REJECTED Because there is no consensus on TRS Control subfield for EHT, the padding requirement for an EHT TB PPDU solicited by the TRS Control should not be considered at this time. |

## Discussion (The following text is only for the discussion)

Three options for the resolutions for CIDs 6144 and 7990 are given here (The final resolution we choose is Option 1. When the TRS Control subfield for EHT is defined, the Option 2 and 3 can be used):

Option 1: REJECTED

Rejected at this time because there is no consensus on TRS Control for EHT.

Option 2: One-Bit Indication

Instructions to the editor (for CID 6144 and 7990)

Please make the changes to P186, L9 in the subclause EHT Operation element in D1.5 as shown below:

The format of the EHT Operation element is shown in Figure 9-1002a (EHT Operation element format(#4261)(#6603)(#1086)(#1667)(#2148)(#2147)).

| Element ID | Length | Element ID Extension | EHT Operation Parameters | EHT Operation Information |
| --- | --- | --- | --- | --- |

Octets:	1	1	1	1	0 or 3 or 5

Figure 9-1002a—EHT Operation element format (#4261)(#6603)(#1086)(#1667)(#2148)(#2147)

The Element ID, Length, and Element ID Extension fields are defined in 9.4.2.1 (General).

The EHT Operation Parameters field is defined in Figure 9-1002b (EHT Operation Parameters field for- mat(#6603)).

|  | B0 | B1 |  | B2 | B3 B7 |
| --- | --- | --- | --- | --- | --- |
|  | EHT Operation Information Present | Disabled Subchannel Bitmap Present |  | EHT Default PE Duration | Reserved |
| Bits: | 1 | 1 |  | 1 | 5 |

Figure 9-1002b—EHT Operation Parameters field format(#6603)

(#6603)The EHT Operation Information Present subfield is set to 1 if the EHT Operation Information field is present and set to 0 otherwise. The EHT Operation Information Present subfield is set to 1 if the channel width indicated in an HT Operation, VHT Operation, or HE Operation element that is present in the same Management frame is different from the Channel Width field indicated in the EHT Operation Information field.

(#6603)The Disabled Subchannel Bitmap Present subfield is set to 1 if the Disabled Subchannel Bitmap field is present and set to 0 otherwise. The Disabled Subchannel Bitmap Present subfield is valid only when the HT Operation Information Present subfield is set to 1.

The EHT Default PE Duration subfield is set to 1 to indicate that the PE field duration for an EHT TB PPDU solicited by a TRS Control subfield is 20 µs and set to 0 to indicate that the PE field duration is the same as that indicated in the HE Operation Parameters field in the HE Operation element.

Option 3: Three-Bit Inidication

Please make the changes to P186, L26 in the subclause EHT Operation element in D1.5 as shown below:

|  | B0 | B1 |  | B2 B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- |
|  | EHT Operation Information Present | Disabled Subchannel Bitmap Present |  | EHT Default PE Duration | Reserved |
| Bits: | 1 | 1 |  | 3 | 3 |

Figure 9-1002b—EHT Operation Parameters field format(#6603)

(#6603)The EHT Operation Information Present subfield is set to 1 if the EHT Operation Information field is present and set to 0 otherwise. The EHT Operation Information Present subfield is set to 1 if the channel width indicated in an HT Operation, VHT Operation, or HE Operation element that is present in the same Management frame is different from the Channel Width field indicated in the EHT Operation Information field.

(#6603)The Disabled Subchannel Bitmap Present subfield is set to 1 if the Disabled Subchannel Bitmap field is present and set to 0 otherwise. The Disabled Subchannel Bitmap Present subfield is valid only when the HT Operation Information Present subfield is set to 1.

The EHT Default PE Duration subfield indicates the PE field duration in units of 4 µs for an EHT TB PPDU that is solicited by a TRS Control subfield. Values 0-5 of the EHT Default PE Duration subfield indicates 0, 4, 8, 12, 16, 20 µs, respectively, and values 6-7 of the Default PE Duration subfield are reserved.

Discussion ends.

## Appendix (Default PE Duration in HE)