IEEE P802.11
Wireless LANs

| CR for Section 9.3.1.19 on P802.11be – part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 12.07.2022 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolution to comment on P802.11be D2.0.

5 comments under subclause 9.1.3.19 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Page and line corrected at CID 11896 |

## CID 11624

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 138.40 | 9.3.1.19 | Redundant to note that EHT NDP Announcment frame contains at most one STA Info Subfield per STA. It is directly derived from that EHT NDPA has same format as HE | Remove this line | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/22-1090r0, under CID 11896 |

## CID 11625

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 139.08 | 9.3.1.19 | Applicability column is not clear. Should those values applicable for ranging variant only, be reserved for other variants? Need to clarify that | Add clarification regarding these values with respect to VHT/HE/EHT variants | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/22-1090r0, under CID 11896 |

## CID 11626

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 139.24 | 9.3.1.19 | AID 2047 is not restricted for EHT, need to clarify the applicability meaning | Add clarification regarding these values with respect to EHT variant | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/22-1090r0, under CID 11896 |

## CID 11627

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 142.61 | 9.3.1.19 | Missing text, sentence is not clear | Change the text to "In an EHT NDP Announcement frame that has more than one STA Info subfield, the RA is a broadcast address and the following applies" | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/22-1090r0, under CID 11896 |

## CID 11896

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 143.08 | 9.3.1.19 | "In an EHT NDP Announcement frame with a single STA Info field, the RA is an individual address and the Nc index subfield is reserved." suggest moving this sentence to the paragraph that defines the RA field settings (toward the beginning of the paragraphs describing EHT NDPA settings). | As in comment. | REVISED Instructions to the editor: Please make the changes to the spec as shown in 11/22-1090r0, under CID 11896 |

Instructions to the editor: please change the text on page 138, L40 as following:

An  NDP Announcement frame contains at most one STA Info field per STA.

Instructions to the editor: please replace the Table 9-42b on PP138-139 with following table:

Table 9-42b—AID11 subfield encoding in an NDP Announcement frame

| AID subfield | Description | NDP Announcment frame variant applicability |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VHT | HE | EHT | Ranging |
| 0 | STA Info field is addressed to the associated AP or mesh | Applicable |  |  |  |
| 1-2006 | STA Info field is addressed to an associated STA whose AID is equal to the value in the AID11 subfield if the NDP Announcement frame is not a Ranging variant. STA Info field is addressed to an unassociated STA or an associated STA whose RSID/AID is equal to the value in the RSID11/AID11 subfield if the NDP Announcement frame is a Ranging variant | Applicable |  |  |  |
| 2007 |  | Applicable | Applicable | Reserved | Applicable |
| 2008-2042 | N/A | Reserved |  |  |  |
| 2043 | STA Info field contains a sequence authentication code | Reserved | Reserved | Reserved | Applicable |
| 2044 | STA Info field contains a partial TSF | Reserved | Reserved | Reserved | Applicable |
| 2045 | STA Info field contains ranging measurement parameters | Reserved | Reserved | Reserved | Applicable |
| 2046 | N/A | Reserved |  |  |  |
| 2047 | STA Info field contains a disallowed subchannel bitmap | Reserved | Applicable | Reserved | Reserved |

Discussion for CIDs 11627, 11896:

The rules for setting the RA address in different NDP Announcement frame variants are discussed at the beginning of subclause 9.3.1.19. In particular, RA address for the case of single STA Info subfield is defined in REVme/D1.0.

The text on P142 LL60-65 and P143 LL1-9 provides the rules for Nc index values.

Mentioning the rules for RA address in this part is redundant.

Discussion ends here

Instructions to the editor: please change the text on page 142, L61 as following:

In an EHT NDP Announcement frame that has more than one STA Info subfield, the following applies:

Instructions to the editor: please change the text on page 143, LL8-9 as following:

In an EHT NDP Announcement frame with a single STA Info subfield the Nc index subfield is reserved.