IEEE P802.11
Wireless LANs

| LB266 - CR for P802.11be D2.0 Section 36.3.11.12 – Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.07.12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following CIDs on P802.11be D2.1.1:

11220, 11696, 11697, 11974, 12206, 12849

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Updated page numbers to fit D2.1.1 |

## CID 11974

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.3 | 36.3.12.11 | Puncturing of a subchannel smaller than 484 is not allowed in 320 MHz when the transmission is non-OFDMA | Replace the text "is not allowed in a 320 MHz PPDU bandwidth" with "is not allowed in a non-OFDMA transmission when the PPDU bandwidth is 320 MHz." | Accepted Instruction to the Editor: Please apply the resolution of CID 11974 and refer to the text in 719.3 in D2.1.1 Same resolution for the following CIDs: 11220, 11974, 12206 |

## CID 11220

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.3 | 36.3.12.11.1 | Puncturing smaller than 484 RU in a 320MHz PPDU is allowed for OFDMA transmission | Change sentence to: "In other words, puncturing a subchannel smaller than a 484-tone RU is not allowed in a 320 MHz PPDU bandwidth, when using non-OFDMA transmission." | REVISED Note to the Editor: Please apply the resolution of CID 11974 and refer to the text in 719.3 in D2.1.1 Same resolution for the following CIDs: 11220, 11974, 12206 |

## CID 12206

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.2 | 36.3.12.11.1 | The two last sentences in this paragraph, from 'The preamble puncturing' to 'allowed in a 320 MHz PPDU bandwidth' are related. However, because the second sentence doesn't say explicitly that the limitation of 484-tone RU is related to non-OFDMA in 320 MHz, it may be understood as a general limitation (both non-OFDMA and OFDMA). | Clarify that the second sentence is related to non-OFDMA. | REVISED Note to the Editor: Please apply the resolution of CID 11974 and refer to the text in 719.3 in D2.1.1 Same resolution for the following CIDs: 11220, 11974, 12206 |

## CID 11696

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.19 | 36.3.12.11.2 | Sounds a bit weird to say the U-SIG field belongs to a certain 80 MHz frequency subblock. Perhaps change it to "where it is being transmitted". | Please refer to the comment. | Revised Instructions to the Editor: In D2.1.1 719.19 replace the text "belongs" with "is transmitted" |

## CID 11697

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.21 | 36.3.12.11.2 | Please elaborate the meaning of the 4 digits, i.e., each digit represent a 20 MHz subchannel from the lowest to highest frequency, for clarity. | Please refer to the comment. | Revised Instructions to the Editor: In D2.1.1 719.25 append the following text to the end of the paragraph: "The 4-bit bitmap is indexed by the 20 MHz subchannels in ascending order with the MSB indicating the lowest frequency 20 MHz subchannel. " |

## CID 12849

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 703.18 | 36.3.12.11.2 | Replace "the punctured 20 MHz channels" with "the punctured 20 MHz subchannel(s)" | Replace "the punctured 20 MHz channels" with "the punctured 20 MHz subchannel(s)" | Accepted |