IEEE P802.11
Wireless LANs

| LB266 CR in 36.3.12.8.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.07.28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Stephen McCann |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D2.0. The changes are based on P802.11be D2.0.

5 comments are resolved.

10723, 10750, 11288, 12192, 13574,

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 10723

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 675.25 | 36.3.12.8.4 | "If the Beamformed field in EHT-SIG of an EHT sounding NDP is 1, then the receiver of the EHT sounding NDP should not perform channel smoothing when generating the compressed beamforming feedback report", if the EHT-LTF of the NDP frame is 2x-LTF then, the interpolation which is a kind of a smoothing is necessary to obtain the channel estimation for all the tones, in this case, Beamformed field is set to zero? We need further explanation. | Beamformed field set to 1 case needs more accurate description to accommodate the 2x-LTF case | Rejected When Ng=4 or 16, interpolation is not required. |

## CID 10750

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 673.20 | 36.3.12.8.4 | "Other values are Validate if dot11..." seems confusing in this case, because all values n is valid for the subfield. | Please remove the sentence or clarify what other values are meant. | Revised For single user non-OFDMA transmission (PPDU Type And Compression Mode field is set to 1), values larger than 0 are Validate. For non-OFDMA MU-MIMO transmission (PPDU Type And Compression Mode field is set to 2), value of 0 is Validate. Instructions to the editor: Please make the changes as shown in 11/22-1135r0, under CID 10750. Note to the editor: the resolution of CID 10750 and 11288 are the same. |

Instructions to the editor, please make the following changes to P685, L15 of P802.11be D2.1:

| B17–B19 | Number Of Non-OFDMA Users | 3 | Indicates the total number of non-OFDMA users. Set to n to indicate n+1 non-OFDMA users. For a non-OFDMA transmission to a single user, set to 0 to indicate one non-OFDMA user. A value larger than 0 is Validate if dot11EHTBaseLineFeaturesImplementedOnly is equal to true. For a non-OFDMA transmission to multiple users, set to a value larger than 0 to indicate more than one non-OFDMA users. A value of 0 is Validate if dot11EHTBaseLineFeaturesImplementedOnly is equal to true. |
| --- | --- | --- | --- |

## CID 11288

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 673.20 | 36.3.12.8.4 | "Other values are Validate if dot11EHTBaseLineFeaturesImplementedOnly equals true.". What are the "other values"? It looks like all possible values are already specified (i.e. 0 and larger than 0). | Clarify. Remove sentence if not applicable | Revised For single user non-OFDMA transmission (PPDU Type And Compression Mode field is set to 1), values larger than 0 are Validate. For non-OFDMA MU-MIMO transmission (PPDU Type And Compression Mode field is set to 2), value of 0 is Validate. Instructions to the editor: Please make the changes as shown in 11/22-1135r0, under CID 10750. Note to the editor: the resolution of CID 10750 and 11288 are the same. |

## CID 12192

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 672.34 | 36.3.12.8.4 | Clarify that for non-OFDMA case, value of "Number Of EHT- LTF Symbols" in EHT-SIG field indicates the initial number of EHT-LTFs or the total number of EHT-LTF symbols, including the extra EHT-LTF symbols. | Add a clarification statement. | Rejected Without mentioning a specific type, it is clear that the number of EHT-LTF symbols include all kinds of EHT-LTF symbols, including extra EHT-LTF symbols. |

## CID 13574

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 670.25 | 36.3.12.8.4 | RUs should be changed to RU or MRUs | Adding MRU between RU and s | Accepted Note to the editor: this resolution is part of the resolution of CID 13573 in 11-22/1130r1. Page 670 line 25 of D2.0 is page 682 line 25 of D2.1. |