IEEE P802.11
Wireless LANs

| Draft Technical Report on support of AMP IoT devices in WLAN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022-09-09 |  |  |  |  |
| Contributors: |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Weijie Xu | OPPO | Beijing China |  | xuweijie@oppo.com |
| YiNan Qi | OPPO | London UK |  | v-qiyinan@oppo.com |
| Zhisong Zuo | OPPO | Shenzhen China |  | zuozhisong@oppo.com |
| Chuanfeng He | OPPO | Beijing China |  | hechuanfeng@oppo.com |
| Shengjiang Cui | OPPO | Beijing China |  | cuishengjiang@oppo.com |
| Rongyi Hu | OPPO | Beijing China |  | hurongyi@oppo.com |
| Shukun Wang | OPPO | Beijing China |  | wangshukun@oppo.com |
| Zhi Zhang | OPPO | Beijing China |  | zhangzhi@oppo.com |
| Lei Huang | Huawei | Singapore |  | Lei.huang1@huawei.com |
| Boyce Bo Yang | Huawei | Nanjing China |  | yangbo59@huawei.com |
| Chenhe Ji | Huawei | Nanjing China |  | jichenhe@huawei.com |
| Shichao Zhao | Haier | Qingdao China |  | zhaoshichao@haier.com |
| Zhaojuan Du | Haier | Qingdao China |  | duzhaojuan@haier.com |
| Jiazhi Ni | Tencent | Beijing China |  | andyni@tencent.com |
| Harry Wang | Tencent | Beijing China |  | harryhwang@tencent.com |
| Wenhao Zhan | China Telecom | Guangzhou China |  | zhanwh@chinatelecom.cn |
| Xiaoqing Tang | HuBei University | Hubei China |  | Tangxq@Hubu.edu.cn |
| Heng Pan | Finsiot | Chengdu China |  | pan.heng@finsiot.com |

Abstract

This document is Technical Report on support AMP IoT devices in WLAN, which provides description on use cases, device types, functional requirements, technical feasibility and economic feasibility. Recommendations from AMP IoT TIG are outlined.

Revision History

r0 –July 08, 2022. skeleton of Technical Report on support AMP IoT devices in WLAN.

Table of Contents

1.	Introduction	3

2.	Use Cases	3

2.1	Use Case 1 Smart Manufacturing	3

2.2	Use Case 2 Data Center	5

2.3	Use Case 3 Logistics/Warehouse	5

2.4	Use Case 4 Smart Home	7

2.5	Use Case 5 Smart Agriculture	8

2.6	Use Case 6 Indoor Positioning	9

3.	AMP IoT device type and functional requirements	10

3.1	AMP-only IoT device	11

3.2	AMP-assisted IoT device	12

4.	Technical Feasibility	13

4.1	Ambient power and energy storage	13

4.2	Challenges of support AMP IoT devices	14

4.3	Potential candidate techniques	15

4.4	Feasibility of supporting AMP IoT devices in WLAN	19

4.4.1	Link budget for different AMP IoT device types	19

4.4.2	Co-existence with legacy 802.11 systems	22

4.4.3	Carrier generation for backscattering	23

4.5	Prototype	24

5.	Economic Feasibility	25

6.	Summary and recommendations	26

7.	References	26

Introduction

Note: Introduction of ambient power enabled IoT

In today’s IoT networks, the legacy IoT devices are usually driven by batteries with limited lifespan, which has significantly affected user experience in a negative way. The astronomical growing of IoT network together with the advent of huge amount of IoT devices has pushed the maintaining expenditure, including both labor and battery costs, to a whole new level. Billions of batteries have been disposed every year and only a small part of it can be efficiently recycled, leading to harmful impacts on the earth’s ecosystem. In some extreme environmental conditions, maintaining the operation of IoT networks and replacing the batteries can be quite challenging. In this regard, battery-free IoT communication has been proposed and it will improve the network performance and sustainability, and expand the application scenarios. In addition, battery-free communication can be much more environmentally friendly and much safer for the kids and the elders. By removing the battery, the device size and cost can be significantly reduced, thus paving the way to a variety of new applications.

The Wi-Fi IoT network is competitive from the perspective of deployment cost, due to already widespread deployment and free use of unlicensed frequency band. However, there are still lots of use cases and applications that can not be addressed using existing Wi-Fi IoT technologies in the following circumstances. Firstly, a device driven by a conventional battery is not applicable, e.g., under extreme environmental conditions (e.g., high pressure, extremely high/low temperature, humid environment). Secondly, maintenance-free devices are required (e.g., no need to replace a conventional battery for the device). Finally, ultra-low complexity, very small device size/form factor (e.g., thickness of mm), longer life cycle, etc. are required.

Ambient power-enabled IoT is a promising technology to enable battery-free communication and fulfil the requirements from various verticals. The operation of such devices relies on the energy harvested from a variety of sources including radio waves, light (sunlight), motion, heat, etc.so that the conventional battery can be removed and  Ambient power-enabled IoT is different from 802.11ah and standard Wi-Fi due to the following reasons: 1) Wi-Fi devices typically are powered by a conventional battery; 2) the typical peak ambient power is less than 1 mw (considering the restriction of the size of the device) and is much lower than the power consumption of legacy WiFi devices, e.g., 802.11ah devices, which is about tens of to hundreds of mw; 3) simple waveforms other than OFDM can be used for reduced complexity and power consumption. Combining ambient power-enabled IoT with Wi-Fi will enable a new kind of IoT service in many to-B and to-C areas, from which the Wi-Fi ecosystem will benefit vastly.

Use Cases

Note:  describe the typical use cases for AMP IoT, e.g., IWSN, smart home, logistics/warehouse, agriculture etc.

This clause summarizes the typical use cases enabled by AMP IoT and relevant requirements to fulfill the goals of various services.

Use Case 1 Smart Manufacturing

Smart manufacturing plays a critical role in increasing productivity and improving sustainability. The key to smart manufacturing is inter-connectivity and inter-operability between assets, storage tanks, materials and other passive objects in factories or industrial areas, enabled by wireless identification and sensing.

In smart manufacturing, identification is an essential service mainly for asset management and worker tracking, where labels can be attached to human beings or assets including production materials, storage tanks, containers, etc. [1] . Following functionalities can be provided

- Inventory check: once these tags are read remotely, the ID of the item to which the tag is attached can be acquired and traced as well as the information associated with the ID, e.g., position, product number, etc. Such information can then be further used for other purpose, e.g., supply chain management, anti-counterfeiting, etc.;

- Attendance check: once a worker enters or leaves a specific area of the factory or a working spot, the tag attached to him can be read and attendance can be automatically checked;

- Real-time inspection and tracing: a product can be assembled and move along multiple steps in a line of machines and workers as shown in Fig. 2.1-1. Real-time inspection and tracing are needed to guarantee that each machine or worker performing a particular job must finish this job before the product moves to the next step in the production line.

Fig. 2.1-1 Inspection and tracing in production line

Environmental sensing and monitoring are needed to collect real-time temperature and humidity information for production lines, computing and data centres and other equipment [1] [2] [3] . The weight and moving speed of a product in a production line can be collected via motion and pressure detection sensors. In addition, to maintain the safety within a factory, gas leakage in gas tanks and pipelines can also be detected by such sensing networks.

This use case requires ultra-small size, ultra-low cost and power consumption labels so that they can be deployed in very high density and the battery-less devices can enable maintenance-free operation. The following requirements are identified:

- Maintenance-free for long service life;

- Battery-less (i.e., no conventional battery is used);

- Coverage: up to 30 m for indoor case, up to 100 m for outdoor case;

- Data rate: up to 100 kbps.

Use Case 2 Data Center

Data center illustrated in Fig. 2.2-1 is the bedrock of modern ICT infrastructure, running software and processing data. The management network is essential to improve the reliability and efficiency, thus guarantees its safe and reliable operating conditions. Data centers are unique from all other building types, which need to be managed intelligently and comprehensively via following functionalities [2] :

- Environmental monitoring to capture data on temperature, pressure, humidity and air flow etc., and highlight potential inefficiencies;

- Facility monitoring to provide visibility into the entire power chain from the generator down to a specific outlet on an intelligent cabinet power unit, and diagnose potential facility problems;

- Asset management to maintain a centralized database that houses all of the IT and facility asset information including where the asset is located and how it is connected to other assets.

Asset management of data center is similar to smart manufacturing and the main objective is to keep an up-to-date record of all hardware and software within the data center and track the real time visibility and availability of the asset via label identification. Environment and facility monitoring in data center can also be done via wireless sensor network to monitor the real time operation, data and events of the DC facility, e.g., power consumption, water supply, AC etc. and respond to alerts and events, and issue tickets.

Fig. 2.2-1 Asset management in Data Center

This use case has the same requirements as smart manufacturing:

- Maintenance-free for long service life;

- Battery-less (i.e., no conventional battery is used);

- Coverage: up to 30 m for indoor case, up to 100 m for outdoor case;

- Data rate: up to 100 kbps.

Use Case 3 Logistics/Warehouse

In logistics, inventory check is needed to view all products and identify any missing assets and discrepancies within inventory [4] . Packages are stored in containers such as cartons and the cartons are normally piled in a warehouse. A pile of cartons in a single storage location can be more than 10 m long and up to 8 m high as shown in Fig. 2.3-1.

Fig. 2.3-1 Piles of cartons

The cartons are labelled for goods tracking and there could be hundreds of labels in a single storage location. It is inevitable that labelling does not follow a standard procedure, thus it is possible that not all labels face the same direction. It is quite time consuming and inefficient to scan these labels manually. Inventory check needs to be completed with high accuracy and in short time. In this regard, manual operation is clearly not preferred. Labels can be attached to the cartons when the cartons are transferred, stored, loaded/unloaded as shown in Fig. 2.3-2. Such tags can provide high sensitivity and omni-directionality so that they can be read remotely from multiple directions.

Fig. 2.3-2 Logistics/warehouse

Warehouses and distribution centers are usually equipped with sorting systems where random flows of items are sorted into orders for shipping. Sorting of the goods should be performed both when the goods are received and when the orders are prepared for sending out. In a sorting system, goods are usually placed in labelled containers and the container specifications can be either uniform, e.g., cartons and wooden trays, or non-uniform, e.g., packing bags. Information needed for sorting, such as the clients and the place of destination is associated with such labels that should be read when goods are sorted and prepared for transportation to the final destination. Two adjacent containers should be distinguished so that the target container is not misidentified with the adjacent container by the sorting system. For non-uniform container specifications, e.g., a packing bag, scanning from all possible sides should also be supported. Roller conveyors are widely used in sorting systems as shown in Fig. 2.3-2 and the tags attached to the containers may move at a speed of 1.5-2 m/s.

In sorting systems the following functionalities should be provided:

- Accurate label identification for closely adjacent items on the conveyor system;

- Fast label identification of the items on the conveyor belt that moves at a speed of 1.5-2 m/s;

- Real-time monitoring and related information acquisition for the items on the conveyor system;

- Environmental monitoring of temperature, humidity, etc. for specific items such as cold-chain transportation goods.

Fig. 2.3-2 Label reading in sorting system

The following requirements are identified,

- Ultra-low cost and ultra-small size for extremely high-density deployment;

- Maintenance-free and battery-less for long service time;

- 99.5% identification accuracy;

- Moving speed up to 2 m/s;

- Minimum distance to distinguish adjacent items: 0.5 m

- Coverage: up to 10-30 m for indoor case

Use Case 4 Smart Home

In a smart home, many devices such as smartphones, tablets, door locks, thermostats, home monitors, etc. are connected with each other. Low energy consumption and maintenance-free devices should be used for sensing and monitoring as shown in Fig. 2.4-1 [3] . For home environment sensing, such as temperature, humidity, etc., once the sensed information is collected by a controlling node, the heater, air-conditioner and (de)humidifier can be switched on/off automatically accordingly to adjust the temperature and humidity to a comfortable level. Such devices can also be used for home safety. For example, once gas leakage happens, a gas detector can send an alert to warn the home owner. Similarly, if a smoke detector senses there is a fire, it can automatically send the alert. In addition, motion detector is needed to detect the intruders and send alert to the home owner. Another important functionality needed is to locate keys, wallets and other personal belongs with attached labels.

Fig. 2.4-1 Smart home

The requirements for smart home use case are identified as follows,

- Low complexity and small size, e.g., thickness of 1 mm and area of several cm2;

- Long service life., e.g., more than 10 years;

- No need to replace/recharge a conventional battery, e.g., maintenance-free

- Coverage up to 10 m

- Horizontal positioning accuracy of 1~3 m

Use Case 5 Smart Agriculture

Smart agriculture focuses on providing the industry with the infrastructure to leverage advanced technology for tracking, monitoring, automating and analyzing operations. Similar to smart home, low energy consumption and maintenance-free devices can be used for sensing and monitoring, such as monitoring of soil moisture, soil fertility, temperature, wind speed, plant growth etc. [3] . For example, once the soil moisture is sensed, the irrigation system can be controlled to increase or decrease the supply of water to land or crops. Asset management for agricultural facilities can also be done by reading the labels attached to those facilities remotely.

Fig. 2.5-1 Smart agriculture

The main difference between smart agriculture and smart home is that smart agriculture needs to handle outdoor sensing and monitoring so that a much wider coverage should be maintained. Due to the much wider coverage, the number of connected devices can be hundreds to thousands. The requirements for smart agriculture use case are as follows,

- Battery-less, thus no need to use a battery;

- Low complexity and small size, e.g., thickness of 1 mm and area of no larger than several cm2;

- Coverage: up to 30m for indoor case, up to 200 m for outdoor case;

- Processing (i.e., reading IDs) hundreds to thousands of devices per second.

Use Case 6 Indoor Positioning

For indoor positioning, reference tags with known location can be densely deployed indoor to establish a navigating and positioning system that has a wide range of potential applicable venues such as giant shopping malls, parking, smart factories, warehouses, etc. [4] . Shopping centers offer a wide range of services and products, including large supermarkets, a collection of retail stores, restaurants, banks, theatres, fitness and leisure facilities, underground parking areas, professional offices and other establishments. Many giant shopping centers have been established all over the world, each can occupy an area of tens to hundreds of thousands m2, composed of one or multiple buildings, each of which has multiple-story both over and underground. While enjoy various services, people often have troubles in finding a vacant parking spot or his/her own car, a target shop/restaurant or a target item in a supermarket. The reference tags can be evenly distributed with high density, e.g., 2-meter intervals, within the entire shopping center on each floor and in each room as shown in Fig. 2.6-1. Indoor positioning can be enabled by a handheld device (e.g., smartphone), which can communicate with the reference tags.

Fig. 2.6-1 Indoor positioning for shopping center and parking

Such navigating and positioning system can also be used in the aforementioned use cases, e.g., smart manufacturing in factories, logistics/warehouse and smart home. In smart manufacturing, a product on the production line or conveyor system should be positioned precisely in order to identify in which step the product is. In the industrial area, there are some dangerous zones with toxic materials that are harmful to the health of the workers. Labels can be attached to workers and when they enter the dangerous zones, a safety alert can be immediately sent based on the real-time position of the workers. For logistics/warehouse, inventory and attendance check can also rely on such navigating and positioning system to locate the item or personnel.

Fig. 2.6-2 Indoor positioning for smart manufacturing and warehouse

The following requirements are identified for indoor positioning,

- Small size, maintenance-free, battery-free, and ultra-low-cost IoT devices;

- Coverage: 10-30 meters for indoor (an exemplary product line has an area of 25 thousand m2)

- Positioning accuracy: 1~3 m horizontal accuracy and 1~2 m vertical accuracy

- Moving speed: 1.5-2 m/s

AMP IoT device type and functional requirements

Note: discuss possible IoT device type and corresponding functional requirements, e.g., support energy harvesting, operation band(s), backward compatibility and co-existence with legacy 802.11 technologies, power consumption requirement, KPI requirements (e.g., coverage/data rate) etc.

To support aforementioned diverse use cases of AMP IoT, multiple device types should be defined. This clause gives definitions of AMP IoT device types. For a particular device type, the specific functional requirements are given accordingly.

In the first category of use cases, such as logistics/warehouse and smart home, object identification is one of the main functionalities provided by the AMP IoT devices. The essential information transmitted to the AP reader is the ID of the devices/tags that requires low peak data rate only (e.g., less than 100kbps). Since the devices/tags should be attached to all the objects within the service area, huge amount of  such devices/tags is needed. In this regard, manual operation of the devices/tags is extremely difficult and simple maintenance or even maintenance-free feature is necessary. In some of the use cases requiring environmental monitoring and sensor data reporting, such as smart manufacturing, smart home, etc., only small packet (e.g., less than 200 bits) and infrequent sensor data reporting (e.g., one packet per minute) are needed, e.g., temperature sensing and reporting. In these use cases, the devices/tags should have the features such as ultra-low complexity, ultra-low power consumption, very small form factor and battery-less (i.e., not using conventional battery). This device type is defined as AMP-only IoT device.

However, in some other use cases, e.g., smart manufacturing and smart agriculture, the AMP IoT devices may act as sensors, monitors and actuators. The communication between the AP reader and devices is more complicated in the sense that it may also require higher volume bi-directional data exchange between the AP reader and devices. The required data rate may be close to the existing IoT technologies. Therefore, higher capabilities which are similar as the current WiFi devices are needed. However, for these use cases, it also expects a maintenance-free IoT network (e.g., without replacing/recharging the battery), which is not possible to be achieved by the current technologies. Therefore, it calls for another type of IoT devices which has high capability but optimized for the power consumption and sustainability to adapt to ambient power usage and eventually may achieve the goal of maintenance-free operation. This type of device is defined as AMP-assisted IoT device.

At least the above 2 types, which can also be denoted as “low-end” and “high-end” device types, are envisioned for the AMP IoT devices[5] . More device types can be considered once identified during study phase.

AMP-only IoT device

The AMP-only IoT device targets at the first category of use cases. It features with ultra-low complexity, ultra-power consumption, very small form factor and battery-less (i.e., not using conventional battery). It may not need power storage or has limited power storage only (e.g., a capacitor).

- AMP-only IoT device has ultra-low complexity and ultra-low power consumption

  - The required data rate for identification is very low (e.g., less than 100kpbs). The device shall be designed as simple as possible while in the meantime fulfilling the requirements of data rate and communication distance. Much lower capability than the current WiFi devices is expected.

  - In order to achieve battery-less, it will use ambient power to drive itself and to communicate with the AP. The available ambient power would be very low as discussed in session 4.1, thus it requires much lower power consumption than the existing WiFi devices.

  - In most of the target use cases, it shall have a small size, which restrict the size of antenna and the energy harvester.

- AMP-only IoT device can have no power storage or very limited power storage.

  - For some of the target use case, the device can’t support power storage due to restriction such as the complexity, the acceptable cost and the constraint of the device size.

  - For some other use cases, the device can have very limited power storage to achieve more functions (e.g., connect to a sensor), higher performance (e.g., longer communication distance), or adapt to some kinds of unstable ambient power (e.g., to store unstable solar power).

- AMP-only IoT device can be used for light-weight applications, such as identification, positioning, infrequent and small sensor data reporting.

For AMP-only IoT device, the potential functional requirements include:

- Supported operation band.

It may be helpful to achieve low complexity and low power consumption by lower frequency band. This is due to its small channel bandwidth and good propagation property in lower frequency band. Therefore, sub 1GHz shall be considered with high priority. The 2.4GHz can also be considered since it is the mature frequency band widely used.

- Constraints of power consumption.

Since ambient power is used and energy harvester with small size can be utilized due to the small size restriction of the device, the power can be harvested is very limited. Therefore, ultra-low power consumption, e.g., less than 1 mill-Watt can be considered as the design target for AMP IoT.

- Coexistence.

Irrespective of sub1GHz or 2.4GHz, AMP-only IoT device will share same frequency band(s) with legacy WiFi devices. Therefore, backward compatibility and coexistence with legacy devices shall be supported. The regulation of these frequency band(s) shall be followed.

- Support energy harvesting.

In order to achieve battery-less (i.e., not using conventional battery), it will use ambient power to drive itself and to communicate with the AP. Therefore, energy harvesting should be supported by the AMP-only devices. For different use cases, different ambient power may be available thus different energy harvester can be supported based on the suitable ambient power for a specific use case.

- Coverage

As discussed in the use cases and requirements in session 2, up to 30m for indoor scenario and up to 100 m for outdoor scenario are required.

AMP-assisted IoT device

For AMP-assisted IoT device, higher capabilities similar as the current WiFi devices should be expected. The key design target is to achieve a maintenance-free IoT network (e.g., without replacing the battery) which can provide relatively high performance, which is similar as what the current WiFi devices can provide. It shall be optimized for the power consumption and sustainability to adapt to ambient power usage and achieve maintenance-free .

- AMP-assisted IoT device may be similar as legacy 802.11 (e.g., 802.11n/11ah) device, it can reuse the current PHY design but with enhanced MAC features to well adapt to operation with a specific kind of ambient power. Since the ambient power is very limited and sometimes unstable and uncontrollable, the enhanced power saving and power management can be considered to adapt for the unstable and uncontrollable ambient power.

- AMP-assisted IoT device can have higher power storage capability than AMP-only IoT device. Although the power consumption of AMP-assisted IoT device can be further optimized on top of the current Wi-Fi devices, its power consumption level will be much high than AMP-only IoT devices, e.g., tens of mill-walt to hundreds of mill-walt during transmission and reception. The typical power storge capacity is comparable to the existing IoT or other electrical devices, 300mAh.

The potential functional requirements for AMP-assisted IoT device include:

- Supported operation band:

- With similar capabilities as the current WiFi devices, it is easy for AMP-assisted IoT device to operate on the current frequency bands such as 2.4GHz and sub 1GHz. For example, if it is optimized on top of the 802.11ah devices which using sub 1GHz, the AMP-assisted IoT device can also use sub 1GHzSupport energy harvesting

In order to achieve maintenance free operation, it will use ambient power to sustain itself and to communicate with the AP. In order to fulfill the required higher power consumption compared with that of AMP-only devices, the energy harvester shall be able to provide a higher output power, e.g., more than 10mill-walt.

- Coverage

Similar coverage as the current WiFi devices can be expected, e.g., up to 30m for indoor case and up to 200m for outdoor case (note that for 802.11ah, it can be up to 1km).

Technical Feasibility

Note: discuss potential candidate techniques to fulfil the function requirements and aspects on feasibility including how to maintain backward compatibility and co-existence with legacy 802.11 devices, evaluation of link budget (for potential different IoT device types), impact on PHY and MAC etc.

This clause introduces different types of ambient power sources and their unique features, discusses potential candidate techniques to fulfil the function requirements and aspects on feasibility including how to maintain backward compatibility and co-existence with legacy 802.11 devices, evaluation of link budget (for potential different IoT device types), impact on PHY and MAC etc. Some prototypes using different energy harvest schemes and communication techniques are also introduced.

Ambient Power and Energy Storage

Energy can be harvested from different types of ambient power sources including RF radio, solar energy, thermal energy and piezoelectric power etc.

RF Energy

RF-based energy can be harvested from radio waves ranging from 3 kHz to 300 GHz using a single-stage or multistage converter. The amount of power that can be harvested depends on the source power, antenna gain, and the distance from the RF source. Ambient RF energy has a relatively low energy density and the conversion efficiency depends on the received power level e.g., the power density of RF power transfer is 0.4 to 60 microwatts at typical working distances (3 ~ 15 meters), assuming 30dBm transmission power, 6dBi antenna gain and 20% RF-DC efficiency, as shown in Figure 4.1-1.

Figure 4.1-1 Received RF power for different working distances

The main advantage of RF-based energy harvesting is its availability in indoor environments and the fact that RF power is controllable (e.g., power can be sent by a transmitter on demand or periodically). Potential applications include logistics/warehouse, manufacturing, smart homes, health monitoring, and environmental monitoring etc. The minimum RF power can be harvested is around -30dB [7] [8] . The conversion efficiency for RF Energy is listed in the table below.

Table 4.1-1 Conversion efficiency for RF Energy [9]

In practical wireless communication system such as Wi-Fi system, the IoT devices may be far away from Wi-Fi transmitters and  the far-field RF energy is weak. In addition, Wi-Fi signals of data communication are not continuous. Channel Utilization (CU) in a typical office, is around 10% in 2.4GHz channels, where

CU =TCCA-Busy/Tobservation.\

It is noted that CCA-Busy has a much lower threshold (i.e. -82dBm). It means IoT devices can only harvest energy from Wi-Fi signals transmitted by nearby transmitters. In order to improve energy harvesting efficiency greatly and make energy harvesting IoT devices more practical, a dedicated power signal from a dedicated energy source (e.g. an AP) may be needed.

Solar Energy/Light

Solar power/light can be transformed into electrical power using photovoltaic cells and it uses photovoltaic effect for energy harvesting with conversion efficiency of 10-40% [10] . For the outdoor case, solar energy is one of the most common ambient power, it can supply inexhaustible clean energy and has high power density of up to 100 mW/cm2 [11] . Solar power is unstable, inconsistent, and intermittent. It is highly dependent on the atmospheric condition, surrounding obstructions, etc. It is available during daytime but inefficient on a cloudy day or during the night. Solar energy harvesting can be mainly used for outdoor environmental monitoring, agriculture, husbandry, transportation, etc. For the indoor cases, light from the lighting equipment can be used. Although the power density is lower than solar, 100uw/cm2, it is much stable and controllable. Energy harvested from light can be used for manufacturing, indoor environmental monitoring etc.

Thermal Energy

Thermal energy is another ambient power source that are available for lots of use cases. Electrical power is directly generated by exploiting the temperature difference in thermoelectric devices taking advantage of thermoelectric effects, such as the Seebeck effect or the Thomson effect. Thermoelectric generators have low efficiency (only about 5–6%) [12] . The power density is 25~1000uw/cm2 depending the environment condition. Although with low conversion efficiency, thermal energy can be used in many outdoor applications or indoor cases as long as temperature difference or temperature fluctuation can be expected in the environment. For example, outdoor environmental monitoring, agriculture, husbandry.

Piezoelectric Energy

The piezoelectric effect generates electrical voltages or currents from mechanical strains, such as vibration or deformation. Typical piezoelectric-based energy harvesters keep creating power when there is a continuous mechanical motion, such as acoustic noises and wind, or they sporadically generate power for intermittent strains, such as human motion (walking, clicking a button, etc.). The volume of the piezoelectric power generators is relatively small and typical output power density values of usual piezoelectric materials are around 250 μW/cm3 but they can create more power when a motion or deformation is intense [13] [14] .

The cons and pros of different energy source types are summarized in the below table.

Table 4.1-2. Energy harvesting sources

Power storage

For typical ambient power, it can be observed the harvested power is very limited, e.g. from 1uW to 100 mW(per cm2/cm3). In some application limited power is not sufficient to drive sensors that requires higher peak power consumption, as shown in Table 4.1-2. For some artificial power sources (e.g., light, RF waves), the power can be stable and constant. However, for other kinds of ambient power such as solar, heat or vibration, the ambient power lacks of stability and it is impossible to use the ambient power directly for electronic devices.

Table 4.1-2 Power consumption of IoT devices

| Components of IoT devices |  | Average power | Peak power | Sleep power | Comments |
| --- | --- | --- | --- | --- | --- |
| Part1: Sensors | Door sensor [27] | 1µW | 5mW | 0.1µW | 5 events /second, 40us each |
|  | Humidity sensors [28] | 1µW | 1mW | 0.1µW | 1 event /second , 1ms each |
|  | Accelerometer [29] | 2µW | 2µW | 0.1µW | average power when output data rate is 100Hz |
|  | Temperature sensors [30] | 2µW | 400 µW | 0.2µW | 1 event/second, 5ms each |
|  | Light sensor [31] | 10µW | 10µW | 1µW | average power at full scale lux |
|  | Proximity sensor [32] | 10µW | 200µW | 10µW | capacitive sensor |
|  | Pressure sensor [33] | 20µW | 1mW+ | 2µW | 1measure/ second |
| Part2: MCU | ~ [34] | ~ | 0.6 ~ 9mW | 1µW | varies for different clock frequencies |
| Part 3: Wireless Radios | Bluetooth Low Energy [35] | 77mW * duty cycle | 77mW | 10µW | 2.4GHz@8dBm |
|  | Wi-Fi [36] | 429mW* duty cycle | 429mW | 40µW | 2.4GHz@8dBm |

In this regard, energy storage element is needed for some AMP IoT devices. The energy storage element is able to stabilize and control the power output, smooth the fluctuation of power output and power usage. Typical IoT sensors, such as temperature sensors, humidity sensors, light sensors, etc., turn to active mode around every 30 seconds and then turn back to doze mode several milliseconds later. Hence, it is possible that the energy storage element can harvest energy from weak ambient power (e.g., in the level of micro ampere or even nano ampere) and provide the required peak discharge current (e.g., tens of uA to hundreds of uA) for the AMP IoT devices. Therefore, the usage of energy storage element makes it possible to harvest energy from more kinds of ambient power sources for AMP IoT.

Capacitor and solid-state battery can be considered as the possible energy storage elements for AMP IoT devices to provide limited but appropriate/sufficient power storage capacity. The capacitor or solid-state battery is just like a cistern. The system works well as long as the average input power is equivalent to or larger than the average output power. (The benefit of adding a capacitor or battery is that the input power doesn’t have to be equivalent to or larger than the output power at any time).

For example, with a capacitor of 24uF, if it is fully charged, it can drive the AMP IoT devices for 3.6 seconds (1.5V and 10uA are assumed). With a solid-state battery of 1uAh@1.5V, it can drive the AMP IoT devices for 6 minutes (1.5V and 10uA are assumed).

Challenges of support AMP IoT devices

AMP IoT devices have distinguished characteristics such as battery-less, ultra-low cost, small size, maintenance-free and long-life cycle. It has great potential to fulfill the unprecedent requirements from vertical industries and create an entirely new market. In order to support AMP IoT devices in Wi-Fi system, following technical challenges should be resolved [5] .

The first challenge is it needs to support low complexity design and  to reduce the complexity of AMP IoT devices to an extremely low level so that the cost and power consumption of the AMP IoT devices can be extremely low, and the size can be very small. The ultra-low complexity can be achieved by simplifying the RF chain and baseband architecture, reducing memory size, and removing unnecessary components. In addition, the communication procedure between the AMP IoT device and the AP should be designed as simple as possible. For example, instead of OFDM, simpler waveform can be supported since OFDM requires more complicated baseband processing.

The second challenge is ambient power is unstable and the energy that can be harvested from RF signals is low thus how to significantly reduce the power consumption of the AMP IoT devices needs to studied. Since the main power source is ambient power and the size of the device constrains the size and efficiency of the energy harvester, the power that can be harvested is very limited. For example, as summarized in session 4.1, only up to tens of micro-Watt power can be harvested from wireless radio waves and less than 1 mill-Watt power can be harvested from solar panel with size of 1 cm2. Therefore, a simplified and low power radio should be designed in the next IEEE 802.11 amendment for IoT devices, e.g., peak power consumption of less than 1 mill-Watt can be considered as the design target for AMP-only IoT devices.

The third challenge is efficient power management of the unstable and uncontrollable power supply from energy harvester. In some typical use cases, only small amount of power can be harvested and the ambient power is unstable. Therefore, the device should adapt itself to such condition and the consumed power should be further saved by applying efficient power management schemes.

Finally, although the AMP IoT device is with much lower capability compared with normal Wi-Fi device, when operating in legacy Wi-Fi frequency band e.g., sub 1 GHz or 2.4 GHz, it should be compatible with legacy Wi-Fi system and thus be able to coexist with legacy Wi-Fi devices. In this regard, the AMP IoT device should follow the regulation for these unlicensed frequency bands and use the frequency bands in a fair way with other Wi-Fi devices. For example, it shall meet the PSD requirement and the maximum transmission EIRP requirement.

Potential candidate techniques

In order to tackle the above challenges faced by the AMP IoT devices, following candidate techniques are investigated [5] .

Narrow bandwidth operation

Narrow bandwidth operation is a natural choice to realize low complexity and power consumption because the complexity of both RF and baseband is significantly simplified compared with wideband operation, e.g., 20 MHz in 2.4 GHz. In addition, the circuit can work with much lower frequency cycle and limited calculation is needed for narrow bandwidth transmission and reception. From the perspective of data rate, narrow bandwidth operation is feasible to meet the target peak data rate of 100 kbps, which is sufficient for most of the use cases in session 2.

One of the target frequency bands could be Sub 1GHz, i.e., the same band as 802.11ah, where the channel bandwidth can be as small as 1MHz. In Sub 1GHz, the device can be designed with an operation bandwidth less than or equal to 1MHz. In other frequency band, e.g., 2.4GHz, where only 20MHz channel bandwidth is allowed, AMP IoT devices using narrow bandwidth can still be supported.

In order to support narrow bandwidth operation, new UL/DL PPDU format shall be defined. It should be noted that 4MHz DL PPDU is already supported for WUR. In the meantime, a wideband preamble for backward compatibility shall be supported for the AMP IoT device. In this regard, potential new PPDU format as shown in Figure 4.3-1 can be considered to support narrow bandwidth operation for AMP IoT devices. In the new PPDU format, there is a legacy preamble followed by AMP portion. The AMP portion is further divided into AMP preamble, AMP header and payload. The legacy preamble has the same bandwidth as the channel bandwidth in the target operation band but the AMP portion can have a smaller bandwidth to meet the peak data rate requirement, e.g., 200KHz in a 1MHz channel bandwidth in sub 1GHz.

Figure 4.3-1 Potential new PPDU format enable narrow bandwidth operation

Simpler waveform/modulation/coding scheme

In order to achieve ultra-low power consumption and ultra-low complexity, simpler waveform and coding scheme are needed. OFDM is the main waveform used in Wi-Fi and the merit of OFDM is that it can achieve high spectrum efficiency and high peak data rate using wide bandwidth. However, it is difficult to use OFDM to achieve ultra-low power consumption since the operations such ADC, data buffering, FFT, channel estimation, etc. requires high power consumption. Therefore, OFDM may not be suitable for AMP IoT and much simple waveform is required.

Figure 4.3-2 OOK modulation

OOK/FSK may be a promising modulation scheme for AMP IoT to enable ultra-low complexity data transmission/reception. In a OOK receiver, envelope detection can be used and complicated baseband digital processing is replaced with simple analog envelope detection circuit. Thus ultra-low power (e.g., several micro walt to tens of micro walt) can be achieved by very simple implementation [15] -[16] [17] . For the transmitter, it can also achieve ultra-low power transmission (e.g., around 200 micro walt) even with an active OOK/FSK transmitter [18] - [20] . In addition, OOK/FSK can be applied together with backscattering to further reduce the device complexity and power consumption significantly. Therefore, with OOK/FSK, the potential target ultra-low power consumption, e.g., lower than 1 mill-Walt, can be achieved. Another merit of OOK is that OOK has already been supported in 802.11ba for WUR thus less PHY specification impact is expected.

Figure 4.3-3 FSK modulation

Backscattering

In a backscattering communication system as illustrated in Figure 4.3-4, load modulation is usually used. The load modulation technology mainly includes two methods: resistance-based load modulation and capacitor-based load modulation. For resistance-based load modulation, a resistor which is called a load modulation resistor, is connected in parallel to the load. The resistor is turned on or turned off according to the clock of the data stream, and the switch is controlled by the binary data encoding. For capacitor-based load modulation, a capacitor is connected in parallel with the load to replace the load modulation resistor.

Figure 4.3-4 Backscattering communication

Taking resistance-based ASK modulation as an example, as shown in Figure 4.3-5, the device can switch between absorption state and reflection state by adjusting the load reflection coefficient. In the absorption state, the device achieves impedance matching thus the input RF signal is completely absorbed by the terminal. Hence, the signal received by the reader will be at low-level, which indicates a bit '0'. On the contrary, in the reflection state, the device adjusts the circuit impedance that leads to a mismatch of the impedance thus a part of the RF signal is reflected. Then the signal received by the reader will be at high-level to indicate a bit '1'.

Figure 4.3-5 Resistance-based modulation

Similarly, the device can also change the response frequency of the circuit by adjusting the capacitance of the circuit to realize FSK modulation. FSK has better BER performance than ASK. It is often used to realize frequency division multi-access.

Therefore, backscattering communication achieves extremely low-complexity signal modulation and transmission via impedance modulation. The backscatter terminal does not require complex RF structures, such as PA, high-precision oscillator, duplexer, and high-precision filter. There is also no need for complex baseband processing, complex channel estimation and equalization operations. In addition, one distinguished characteristic is that it doesn’t need to generate a high frequency carrier but instead uses the incoming carrier as the carrier for backscattering transmission. It is a promising scheme to enable ultra-low complexity and ultra-low power consumption (e.g., lower than 1 mill-Walt). In addition, it is beneficial to use backscattering to support co-existence with legacy devices, e.g., by backscattering the preamble sent by the AP, as discussed in session 4.4.2. For backscattering, since it uses the carrier signal from the AP. The carrier signal has to propagate within both the DL and UL: the carrier signal is sent from the AP to the device and the device backscatter the signal to the AP. Hence, the communication distance will be limited and may not be sufficient for some use cases requiring relative long communication distance. LNA (low noise amplifier, LNA) can be used to boost the backscattering signal [21] [22] . The integration of LNA in AMP IoT with high sensitivity receiver can effectively make up the communication distance of backward link (see session 4.4.1 for more details).

Light-weight MAC protocol design and enhanced power saving/management

In order to achieve ultra-low power consumption and ultra-low complexity, MAC can be further simplified on top of 802.11 ah, e.g., simplified MAC PPDU format and communication process between AP and AMP IoT devices, and introducing efficient access control mechanisms etc.

Schemes to support ultra-low power operation with limited power supply from energy harvester also needs to be considered. Currently, TWT/RAW, Energy limited operation, PS-poll etc are introduced in Wi-Fi for power saving. On top of these mechanisms, it can be investigated whether there are additional methods for further power saving. In addition, the characteristics of ambient power sources, e.g., instability, limited amount of harvested power call for efficient power management schemes. Furthermore, the low peak data rate requirement for AMP-IoT devices provide a good perquisite to optimize power consumption and power management.

Support coexistence schemes with legacy devices

It is important to maintain coexistence with legacy Wi-Fi for AMP IoT. See session 4.4.2 for the details.

- Potential Technologies for AMP-only IoT devices

As discussed in session 3, it targets at use cases requiring ultra-low complexity, ultra-low power consumption and maintenance-free for AMP-only IoT devices. Based on the discussion above, it can be summarized there may be two possible techniques combination for AMP-only IoT devices.

- Combination 1: Ultra-low power receiver + Backscattering

It can utilize techniques such as narrow bandwidth operation for AMP portion (e.g., 187.5 kHz in sub-1 GHz), simpler waveform/modulation/coding scheme (e.g., OOK/FSK), backscattering, simplified MAC protocol design and enhanced power saving/management.

- Combination 2: Ultra-low power receiver + Ultra-low power active transmitter

It can utilize techniques such as narrow bandwidth operation for AMP portion (e.g., 187.5 kHz in sub-1 GHz), simpler waveform/modulation/coding scheme (e.g., OOK/FSK), simplified MAC protocol design and enhanced power saving. The difference with the above combination 1 is the AMP IoT device has an active transmitter with ultra-low power consumption.

- Potential Technologies for AMP-assisted IoT devices

For AMP-assisted IoT devices, it targets at uses case where similar performance as legacy WiFi SAT are needed but it shall support ambient power. It requires schemes to adapt itself to operate with ambient power by taking into account its characteristics, e.g., instability, limited amount of harvested power (but it can be higher than use case for AMP-only IoT devices). The key enhancement for AMP-assisted IoT devices is further power saving and power management enhancement. Therefore, the possible techniques combination for AMP-assisted IoT devices is:

- Combination 3: Follow legacy PHY design with MAC enhancement

The AMP IoT device has similar capability as legacy STA, but possibly with simplified MAC protocol design and/or enhanced power saving/management.

Feasibility of supporting AMP IoT devices in WLAN

Link budget for different AMP IoT device types

In order to evaluate the coverage performance of AMP IoT. Link budget analysis is performed. AMP-only device defined in session 3 are assumed since AMP-assisted IoT device has similar coverage as the current Wi-Fi device. The frequency band are assumed to be sub 1GHz and 2.4GHz respectively. Three types of AMP IoT device with backscatter transmitter are assumed:

Case 1: energy is harvested from RF and without power storage

Case 2: energy is harvested from RF and with power storage

Case 3: energy is harvested from light and with power storage

In case 1, AMP-only IoT device has no capability of power storage and the device shall operate using the instantaneously harvested power. Typically, with the current implementation, the minimum received RF power shall be no less than -20dBm to power up the device. In case 2, AMP-only IoT device has the capability of power storage and the device can work using the RF power that is harvested and stored. Hence, the minimum received RF power can be relaxed to -30dBm. Even though the instantaneously harvested power may not be sufficient to drive the device but it can be accumulated in the power storage unit. In case 3, other kinds of ambient powers, e.g., light, are assumed so that it doesn’t rely on RF power for energy harvesting. The minimum received signal strength, i.e., the receiver sensitivity of AMP IoT device is assumed to be -45dBm in the evaluation. In order to achieve a long communication distance in the UL, an LNA with 30 dBm gain is assumed to amplify the backscattering signal. It should be noted that Friis equation is applied in the link budget evaluation. The evaluation results for case 1/2/3 at sub1 GHz and 2.4GHz are illustrated in the below Tables.

Table 4.4 -1 Link budget at sub 1GHz for case 1/2/3

|  | Case1 | Case2 | Case3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 920 | 920 | 920 |
| EIRP of AP (dBm) | 30 | 30 | 30 |
| Receiver sensitivity of AP (dBm) | -95 (Note 1) | -95 | -95 |
| Antenna gain of IoT device (dBi) | 2 | 2 | 2 |
| Minimum receiving power for IoT device (dBm) | -20 (Note 2) | -30 (Note 2) | -45 (Note 3) |
| Maximum communication distance from AP to IoT device (m) | 10.33 | 32.67 | 183.71 |
| Backscattering loss at IoT device (dB) | 5 | 5 | 5 |
| Low Noise Amplifier factor (dB) | 0 | 0 | 30 (Note 4) |
| Maximum communication distance from IoT device to AP (m) | 103.31 | 32.67 | 183.71 |
| *Notes: - Reuse the receiver sensitivity of 802.11 ah AP - The minimum required signal power for an IoT device is -20dBm when the IoT device can’t store power itself. It can be -30dBm when the IoT device has the capability of power storage. - -45 dBm is assumed as the sensitivity of ultra-low power receiver[15] [16] [17] . - LNA with 30 dBm gain is assumed to boost the backscattering signal, it can have ultra-low power consumption [21] [22] . |  |  |  |

Table 4.4-2 Link budget at 2.4GHz for case 1-3

|  | Case1 | Case2 | Case3 |
| --- | --- | --- | --- |
| Frequency (GHz) | 2.4 | 2.4 | 2.4 |
| EIRP of AP (dBm) | 27 | 27 | 27 |
| Receiver sensitivity of AP (dBm) | -95 (Note 1) | -95 | -95 |
| Antenna gain of IoT device (dBi) | 2 | 2 | 2 |
| Minimum receiving power for IoT device (dBm) | -20 (Note 2) | -30 (Note 2) | -45 (Note 3) |
| Maximum communication distance from AP to IoT device (m) | 2.8 | 8.87 | 49.85 |
| Backscattering loss at IoT device (dB) | 5 | 5 | 5 |
| Low Noise Amplifier factor (dB) | 0 | 0 | 30 (Note 4) |
| Maximum communication distance from IoT device to AP (m) | 39.60 | 12.52 | 70.42 |
| *Notes: - Reuse the receiver sensitivity of 802.11 ah AP - The minimum required signal power for an IoT device is -20dBm when the IoT device can’t store power itself. It can be -30dBm when the IoT device has the capability of power storage. - -45 dBm is assumed as the sensitivity of ultra-low power receiver. - LNA with 30 dBm gain is assumed to boost the backscattering signal, it can have ultra-low power consumption. |  |  |  |

One additional type of AMP IoT device with an active transmitter is assumed:

Case 4: energy is harvested from light, and the AMP IoT device is with power storage and using active transmitter

In case 4, light is assumed to be the ambient power source. Similarly, -45dBm is assumed as the receiver sensitivity of the AMP IoT device. The difference from case 3 is that an active transmitter is used and the maximum transmission power is assumed to be -15dBm. With such an active transmitter, the power consumption can still be lower than 1mw [18] -[20] . The evaluation results are illustrated as below.

Table 4.4-3 Link budget at sub 1GHz and 2.4GHz for case 4

|  | Case4 | Case4 |
| --- | --- | --- |
| Frequency (GHz) | 920 | 2.4 |
| EIRP of AP (dBm) | 30 | 30 |
| Receiver sensitivity of AP (dBm) | -95 | -95 |
| Antenna gain of IoT device (dBi) | 2 | 2 |
| Minimum receiving power for IoT device (dBm) | -45 | -45 |
| Maximum communication distance from AP to IoT device (m) | 183.71 | 70.42 |
| Maximum transmission power of IoT device (dBm) | -15(Note) | -15(Note) |
| Maximum communication distance from IoT device to AP (m) | 259.49 | 99.47 |
| *Notes: Ultra-low power active transmitter is assumed. |  |  |

From the link budget results, it can be observed that:

- When RF power is adopted, the communication distance of downlink is the bottleneck due to RF energy harvesting.

- With RF power harvesting, the communication distance of downlink would be limited and at 2.4GHz if the device has no power storage.

- Energy harvesting from other ambient power source can support a medium downlink coverage (up to 180 meters at sub1 GHz and up to 50 meters at 2.4GHz) for AMP IoT device.

- A low power LNA can efficiently boost the UL coverage.

- With an active low power transmitter, a relative larger uplink coverage can be achieved (up to 180 meters at sub 1GHz and up to 70 meters at 2.4GHz).

Co-existence with legacy 802.11 systems

As discussed in session 3, AMP IoT will operate at sub 1GHz or 2.4GHz, the co-existence with legacy 802.11 technologies should be studied to guarantee backward compatibility[6] . Co-existence requires the AMP IoT device to follow the regulation for these unlicensed frequency bands and use the frequency bands in a fair way with other Wi-Fi system. For example, it shall meet the PSD requirement and the maximum transmission EIRP requirement.

The following issues are identified for the co-existence of AMP IoT and legacy 802.11 systems.

- AMP IoT device may only support simple waveform due to its ultra-low complexity requirement, such as ASK modulated waveform, which is different from OFDM waveform used by legacy Wi-Fi system in Sub 1GHz or 2.4GHz. How to support co-existence if new waveform for AMP IoT is used should be studied.

- When AMP IoT and legacy 802.11 systems share the same frequency band, a uniform CSMA/CA mechanism should be used between them. It is a challenge for AMP-only IoT device to support CSMA/CA due to its ultra-low complexity. For example, it is difficult or even impossible for AMP-only IoT device to transmit and detect legacy preamble for carrier sensing. In addition, the power consumed for CSMA/CA operation may be beyond the capability of AMP IoT devices with ultra-low power consumption.

In order to solve the co-existence issue, proper mechanism taking into account both the regulation requirement and the constraints of AMP IoT devices shall be studied.

AMP PPDU format should be defined for AMP IoT. The co-existence issues shall be taken into account when designing the AMP PPDU format. In Figure 4.4-1, one example of AMP PPDU format is illustrated.   In this PPDU format, there is a legacy preamble portion and an AMP portion. The preamble portion is transmitted for carrier sensing by legacy 802.11 devices. With this preamble, the legacy 802.11 devices can detect the transmission from AMP IoT devices. For AMP-only IoT devices operating in Sub 1GHz, the AMP PPDU format contains 802.11ah preamble (e.g., LTF, STF and SIG portion), followed by the AMP portion. For AMP-only IoT devices operating in 2.4GHz, the AMP PPDU format contains legacy preamble (e.g., L-STF, L-LTF, L-SIG), followed by AMP portion.

However, it is challenging for AMP-only IoT device to generate the OFDM based preamble with a low complexity transmitter which can only generates simple waveform such as OOK. Backscattering can be one method to realize the transmission of legacy preamble from AMP-only IoT device. The AP can help to generate a preamble and send it to the AMP-only IoT device. Then the device backscatters the preamble. Immediately after the preamble, the AMP-only IoT device transmits the AMP portion. Therefore, the whole AMP PPDU format can be transmitted by the AMP-only IoT device. This procedure is shown in the following Figure.

Figure 4.4-1 Preamble transmission using backscattering

Although AMP-only IoT device can send legacy preamble through backscattering, it is still difficult for AMP-only IoT device to perform channel access due to its ultra-low power and low complexity receiver.

TXOP sharing defined in 802.11ah can be reused for the channel access for AMP-only IoT device. AP can be responsible for channel occupation with normal channel access procedure. Then, the AP can share its TXOP to AMP-only IoT device when there is uplink transmission. With this method, AMP-only IoT device does not have to perform channel access by itself.

The AMP PPDU with the AMP portion can only be received by the AP that is able to support AMP IoT. As shown in Figure 4.3-1, the bandwidth of AMP portion can be narrower than that of the preamble part and simple waveform (e.g., OOK) will be adopted for the AMP portion. For the AP, it is beneficia to ease the implementation if the legacy OFDM transmitter can be reused to transmit the AMP PPDU format which contains a AMP portion.

Carrier generation for backscattering

Based on previous discussion, it can be concluded that backscattering is a promising technique to achieve ultra-low power consumption and ultra-low complexity for AMP IoT. However, continuous wave, e.g., sine wave, is used as the carrier for backscattering in a typical backscattering system. If backscattering is introduced for AMP IoT , the following issues need to be considered:

- The carrier shall be able to provide enough RF power to drive the IoT devices

- The higher the power, the longer the communication distance. Based on the evaluation in 4.3.1, a high power is needed in order to provide a sufficient communication distance.

- There is PSD restricted by regulation, e.g. 10 dBm/MHz (or 17 dBm in China for device with high antenna gain) at 2.4GHz. It means that a narrow bandwidth signal can not provide sufficient power for backscattering. For example, a carrier narrower than 1MHz can only provide a signal lower than 10dBm at 2.4GHz.

- The maximum transmission power of Wi-Fi devices, e.g. 20/27dBm

- Backward compatibility shall be maintained considering the impact of specification and implementation

- For a typical OFDM transmitter, it may not be able to generate a sine wave with a narrow bandwidth.

There may be two potential carrier signals for AMP IoT devices.

- Carrier signal with narrower bandwidth, e.g., a sine wave

- Carrier signal spanning a wider bandwidth, e.g., the signal spanning across the 20MHz channel bandwidth at 2.4GHz

It seems that carrier signal spanning a wider bandwidth is more appropriate to serve as the carrier signal for backscattering. It allows higher RF power with wider transmission bandwidth due to the maximum PSD restriction. It can also improve the RF energy transfer (i.e., the diversity gain) and energy harvesting efficiency with a carrier signal spanning a wider bandwidth [23] . A carrier signal spanning a wider bandwidth is illustrated in the following Figure 4.4-2. In addition, from backward compatibility perspective, it is also beneficial for carrier signal to span a wider bandwidth as the legacy transmitter can be reused.

Figure 4.3-2 Carrier signal spanning wider bandwidth within a channel bandwidth

Prototype

Note: To be updated

There are already some prototypes to verify the feasibility of AMP IoT. Three typical prototypes are presented to show the potential communication techniques, the feasibility energy harvesting from ambient power, e.g., RF radio, heat, etc., and the achieved performance.

Figure 4.5-1 Prototype 1 for RF radio energy harvesting and backscattering

Prototype 1 is shown in Fig. 4.5-1, where the source for energy harvesting is RF radios and the communication is via backscattering. This prototype shows that the effective RF radio energy harvesting distance is up to 15 m and the communication distance can be up to 100 m in outdoor. The size of the AMP IoT device is smaller than a one Chinese yuan coin.

Figure 4.5-2 Prototype 2 for thermal energy harvesting and sensing

Prototype 2 and 3 are shown in Fig. 4.5-2 and Fig. 4.5-3, where the energy is harvested from different ambient power sources, e.g., thermal energy and induced current. The harvested energy will be used for temperature sensing, voltage and current.

Figure 4.5-3 Prototype 3 for induced current energy harvesting and sensing

Economic Feasibility

Note: discuss the economic feasibility of AMP IoT.

To be updated based on further input.

IoT network has spread into nearly every aspect of life, and, as such, is a driver for economic growth, social cohesion/inclusion, and for the improvement of welfare and well-being. Disruptive solutions are however required to sustain this evolution. In this regard, explosive market growth is predicted for battery-free tags and sensors. AMP IoT will tackle all the main challenges, in order to create batter-free via energy harvesting and thus maintenance free and sustainable networks, enabling IEEE to strengthen the position in the field and remain at the forefront of the evolution. One of the most important functionalities that is required in many identified use cases is object identification. Nowadays, the main technique used for object identification is RFID. The RFID Market is projected to reach USD 35.6 billion by 2030 from USD 14.5 billion in 2022 and it is expected to grow at a CAGR of 11.9% from 2022 to 2030 [24] . The RFID sensor market is expected to expand from an output of 18,836.5 million units in 2021 and surpass 49,116.4 million units by 2031 [25] . AMP IoT is a promising candidate technology to play a critical role in the market of object identification by potentially providing remote, automatic, omni-directional, highly efficient and reliable object identification. With these new characteristics, AMP IoT can be further applied in asset management in new market such as smart agriculture where the coverage requirement is beyond the capability of RFID.

AMP IoT can also be used to establish an indoor positioning and navigation system. The global Indoor positioning and navigation market was valued at $6.92 billion in 2020 and is projected to grow to $23.6 billion in 2025 at a CAGR of 27.9% [26] . Unlike the outdoor use cases where Global Navigation Satellite System can be used, it calls for much more efficient indoor positioning and navigation technologies for indoor use cases. AMP IoT based indoor positioning and navigation system can be easily deployed with low cost and can be easily used by the verticals and individual customer.

Summary and recommendations

To be updated.

References

- IEEE 802.1122/1339r0, Use Cases of smart manufacturing

- IEEE 802.1122/1341r1, Use Cases of Data Center Infrastructure Management.

- IEEE 802.11-22/0963r0, Use Cases for AMP IoT Devices.

- IEEE 802.11-22/1559, Updated Use Cases for AMP IoT Devices.

- IEEE 802.11-22/0962r0, Potential Techniques to Support AMP IoT Devices in WLAN

- IEEE 802.11-22/970r0, Feasibility of supporting AMP IoT devices in WLAN

- Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

- Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

- Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

- Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

- Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

- Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

- Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

- H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

- J. Im, H. -S. Kim and D. D. Wentzloff, "A 470µW −92.5dBm OOK/FSK Receiver for IEEE 802.11 WiFi LP-WUR," ESSCIRC 2018 - IEEE 44th European Solid State Circuits Conference (ESSCIRC), 2018, pp. 302-305, doi: 10.1109/ESSCIRC.2018.8494331.

- J. Im, H. Kim and D. D. Wentzloff, "A 217µW −82dBm IEEE 802.11 Wi-Fi LP-WUR using a 3rd- Harmonic Passive Mixer," 2018 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), 2018, pp. 172-175, doi: 10.1109/RFIC.2018.8428988.

- R. Liu et al., "An 802.11ba-Based Wake-Up Radio Receiver With Wi-Fi Transceiver Integration," in IEEE Journal of Solid-State Circuits, vol. 55, no. 5, pp. 1151-1164, May 2020, doi: 10.1109/JSSC.2019.2957651.

- K. Tang et al., "A 75.3 pJ/b Ultra-Low Power MEMS-Based FSK Transmitter in ISM-915 MHz Band for Pico-IoT Applications," 2021 IEEE International Symposium on Circuits and Systems (ISCAS), 2021, pp. 1-4, doi: 10.1109/ISCAS51556.2021.9401715

- M. S. Jahan, J. Langford and J. Holleman, "A low-power FSK/OOK transmitter for 915 MHz ISM band," 2015 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), 2015, pp. 163-166, doi: 10.1109/RFIC.2015.7337730.

- J. Bae and H. Yoo, "A low energy injection-locked FSK transceiver with frequency-to-amplitude conversion for body sensor applications," 2010 Symposium on VLSI Circuits, 2010, pp. 133-134, doi: 10.1109/VLSIC.2010.5560325

- J. Kimionis, A. Georgiadis, Sangkil Kim, A. Collado, K. Niotaki and M. M. Tentzeris, "An enhanced-range RFID tag using an ambient energy powered reflection amplifier," 2014 IEEE MTT-S International Microwave Symposium (IMS2014), 2014, pp. 1-4, doi: 10.1109/MWSYM.2014.6848653.

- D Matos, R Correia,NB Carvalho, ”Dual-Band FET-Based Reflection Amplifier for Backscatter Modulator Performance Enhancement”  URSI Radio Science Letters, 202

- Clerckx B, Huang K, Varshney L R, et al. Wireless power transfer for future networks: Signal processing, machine learning, computing, and sensing[J]. IEEE Journal of Selected Topics in Signal Processing, 2021, 15(5): 1060-1094.

- 11-22-0645-00-0wng-ambient power enabled IoT for Wi-Fi

- https://www.marketsandmarkets.com/Market-Reports/rfid-market-446.html?gclid=EAIaIQobChMI1KnTy-Tl-AIVydeWCh1CqArgEAAYASAAEgJhX_D_BwE

- https://www.researchandmarkets.com/reports/4531980/indoor-positioning-and-navigation-market.

- Texas Instruments, DRV5032 datasheet

- Texas Instruments, HDC2080 datasheet

- Analog Devices, Inc., ADXL367 datasheet

- Sensirison, STS4x datasheet

- Texas Instruments , OPT3001 datasheet

- Analog Devices, Inc., AD715 datasheet

- ST, ILPS22QS datasheet

- Texas Instruments. MSP430 datasheet

- Silicon Labs,  BGM111  datasheet

- Silicon Labs, RS9116 datasheet