IEEE P802.11
Wireless LANs

| LB266 CR for 36.3.10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.10.12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Lei Huang | Huawei |  |  | lei.huang1@huawei.com |

Abstract

This submission contains proposed comment resolutions to 11 comments related to clause 36.3.10 of P802.11be D2.0.

CID 11218, 11282, 11283, 10376, 10179, 11207, 11352, 11353, 12846, 10827, 10828

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# #11218

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 11218 | ron porat | 36.3.10 | 622.62 | Table 36-18 incorrect formula for T_L-LTF | Change "T_GI,LTF" to "T_GI,L-LTF" | Accepted |

Background (P638 of D2.1.1):

# #11282, 11283

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 11282 | Sigurd Schelstraete | 36.3.10 | 623.34 | "N_service" does not belong in a table with "Timing-related Constants" | Move from Table 36-18 to Table 36-23 | Rejected Since Table 36-23 includes frequently used parameters whose values are configurable, it is not a suitable place to accommodate the constant “N_service”. In addition, it should be noted that in 11ax, “N_service” is also included in the table with “Timing- related constants”. As a result, it is better to keep “N_service” in Table 36-18. |
| 11283 | Sigurd Schelstraete | 36.3.10 | 623.36 | "N_tail" does not belong in a table with "Timing-related Constants" | Move from Table 36-18 to Table 36-23 | Rejected Since Table 36-23 includes frequently used parameters whose values are configurable, it is not a suitable place to accommodate the constant “N_tail, N_tail,u”. In addition, it should be noted that in 11ax, “N_tail, N_tail,u” is also included in the table with “Timing-related constants”. As a result, it is better to keep “N_tail, N_tail,u” in Table 36-18. |

# #10376

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 10376 | Mengshi Hu | 36.3.10 | 624.12 | CBW320 seems not that accurate because CBW320-1 and CBW320-2 should also be considered. If this only indicates CBW320, what about the case CBW320-1 and CBW320-2? If this indicates 320MHz BW, then CBW320 should not be used because it is a little bit confusing. The next two tables in the same page and next page also have the same problem. | Add CBW320-1 and CBW320-2. | Rejected In Table 36-3, there is a note saying “The CH_BANDWIDTH of CBW320-1 and CBW320-2 is interpreted as CBW320 for the transmission of an EHT PPDU of 320 MHz bandwidth.” As a result, it is unnecessaty to add CBW320-1 and CBW320-2 in these tables. |

Background (P640 of D2.1.1):

P579 of D2.1.1:

# #10179, 11207, 11352, 11353, 12846

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 10179 | Xiaogang Chen | 36.3.10 | 626.58 | The "effective number of" may need to clarify. For instance, the note of Nsd need to cover MCS14. also the "unique" in N_sd,total may confused bystandard. | suggest to change the note to include MCS14 in the defintion of N_SD. Maybe the same note need to be added for Nsd_total. | Rejected 80 MHz, 160 MHz and 320 MHz EHT DUP transmission are actually based on 484-tone RU, 996-tone RU and 2x996-tone RU, respectively. For each 484-tone RU, 996-tone RU and 2x996-tone RU, the N_SD value with DCM is half of the N_SD value without DCM, and this is true for all EHT-MCSs. As a result, the N_SD note need not to be clarified especially for EHT-MCS 14. |
| 11207 | ron porat | 36.3.10 | 626.62 | Table 36-23: parameter being defined should be N_SD,u instead of N_SD,total (see original text in 11-21/1893r2) | Replace "N_SD,total" with "N_SD,u" | Revised Agreed with the comment in principle. The proposed changes define N_SD,u and N_SD,total separately. TGbe editor: please incorporate the changes shown in 11-22/1716r0 under tag #11207. |
| 11352 | Bin Tian | 36.3.10 | 626.61 | Based on explanation, NSD,total should be NSD,u. Add another row for NSD,total | as in the comment | Revised Agreed with the comment in principle. The proposed changes define N_SD,u and N_SD,total separately. TGbe editor: please incorporate the changes shown in 11-22/1716r0 tag #11352. |
| 11353 | Bin Tian | 36.3.10 | 626.61 | The denifition of NSD,total is wrong. It should be the total number of data subcarriers in a RU or MRU | as in the comment | Revised Agreed with the comment in principle. The proposed changes define N_SD,u and N_SD,total separately. TGbe editor: please incorporate the changes shown in 11-22/1716r0 tag #11353. |
| 12846 | Yan Xin | 36.3.10 | 626.61 | Based on the explanation, it should be "N_(SD,u)" rather than "N_(SD,total)". | Replace "N_(SD,total)" with "N_(SD,u)" | Revised Agreed with the comment in principle. The proposed changes define N_SD,u and N_SD,total separately. TGbe editor: please incorporate the changes shown in 11-22/1716r0 tag #12846. |

Background (P642 of D2.1.1):

Proposed Changes:

Instruction to TGbe Editor:

Change Table 36-23 (P802.11be D2.1.1 P642) as follows:

## Table 36-23—Frequently used parameters

| Symbol | Explanation |
| --- | --- |
| NRU | For pre-EHT modulated fields, NRU = 1 . For EHT modulated fields, NRU represents the number of occupied RU(s) or MRU(s) in the transmission. |
| Nuser r | For pre-EHT modulated fields, Nuser r = 1 . For EHT modulated fields, Nuser r represents the total number of users in the r-th occupied RU or MRU of the transmission. |
| Nuser total | Total number of users in all occupied RU(s) or MRU(s) of an EHT transmission, i.e., NRU – 1 Nuser total =  Nuser r . r = 0 |
| NCBPS u | Number of coded bits per OFDM symbol for user u, u = 0 1  Nuser total – 1 . |
| NSD | Effective number of data tones carrying unique data. NOTE—The NSD value with DCM (when applicable) is half of the NSD value without DCM, for each RU or MRU size. |
| NSD u | Effective number of data tones carrying unique data for user u, u = 0 1  Nuser total – 1 . |
| NSD total | Effective number of data tones in a RU or MRU. |

# #10827, 10828

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 10827 | Dong Guk Lim | 36.3.10 | 627.10 | NCBPSS,l,u is not the same as NCBPSS,u. To clarify it, the explanation should be modified. | Change " Number of coded bits per OFDM symbol per spatial stream for user u" with "Number of coded bits per OFDM symbol per spatial stream for user u in the l-th 80 MHz frequency block " and delete "in the l-th 80 MHz frequency block " in the second sentence. | Accepted |
| 10828 | Dong Guk Lim | 36.3.10 | 627.23 | NBPSCS,l,u is not the same as NBPSCS,u. To clarify it, the explanation should be modified. | Change " Number of coded bits per subcarrier per spatial stream for user u" with "Number of coded bits per subcarrier per spatial stream for user u in the l-th 80 MHz frequency block " and delete "in the l-th 80 MHz frequency block " in the second sentence. | Accepted |

Background (P643 of D2.1.1):