IEEE P802.11
Wireless LANs

| CR for Section 9.3.1.19 on P802.11be – part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 02.11.2022 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolution to comment on P802.11be D2.0.

7 comments under subclause 9.1.3.19 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 10796

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 142.65 | 9.3.1.19 | For the Nc index subfield, we should assign 3bits because Max Nss is equal to 8. So, we don't need this text | Delete the following text " Nc Index subfield values above 7 are reserved." | REJECTED The TGbe decided to support up to 8SS and keep the format of all subfields related to spatial streams unchanged in SP#4 DCN 22-1238r1 The SP text was: “Do you agree that 802.11be shall not define operation with more than 8 spatial streams and that the format of all subfields related to spatial streams shall remain unchanged (i.e. no changing the number of bits)?" The result in the PHY ad-hoc was 22Y, 4N, 5A The text is required to ensure correct understanding |

## CID 10797

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 143.03 | 9.3.1.19 | For the Nc index subfield, we should assign 3bits because Max Nss is equal to 8. So, we don't need this text | Delete the following text " Nc Index subfield values above 7 are reserved." | REJECTED The TGbe decided to support up to 8SS and keep the format of all subfields related to spatial streams unchanged in SP#4 DCN 22-1238r1 The SP text was: “Do you agree that 802.11be shall not define operation with more than 8 spatial streams and that the format of all subfields related to spatial streams shall remain unchanged (i.e. no changing the number of bits)?" The result in the PHY ad-hoc was 22Y, 4N, 5A The text is required to ensure correct understanding |

## CID 11892

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 138.11 | 9.3.1.19 | Should we define different subclauses for each of the variants? It will help identify which paragraphs apply to which variant. | As in comment. | REJECTED All 4 variants have common purpose, fields and parameters, while the differences between the variants are described in detailes in this subclause. Splitting already existing subclause to different subclauses might lead to unneccesary delay in spec progress |

## CID 12208

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 138.35 | 9.3.1.19 | Nc is extended to 4 bits without any justification. Either we mention explicitly that Nss > 8 is being supported or we should keep Nc size of 3 bits as in HE | As in comment | REJECTED The TGbe decided to support up to 8SS and keep the format of all subfields related to spatial streams unchanged in SP#4 DCN 22-1238r1 The SP text was: “Do you agree that 802.11be shall not define operation with more than 8 spatial streams and that the format of all subfields related to spatial streams shall remain unchanged (i.e. no changing the number of bits)?" The result in the PHY ad-hoc was 22Y, 4N, 5A |

## CID 12209

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 142.06 | 9.3.1.19 | Ng values and codebook size were designed for up to 8 Nss. As we plan to support more Nss it's required to provide more Ng values and codebook sizes. Or at least define a placeholder for new values | As in comment | REJECTED The spec doesnt support Nss larger than 8, thus no need for new Ng values at this stage |

## CID 12210

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 142.65 | 9.3.1.19 | Its unclear why we need Nc Index values larger than 7. Either we mention explicitly that Nss > 8 is being supported or we should keep Nc Index values as in HE | As in comment | REJECTED The spec defined Nc Index size as 4 bits, thus this text is required to ensure correct understanding |

## CID 12211

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 143.02 | 9.3.1.19 | Its unclear why we need Nc Index values larger than 7. Either we mention explicitly that Nss > 8 is being supported or we should keep Nc Index values as in HE | As in comment | REJECTED The TGbe decided to support up to 8SS and keep the format of all subfields related to spatial streams unchanged in SP#4 DCN 22-1238r1 The SP text was: “Do you agree that 802.11be shall not define operation with more than 8 spatial streams and that the format of all subfields related to spatial streams shall remain unchanged (i.e. no changing the number of bits)?" The result in the PHY ad-hoc was 22Y, 4N, 5A The text is required to ensure correct understanding |