IEEE P802.11
Wireless LANs

| Proposed IEEE 802.11 AIML TIG Technical Report Text for the Distributed Channel Access Use Case |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ziyang Guo | Huawei | Huawei Base, Shenzhen, China |  | guoziyang@huawei.com |
| Ming Gan | Huawei |  |  |  |
| Peng Liu | Huawei |  |  |  |

Abstract

This document contains the proposed technical report of IEEE 802.11 AI/MLAIML TIG, especially for the distributed channel access use case

R0: initial version

R1: editorial updates

R2: updated based on comments in AIML meeting on 10/16/2023

# Table of Contents

- Introduction

  - Terminologies

  - Background information

- AIML Use cases for IEEE 802.11

Note: use cases potentially can be organized into different categories

Note: use cases potentially can identify KPIs

  - Use case 1: CSI feedback compression

  - Use case 2: Distributed channel access

    - Use case description

In 802.11, the fundamental access method of the MAC used by STAs is a Distributed Coordination Function (DCF) known as carrier sense multiple access with collision avoidance (CSMA/CA). A STA is required to sense the wireless medium before transmitting. If it finds that the medium is continuously idle for a minimum specified duration (DIFS), the STA is then permitted to start transmission of a frame after waiting for an additional random backoff. The range of the generated random backoff counter is bounded by the Contention Window. The initial range is set between 0 and the minimum Contention Window size (CWmin) . If the medium is occupied, the contention window is doubled until it reaches the maximum Contention Window size (CWmax) in order to further reduce collisions.

Higher throughput and lower latency have been an inevitable trend in WLAN for more than a decade as mentioned in [3] [4] [5] [6] . An efficient channel access protocol is fundamental to achieve this goal. However, the analysis in [7] shows that current contention-based access protocol suffers large performance degradation in dense deployments. The binary exponential backoff leads to short-term unfairness [8] , i.e., consecutive unsuccessful transmissions lead to worse latency perfomance. Therefore, more efficient channel access schemes are needed to increase throughput and to reduce latency and jitter.

Some studies [9] [10] [11] [12] have shown that AIML based channel access schemes can efficiently improve throughput/channel efficiency and reduce latency and jitter. For example, Centralized Contention window Optimization with Deep reinforcement learning (CCOD) [9] , applied Deep Q Network (DQN) or Deep Deterministic Policy Gradient (DDPG) contribute to contention window optimization. Simulation results show that the throughput improvement of CCOD over standard 802.11, ranges from 1.5% (for 5 stations) to 40% (for 50 stations). In [10] , QMIX-advanced Listen-Before-Talk (QLBT) is proposed for distributed channel access and improves the performance by leveraging the powerful learning capability of deep neural networks. Experimental results show that QLBT increases the channel efficiency by 18% and reduces the lantency and jitter by 25% and 90% on average.

This use case proposes to apply AIML techniques to distributed channel access to improve throughput and reduce latency and jitter.

    - KPIs

KPIs considered in this use case include:

- Throughput

- Latency and jitter

- Fair coexistence

    - Requirements

- Fair coexistence with legacy 802.11

  - Support fair coexistence with legacy 802.11 channel access schemes. Introducing AIML based channel access shall not degrade the performance of STAs using conventional channel asscess schemes. Performance metrics include but are not limited to throughput and delay.

- Performance should follow the guidiance below:

  - Throughput improvement measured at MAC data service access point (SAP): Achieve throughput improvement compared to 802.11be for different scenarios.

  - Latency and jitter reduction: Achieve latency and jitter reduction compared to 802.11be for different scenarios. Latency is the time from when a packet enters the queue to when it is successfully received. Jitter is the standard deviation of the latency.

  - Additional complexity introduced by AIML: Minimize the additional complexity, e.g., computation complexity, introduced by AIML process.

  - Performance evaluation need to consider both the single BSS and OBSS scenasios.

    - Technical Feasibility Analysis

      - Standards Impact

The standards impact include:

Signaling and protocols related to parameter exchange between AP and non-AP STAs, e.g., capability indication, data report to facilitate training, neural network parameters distribution, etc.

      - Technical feasibility

- Hardware/software capability: The STAs that support AIML based channel access shall have the hardware and software capability to support AIML algorithm(s). The STAs that support model training may require higher computation capabilities.

- Data availability and accesibility: There are some STAs that are able to use data to perform AIML model training and/or inference [13] . The data used for model training and/or inference shall be accessible for these STAs. It is expected that signaling and protocols related to data accessibility will be specified.

- Model distribution: There may exist model exchange between AP and non-AP STAs during model training and/or inference [13] . It is expected that signaling and protocols related to model distribution will be specified.

  - Use case N

- Summary

- References

- IEEE 802.11-REVme D2.0, October 2022

- IEEE P802.11be D2.2, October 2022

- 11-14-0165-01-0hew-802-11-hew-sg-proposed-par

- 11-18-1231-06-0eht-eht-draft-proposed-par

- 11-22-0030-01-0wng-look-ahead-to-next-generation

- 11-22-0046-01-0wng-next-generation-after-802-11be

- F. Abinader et al., “Performance evaluation of IEEE 802.11n WLAN in dense deployment scenarios”, 2014 IEEE 80th vehicular technology conference (VTC2014-Fall)

- A. Bhattacharya and A. Kumar, “Analytical Modeling of IEEE 802.11-Type CSMA/CA Networks with Short Term Unfairness”, IEEE/ACM Transactions on Networking, Vol. 25, Issue 6, Dec 2017

- W. Wydmański and S. Szott, “Contention Window Optimization in IEEE 802.11ax Networks with Deep Reinforcement Learning”, 2021 IEEE Wireless Communications and Networking Conference (WCNC)

- Z. Guo, Z. Chen, P. Liu, J. Luo, X. Yang and X. Sun, “Multi-agent reinforcement learning-based distributed channel access for next generation wireless networks”, IEEE Journal on Selected Areas in Communications, Vol. 40, Issue 5, May 2022

- Y. Ding, S. Liew and T. Wang, “Non-Uniform Time-Step Deep Q-Network for Carrier-Sense Multiple Access in Heterogeneous Wireless Networks”, IEEE Transactions on Mobile Computing, Vol. 20, Issue 9, Sept 2021

- L. Zhang, H. Yin, Z. Zhou, S. Roy and Y. Sun, “Enhancing Wi-Fi multiple access performance with federated deep reinforcement learning”, 2020 IEEE 92nd Vehicular Technology Conference (VTC2020-Fall)

- 11-22-0723-01-0wng-further-discussion-on-next-generation-wlan