IEEE P802.11
Wireless LANs

| LB271 CR for EHT-SIG Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.03.08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Mengshi Hu |  |  |  |  |
| Stephen McCann |  |  |  |  |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D3.0.

16 comments are resolved.

CID 15222, 15223, 15254, 15255, 15713, 16268, 16269, 16270, 17211, 17212, 17240, 17241, 17242, 17243, 17244, 17926

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Correct some typo |

## CID 15222

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 781.12 | 36.3.12.8.2 | Add "subfield" at the end of "Number of Non-OFDMA Users" for Common field in Figure 36-34 and 36-36. | As in comment | REVISED Instructions to the editor: Please add "subfield" at the end of "Number of Non-OFDMA Users" for Common field in Figure 36-34, 36-36, 36-37, 36-38, 36-39, 36-40, and 36-41 of P802.11be D3.0. NOTE to the editor: will provide the visio files separately when the motion is passed. |

Discussion

## CID 15223

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 810.06 | 36.3.12.8.6 | Add "subfield" at the end of "Number of Non-OFDMA Users" for Common field in Figure 36-42, 36-43, 36-44, 36-45 and 36-46. | As in comment | ACCEPTED NOTE to the editor: will provide the visio files separately when the motion is passed. |

Discussion

## CID 15254

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 787.10 | 36.3.12.8.3 | RU Allocation subfield may be used to indicate punctured 242 RU or unassigned 242-tone RU. It is confused whether the above two cases are RU or not RU. For example, if the punctured 242 tone RU is an RU, the RU definition should contain this RU. If it is not an RU, it is conflicted with the description "Each RU Allocation-A subfield in an EHTSIG content channel corresponding to a 20 MHz frequency subchannel indicates the RU or MRU assignment, including the size of the RU(s) or MRU(s) and their placement in the frequency domain" | Need to clarify whether the punctured RU and unassigned 242-tone RU are RUs or not. | REJECTED Punctured 242-tone RU or unassigned 242-tone RU both refer to 242-tone RU. |

Discussion

None.

## CID 15255

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 787.10 | 36.3.12.8.3 | Punctured 242-tone RU and Unassigned 242-tone RU use the wording "-tone RU". However, the other rows do not use this description. For example, B8-B1 (67-71) corresponds to "242". | The wording should be consistent. | REJECTED Punctured 242-tone RU or unassigned 242-tone RU both refer to 242-tone RU. The full naming “242-tone RU” is used for special cases where additional descriptions are needed. |

Discussion

## CID 15713

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 802.07 | 36.3.12.8.5 | "242 subcarriers" should be unify to "242-tone".Also in P793L49, P802L15. | As in comment | REVISED Instructions to the editor: Please change “242 subcarriers” to “242 tones” in P802L07, P793L48 and P802L15 of P802.11be D3.0. |

Discussion

## CID 16268

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 779.42 | 36.3.12.8.2 | typo "is 20" in the title of Figure 36-31 | Change "is 20" to "is equal to 20" | ACCEPTED |

## CID 16269

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 779.59 | 36.3.12.8.2 | typo "is 160" in the title of Figure 36-32 | Change "is 160" to "is equal to 160" | ACCEPTED |

## CID 16270

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 780.14 | 36.3.12.8.2 | typo "is 320" in the title of Figure 36-33 | Change "is 320" to "is equal to 320" | ACCEPTED |

## CID 17211

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 790.21 | 36.3.12.8.3 | "[Gap-242/484/996]" should have lowercase g for consistency with the rest of the table. | Change "[Gap-242/484/996]" to "[gap-242/484/996]" | REJECTED Capitalized G is used because it is the intial wording of a sentence. |

## CID 17212

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 790.39 | 36.3.12.8.2 | "Punctured 242-tone RU or unassigned 242-tone RU contributes to zero User fields" | Change "contributes to zero" to "contributes zero" | ACCEPTED |

Discussion

None

## CID 17240

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 782.35 | 36.3.12.8.3 | Values 5-7 are not specified | Add "Values 5-7 are reserved" | REVISED Instructions to the editor: Please add the following paragraph on P782L40 of P802.11be D3.0: Values 5-7 are Validate. |

Discussion

## CID 17241

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 794.35 | 36.3.12.8.4 | Values 5-7 are not specified | Add "Values 5-7 are reserved" | REVISED Instructions to the editor: Please add the following paragraph on P794L40 of P802.11be D3.0: Values 5-7 are Validate. |

Discussion

## CID 17242

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 796.24 | 36.3.12.8.4 | Value 2 is not specified | Add "Value 2 is reserved" | REVISED Instructions to the editor: Please add the following paragraph on P796L28 of P802.11be D3.0: Value 2 is Validate. |

Discussion:

## CID 17243

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 796.29 | 36.3.12.8.4 | Values 5-7 are not specified | Add "Values 5-7 are reserved" | REVISED Instructions to the editor: Please add the following paragraph on P796L35 of P802.11be D3.0: Values 5-7 are Validate. |

Discussion:

## CID 17244

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 796.37 | 36.3.12.8.4 | Values 8-15 are not specified | Add "Values 8-15 are reserved" | REVISED Instructions to the editor: Please add the following paragraph on P796L40 of P802.11be D3.0: Values 8-15 are Validate. |

Discussion:

## CID 17926

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 797.39 | 36.3.12.8.5 | "Â¡. non-OFDMA transmission" should be " a non-OFDMA transmission" | As in the comment. | REVISED Instructions to the editor: Please add “a” between “and” and “non-OFDMA” on P797L40 of P802.11be D3.0. |

Discussion: