IEEE P802.11
Wireless LANs

| CR for Section 36.3.17 on P802.11be |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 08.03.2023 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolution to comment on P802.11be D3.0.

3 comments under subclause 36.3.17 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 15469

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 859.52 | 36.3.17.1 | Incomplete description | disjoint subsets of the spatial streams are intended for reception at different STAs in an RU or MRU of size greater than or equal to 242-tone | ACCEPTED Note to the Editor: please change the text as specified in 11/23-0331r0 under CID #17152 |

## CID 17149

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 861.7 | 36.3.17.3 | The first two sentences of this paragraph are redundant. Suggest combine them in one sentence. | Combine the first two sentences as "If the EHT NDP Announcement frame requests CQI feedback, then upon receipt of the EHT sounding NDP, the beamformee computes CQI feedback, CQI_s,r,u, for beamformee u in RU r for spatial stream s as described in 9.4.1.73 (EHT CQI Report field)." | REVISED Note to the Editor: please change the text as specified in 11/23-0331r0 under CID #17152 |

## CID 17152

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 861.18 | 36.3.17.3 | The subsequent transmission may not be MU transmission. Suggest remove "MU". | as in comment | REJECTED The MU transmission is mentioned as an example, where CQI feedback might be used for RU assignment. It doesn’t imply necessary subsequent MU transmission |

Instructions to the editor: please change the text on page 859, L52 as following:

disjoint subsets of the spatial streams are intended for reception at different STAs in an RU or MRU of size greater than or equal to 242-tone

Discussion on CID 17149

Those two sentences indeed repeats each other, however, the purpose of the first sentence is to provide the general instructions for CQI feedback. While the second sentence instructs how to estimate CQI feedback per requested RU.

End of discussion

Instructions to the editor: please change the text on page 861, L7 as following:

If the EHT NDP Announcement frame requests CQI feedback, then upon receipt of the EHT sounding NDP,
the beamformee computes CQI feedback for all the RUs indicated by the EHT NDPA Announcment frame as described in 9.4.1.73 (EHT CQI Report field). The CQI feedback, for beamformee u in RU r for spatial stream s shall be estimated using the method described in 9.4.1.73 (EHT CQI Report field).